3GPP SA WG2 Meeting #172	S2-2510496

17 - 21 November, 2025, Dallas, USA

Title:	LS on SA2 progress for UE data collection and data transfer

Response to:	Reply LS on UE data collection and data transfer (R2-2507928/S2-2509869)

Release:	Release 20

Work Item:	FS_AIML_CN_Ph2

Source:	SA2

To:	RAN2, RAN

Cc:	-

Contact Person:

Name:	Tingfang Tang

Tel. Number:

E-mail Address:	tangtingfang@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

SA2 thanks RAN2 for the LS on UE data collection and data transfer(R2-2507928/S2-2509869).

Regarding the following request from RAN2:

“SA2 can continue their work and take these answers into account to provide feedback to RAN by December.”

Based on the latest progress in SA2 and feedback from RAN2, SA2 would like to provide the following information:

- SA2 is studying the following WT#1 in FS_AIML_CN_Ph2 with any progress captured in TR 23.700-04.

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

SA2 has already discussed 21 potential solutions for the WT#1, however, no conclusion has been reached for all aspects of the key issue for WT#1 with related concerns, including the following stated for each key issue aspect:

a) How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

For this key issue, different understanding of the following 2 aspects leads to different solutions on this key issue.

      - Which CN node is used for the UE data collection;

      - The UP path of "the data transfer path UE-> CN -> Server for data collection for UE-side model training/OTT server" can be implemented as a) concatenation of two separate UP paths which are UE-> CN, and CN -> Server for data collection for UE-side model training/OTT server; b) one UP path UE-> CN-> Server for data collection for UE-side model training/OTT server with e2e termination at UE and Server for data collection for UE-side model training/OTT server, CN node act as an intermediary node of the data transfer path.

- How to support in 5GC the full visibility of standardized UE data contents.

No common understanding on the "visibility" to conclude on this key issue.

- The clarification on visibility is from RP-243316 with the following clarification from RAN:

Visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

However, for verify/match a) the data specified/configured to be collected and b) the data that is being reported, it is not clear what a) is, and there are different assumptions in different solutions.

- How 5GC can support the full controllability of standardized UE data transfer.

No common understanding on the "controllability" to conclude on this key issue.

- The aspects considered for bullet a) also apply for bullet c);

  - There is understanding that the interaction between UE client (transferring data) and AS layer (collecting data) with 5GCN controllability shall be assessed in coordination with RAN2, SA3, and CT1.

- How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

      - No agreement on whether to support both PDU session level and QoS flow level traffic differentiation for UE data collection from the UE regular traffic in the 5GC.

- Whether and how the UE knows about what the standardized data to be transferred are.

      - In LS from RAN 2(S2-2509869), it is stated that RAN can provide a list of candidate data configuration options, and the UE can indicate the preferred data configuration options from this list. However, the related SA2 question on "SA2 would like to ask RAN2 whether the trigger for UE data collection is possible and required from UE side model training server (OTT server), from the UE, or from both." is not solved, where the related answer will impact the conclusion on this key issue.

- The TR 23.700-04 is sent for information at TSG#109, Sep 2025.

- WT#3 to study on whether and how to support NW standardized signalling of dataset and model parameter sharing for non-OTA(Over The Air) scenarios is added into FS_AIML_CN_Ph2 and the date for approval for TR 23.700-04 is planned to be changed to TSG#112, June 2026.

Once the conclusion has been reached for AI/ML UE sided data collection based on the above, SA2 will update RAN2/RAN.

# 2. Actions:

To RAN2/RAN:

ACTION: SA2 kindly asks RAN2 and RAN to take the above information into account.

# 3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #173		09-13 February 2026	India, IN

TSG-SA2 Meeting #174		13-17 April 2026	Malta, MT