SA WG2 Meeting #172	S2-2510535

17 - 21 November, 2025, Dallas, USA	(revision of S2-2509794)

Source:	Samsung

Title:	[WT#5 Data framework] WT scope and KI on 6G Data framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses and proposes work task scope and corresponding KI description for WT#5 on 6G Data framework.

- Discussion

The below table provides an example for categorization of data that can be considered to be generated in 6G system along with the practices in 5G.

| Data source | Data category | Data type /Purpose | Data consumer/ processing entity (expected in 6G) | Practices in 5G / Data collection path |  |
| --- | --- | --- | --- | --- | --- |
| UE | AS layer related data | Radio measurement data for AIML data collection (UE/RAN/NF-side model training at NF or OTT server) + timestamp, quality info | Data collection NF and/or OTT training server | SA2/RAN LMF-side model (positioning case3b) RAN-side model (positioning case 3a) | Path for AIML data collection (UE-side model training at OTT) in 5G |
|  |  | Radio measurement data for positioning + timestamp, quality info | LMF | SA2/RAN | Path for AIML data collection (LMF-side model training at LMF/NWDAF) in 5G |
|  |  | Radio measurement data for sensing (3GPP-sensing data) | Sensing NF | - | No practice. UE RAN Sensing NF |
|  |  | Radio measurement data for OAM (e.g., MDT data) | OAM (e.g., TCE, MnS producer) | SA5 OAM | UE RAN OAM |
|  |  | Radio measurement data for AIML data collection (for RAN2/3 specified use case, e.g., mobility, SON) | RAN | RAN | UE RAN |
|  | NAS layer data or upper layer (e.g., application layer) data | Application-layer data for NWDAF-side model training EVEX |  | SA2 SA4 |  |
|  |  | Positioning data (not related to radio measurement) for positioning, e.g., SSID | LMF |  |  |
|  |  | Non-3GPP Sensing data |  | - | No practice. UE Sensing NF (transparent to RAN) |
| NF | NF-generated data | SA2 control plane data for training NF-side model (for the purpose of analytics service, e.g., per UE or per PDU Session) | Data collection NF / AI NF | SA2 NWDAF | NF NWDAF |
|  | Context data | Context data for NF service | NF | NF, UDSF, UDM | NF, NFUDSF, NFUDM |
|  | AF-provided data | Data provided by AF via Naf service for purpose of NF-side processing (EVEX data from AF to NWDAF) or provisioning info to UE (e.g., GNSS assistance data) | Data collection NF AI NF | NWDAF LMF | AF (NEF) NWDAF AF (NEF) LMF (UE) |
| RAN | RAN measurement or RAN-generated data | Radio measurement data for sensing (3GPP-sensing data) | Sensing NF | - | RAN Sensing NF |
|  |  | Radio measurement data for AIML data collection (RAN/NF-side model) | LMF/RAN | SA2/RAN LMF-side model (positioning case3b) RAN-side model (positioning case 3a) | Path for AIML data collection (LMF-side model training at LMF/NWDAF) in 5G |
|  |  | Radio measurement data for RAN-specified use case | RAN | RAN | RAN |
|  |  | Radio measurement data for OAM | OAM | OAM | RAN OAM |
| OAM | Management domain data | Management data generated/managed for the purpose of MnS producer | OAM | SA5 OAM | RAN/NF OAM |
|  |  | Management data generated/managed for the purpose of MnS producer | NF requiring node-level management data | SA5-provided MnS API is invoked by SA2 NF | RAN/NF OAM |

※Green-coloured cells can be considered as candidates of data to be addressed as a starting point for the 1st bullet, “Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework”

Additionally, the WT scope description (S2-2509794) is considered as being stable, so this pCR proposes to keep the text for WT scope. However, this pCR proposes to separate key issue into two parts: (i) identification of use case along with the actual architecture/functionalities discussion together (1st and 2nd bullets of WT scope), and (ii) the 3rd bullet regarding system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective. Those bullets are somehow potentially correlated, but for the convenience at conclusion phase, it is proposed to keep them as separated key issues.

2. proposal

It is proposed to approve the following WT scope and KI description in TR 23.801-01.

**** First Change (no change from the baseline pCR S2-2509794 in SA2#171)****

A.X 	Work Task X: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

- Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework.

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

- Study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution) )

- data labelling/metadata handling

- data processing (e.g. data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 4:	 Coordination with SA5 may be required for charging aspects.

NOTE 5:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 6:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

- Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE 7:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** Next Changes (all new)****

5.X. Key Issue #X.1: Data framework for handling of data generated in the 6G system

The key issue investigates the following aspects:

- Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework.

NOTE 1:	This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 2:	The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

- Study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution))

- data labelling/metadata handling

- data processing (e.g. data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 3:	Coordination with SA5 may be required for charging aspects.

NOTE 4:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 5:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

5.X. Key Issue #X.2: Data framework for handling of data generated in the 6G system

Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** End of Changes ****