3GPP TSG-SA WG2 Meeting #172	S2-2510604

Dallas, USA, 17 - 21 November, 2025

Source:	CATT, vivo

Title:	23.700-19: Discussion and Interim Agreement for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper discusses the CP/UP and IP/Non-IP of KI#1, and proposes Interim Agreements for KI#1.

1 Background

  - SA1 Requirement of IMS voice using GEO satellite access

The requirements of IMS voice using GEO satellite access defined in SA1 TS22.261 as follows:

-	The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [55].

-	The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

The performance requirements of IMS voice using GEO satellite access defined in SA1 TS22.261 as follows:

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity(DL) | Area traffic capacity(UL) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |

1.2 Summary of the solutions on KI#1

| Sol# | CP/UP | Bearer (single/two bearers) & related optimizations | IMS voice data over IP/Non-IP | IMS signalling over IP/Non-IP |
| --- | --- | --- | --- | --- |
| #1 | UP | - default EPS bearer for both IMS signalling and voice data; - non-GBR QoS update / resource reservation is skipped / pre-initiated | IP | IP |
| #2 | UP | - default EPS bearer for IMS signaling; - a dedicated EPS bearer for IMS voice; - established together / pre-establish; | IP - PDCP(ROHC) | IP |
| #3 | UP | - default EPS bearer for IMS signaling; - a dedicated EPS bearer for IMS voice; - new Packet filter component type identifier, e.g. "(IMS) voice traffic type" | Non-IP; - PGW translate between non-IP and IP/UDP/RTP | IP |
| #4 | UP | - two default DRBs, one for signalling and one for voice media; - assign an existing or newly defined QCI / configure separate APNs; | Non-IP; - PtP tunnel based on UDP/IP are pre-configured on the P-GW | Non-IP; - security association is FFS |
| #5 | CP | - new SRBx / dedicated EPS bearer for IMS voice; - a separate SRB for the IMS signalling is FFS; - dedicated EPS bearer is non-GBR and possibly needs a new QCI; - associated EPS bearer identities for the corresponding SRB(s); - UE can locally derive the UL TFT based on the SDP information; | Option 1:IP Option 2:Non-IP Option 3:Non-IP In case of non-IP: - UE support TFTs in Session Management request - how the PGW performs the IP and non-IP conversion is FFS | Option 1:IP Option 2:Non-IP Option 3: IP |
| #6 | CP | - SRBx / default EPS bearer for IMS signalling; - SRBy / a dedicated EPS bearer for IMS voice; - Whether and how to provide QoS to the IMS voice service via SRB is FFS - pre-establish; new QCI is FFS - the UL TFT can be updated by the UE locally based on the IMS SDP negotiation result | IP - NAS(ROHC) | IP |
| #7 | CP | - default EPS bearer for both IMS signalling and voice data - an SRB is used for IMS signalling and can be switched to data delivery via DRB and S1-U bearer for voice data transmission - non-GBR QoS update / resource reservation is skipped / pre-initiated - It is FFS whether or how to differentiate QoS/priority further between IMS signalling and voice data. | IP | IP |
| #8 | CP+UP | - CP for IMS signalling / default bearer - UP for voice data / dedicated bearer - SGW needs to simultaneously maintain corresponding S11-U path and S1-U path | IP | IP |
| #9 | CP+UP | - UP for IMS signalling and voice when the UE is in ECM-CONNECTED - dedicated GBR bearer for voice; - IMS SIP INVITE data transport in CP | IP | IP |
| #10 | CP+UP | - a pair of PDN connections - two PDN connections, one without DRB for SIP, the other with DRB for voice media. - coexistence of S11-U and S1-U | Non-IP | Non-IP |
| #11 | CP+UP | - used together with sol#10 - P-GW is assumed to know the IP address of P-CSCF by pre-configuration. - UE: Extracting an IMS AGW IP address from SDP and informing it to MME - The current description mentions IP header only. FFS whether to consider UDP/TCP header addition/removal as well. | Non-IP； - It is FFS how the UE generates TCP/UDP payload without IP header - PGW performs the IP and non-IP conversion | Non-IP； - PGW performs the IP and non-IP conversion |

2 Discussion

2.1 CP vs UP for IMS signalling

The UP approach has lower protocol overhead compared to the CP approach and can provide QoS differentiation through default and dedicated bearers with QCI-based handling, while CP relies on SRBs intended for small NAS messages without per-flow QoS control. Additionally, the current IMS architecture typically uses a single PDN connection with a common APN, where signaling and voice media are transmitted over separate EPS bearers.

The LS replies from CT1 and SA3 indicated that NAS-based CP optimization brought only limited benefit. CT1 confirmed that reducing NAS message size for CP signalling offers minimal gain and that the new NAS message format would likely not be reusable for other cases. SA3 reported that the majority of companies were not in favour of allowing non‑integrity‑protected voice packets to be sent over NAS, and therefore there was no consensus to remove the 5 bytes of NAS security overhead. Overall, the CP-based approach is considered inefficient, complex, and constrained by existing NAS security requirements.

The LS reply from RAN2 confirmed that CP-based solutions would increase specification complexity, while UP can reuse existing EPS bearer procedures. RAN2 also indicated that it is technically feasible to support more than two DRBs, allowing separate handling of signalling and voice traffic with independent QoS control.

As the interim conclusion for voice packets has already adopted the User Plane, and considering the feedback from RAN2, CT1 and SA3 showing that CP optimization is impractical, the User Plane approach should also be applied for IMS signalling over NB‑IoT (GEO) to ensure consistent architecture and efficient resource control.

Principle 1: The IMS signalling packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Principle 2: Separate EPS bearers in one PDN connection should be used for voice and signalling to enable independent QoS control and optimized resource management.

2.2 IP vs non-IP Comparison for Voice Media Transfer

| Analysis Aspects |  | IP with RoHC | Non-IP(RTP only) | Comparison |
| --- | --- | --- | --- | --- |
| Protocol Stack |  | IP/UDP/RTP + RoHC | RTP (Non-IP transport) |  |
| Protocol Overhead |  | SO (Steady State): 3 B FO (Recovery State): 8 B IR/IR-DYN (Init State): 15-40 B | 1 B (SN only) ~12 B (typical two-way voice call) (NOTE 1) | RoHC steady state competitive with Non-IP realistic scenarios. Non-IP advantage exists but is limited to basic functions. |
| RTP Functionality Support | Multi-call Support | Support, ROHC header no change | Requires SSRC (+4 B) | Supporting RTP functionality may introduce additional protocol header overhead. |
|  | Forward Error Correction | Support, ROHC header no change | Requires SSRC (+4 B) |  |
|  | Jitter Buffer Management | Support, ROHC header no change | Requires TS (+4 B) |  |
| Performance Efficiency | 20ms P-time (6B payload) | 3B → 3B/(6B+3B) = 33.3% | 1B → 1B/(6B+1B) = 14.3% to 5B → 5B/(6B+5B) = 45.5% | RoHC becomes highly efficient at operational p-time, while non-IP advantage reduced. |
|  | 80ms P-time (24B payload) | 3B → 3B/(24B+3B) = 11.1% | 1B → 1B/(24B+1B) = 4.0% to 5B → 5B/(24B+5B) = 17.2% |  |
| IMS Architecture Compatibility |  | Native IP PDN | Requires Non-IP to IP conversion | IP seamlessly integrates with existing IMS core. |
| NOTE 1: RTP protocol is used to transmit real-time voice defined in RFC 3550, and the first twelve octets are present in every RTP packet as mentioned. Any RTP reduction relies on SA4 work. |  |  |  |  |

At GEO-appropriate 80ms p-time, IP with RoHC achieves 11.1% overhead while maintaining full IMS functionality, compared to Non-IP's 4.0-17.2% range with significant feature limitations. IP with RoHC can operate in steady state with 3B overhead, and any remaining issues can be resolved through further optimizations, providing more benefits compared to non-IP solutions that rely on RTP reduction with functional clipping. Non-IP's theoretical minimum (1B) may only apply to basic voice without essential IMS features.

According to SA1, the UE type is handheld, and the support of multiple concurrent calls and DTMF in NB‑IoT NTN  scenarios depends on SA1’s forthcoming reply. If SA1 confirms support for these features, the Non‑IP  protocol overhead is expected to increase to no less than 5 B.

Since IMS PDN is inherently an IP-type PDN Connection, non-IP solutions require UE/PGW enhancements for Non-IP to IP conversion with address/port management, which can be implemented by reusing the P2P tunnel or designed similar to solution #3 in TR 23700-19.

According to the RAN1 LS reply, the probability of consecutive loss of 16 or 64 packets is negligible under LOS conditions. However, in NLOS conditions with intermittent blockage (e.g. according to the land mobile satellite channel model in TR 38.811), the probability of having 16 or 64 packets consecutively lost may not be negligible. RAN1 has not reached consensus on whether to consider this case.

Considering the trade-offs between the two approaches—IP with RoHC maintaining full IMS functionality but with potential challenges in occasional NLOS conditions, versus non-IP reducing overhead but requiring protocol enhancements and feature limitations—the following principle is proposed:

Principle 3: For voice media transport over NB-IoT (GEO), both IP-based transmission with RoHC and non-IP-based transmission can be optionally supported by the operator based on deployment scenarios and requirements.

NOTE 1:	RoHC enhancement and other optimizations can be used for IP/UDP/TCP header.

2.3 IP vs non-IP Comparison for IMS Signalling Transfer

| Analysis Aspects |  | IP with RoHC | Non-IP (SIP only) | Comparison |
| --- | --- | --- | --- | --- |
| Protocol Stack |  | IP/(UDP or TCP) + RoHC + SIP | SIP (Non-IP transport) |  |
| Protocol Overhead |  | SO (Steady State): 3 B FO (Recovery State): 8 B IR/IR-DYN (Init State): 15-40 B | 0 B |  |
| Voice SIP Message Analysis (NOTE 1) | SIP INVITE (simplified voice) | 3B + 600B payload = 603B | 600B payload = 600B | Marginal Non-IP advantage diminished by large SIP payload |
|  | SIP OK (200, voice only) | 3B + 400B payload = 403B | 400B payload = 400B |  |
|  | SIP ACK (confirmation) | 3B + 180B payload = 183B | 180B payload = 180B |  |
| Security |  | IPsec ESP support | No standardized security | TS 33.203 mandates IPsec ESP for SIP signalling integrity protection between UE and P-CSCF. Non-IP solutions lack this mandatory security framework. |
| IMS Architecture Compatibility |  | Native IP PDN | Requires Non-IP to IP conversion | IP seamlessly integrates with existing IMS core. |
| NOTE 1: For GEO voice calls, assume the SIP messages are simplified or compressed using SigComp; therefore, propose 600B, 400B, and 180B as examples. |  |  |  |  |

Even with simplified GEO voice SIP messages, the efficiency gain from non-IP solutions is likely negligible.

According to TS 33.203 for IMS security, IPsec ESP shall provide integrity protection of SIP signalling between the UE and the P-CSCF, protecting all SIP signalling messages at the IP level. In contrast, Non-IP solutions lack any standardized security mechanisms between the UE and the P-CSCF, and require further work in SA3.

NOTE 1:	I1 protocol is a transport independent protocol, so both IP and non-IP can support it, and the security association between UE and SCC AS may be established with IP more conveniently.

The IMS network architecture is inherently designed based on IP. IMS signalling relies on IP addressing for session handling and routing within the core network, and requires IP-based transport to ensure IMS entities such as P-CSCF can operate normally without requiring special modifications. A non-IP solution would complicate the association between signalling and media contexts, increase processing overhead, and introduce interworking complexity at network elements.

Therefore, to minimize impacts on the IMS Control Plane, preserve compatibility with standard IMS components, and ensure consistent addressing and alignment between signalling and media within the IMS core, IP transport should be maintained for the default bearer carrying IMS signalling.

The following principle is proposed:

Principle 4: IP-based IMS signalling packets shall be transferred over NB-IoT (GEO).

3 Proposal

Based on the above discussion, it is proposed to agree the interim conclusions as follows:

* First change*

8.1	Interim conclusions for Key Issue #1

Editor's note 1:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The IMS signalling packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	Separate EPS bearers in one PDN connection shall be used for voice and signalling to enable independent QoS control and optimized resource management.

-	For voice media transport over NB-IoT (GEO), both IP-based transmission with RoHC and non-IP-based transmission can be supported by the operator based on deployment scenarios and requirements.

NOTE 1:	RoHC enhancement and other optimizations can be used for IP/UDP/RTP header.

-	IP-based IMS signalling packets shall be transferred over NB-IoT (GEO).

* End of change *