SA WG2 Meeting #172	S2-250xxxx

# November 17 - 21, 2025, Dallas, USA

Source:	NTT DOCOMO

Title:	WT 1.4 KI text update

Document for:	Approval

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes an update for the Diameter related KI text and suggests a text for Fallback if the current text cannot be agreed.

Discussion

P-3e: Considering (i) Diameter is used not only for the context of IMS but also for other purposes (e.g., for SMS between SMSF and SMS IWMSC (TS 29.338)), and (ii)when migration to Diameter was considered, even a TS (TS 29.305) was created in CT WG4 for the converter between MAP and Diameter, it could be argued that CT is a better place to discuss the graceful migration from Diameter to SBI. However, since this issue is so crucial both for operators and vendors, it is proposed here to keep a text with a NOTE suggesting further discussion with CT. The text needs to be editorially revised, since Cx and Sh are not related to Policy Function.

P-3b, Q-4a: 6G native voice could be ideal, but some operators, including us, most probably cannot prepare the network for that due to economic reasons. Then, if voice service is not available to UE camping under 6G RAT, UE selects LTE or NR and 6G RAT deployment would become useless. Considering this way, operator would decide not to deploy 6G RAT. So, (not only domain selection, but also) fallback is so critical for operator. It is proposed to keep the current texts. If those texts sound to imply a solution, it is suggested to use a text "Study how to provide voice service to UE camping on 6G RAT cell at the call setup attempt, when native IMS voice over PS session in 6GS is not supported."

In the below, text is proposed for Diameter related aspects.

********** First Change **********

Annex A.X. WT#1.4 6G Essential Services

Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in  5GS to support voice services as the starting point for discussion) – the following aspects will be studied:

1.	How a UE supporting 6G RAN selects and connects to a network for voice service.

2.	Whether and how to update the IMS specification to support an IPCAN in the 6GS.

3.	How the 6GS supports IMS based voice over the 6G access network

4.	How to support the voice continuity between 6G RAT and selected 3GPP  RATs or between 6G RAT and selected non-3GPP Accesses.

P-3b.	Whether and how to support fallback for voice services to 5GS and EPS.

P-3e.	How to support the compatibility between 6G CN and the existing deployed IMS with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh)

NOTE 1:	Any NTN specific aspects for voice will be studied as part of Key Issue related to WT#7

NOTE 2:	Interworking, migration and service continuity aspects need to be coordinated with WT#2.

NOTE 3:	Support of voice services via 6G NTN need to be coordinated with WT#7.

NOTE 4: The existing IMS architecture is used as a starting point for the study.

NOTE 4a:	Coordination with RAN and SA4 will be required.

NOTE 0e: For P-3e, coordination with CT is required, including to consider whether SA2 or CT WG leads the study.

5.Q. Key Issue #Q: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key issue P.

Additionally, this key issue will study:

1.	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

2.	The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

3.	How to identify emergency services in 6GS in case of non UE detectable emergency call, such that the required priority for an emergency call is applied.

4.	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

Q-4a.	Whether and how to support for emergency voice services if not natively provided by 6GS.

5.R. Key Issue #R: Location Services for 6G

This Key Issue will investigate how to support location service in 6G – the following aspects will be studied:

1.	The architecture to support location services in 6GS.

2.	How to support regulatory location services for emergency voice service in 6GS.

3.	How to support location service exposure, e.g. to 6GCN, NF, AF.

NOTE 1:	The focus in this release of the specifications will be on the required location service capability to support emergency and other regulatory services, however, where possible a unified location service architecture (using 5GS as the starting point for discussion) should be pursued such that additional location services can be supported in future releases.

NOTE 2:	The scope will keep alignment with RAN.

5.S. Key Issue #S: Messaging Services for 6G

For messaging service in 6G this Key Issue will study:

1.	How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [x1], TS 22.261 [x2], TS 22.105 [x3].

NOTE:	For SMS to Emergency centre support it is assumed that the outcome of 5G-A Rel-20 study will be adopted also for 6G.

5.T. Key Issue #T: Other Essential/Regulatory Services for 6G

This Key Issue will study the following services:

1.	How to support Multimedia Priority Services (MPS) in 6GS.

2.	How to provide priority treatment to access control, signalling and media packets delivery for Mission Critical Services (MCX Services) in 6GS.

3.	How to support Public Warning System (PWS) in 6GS.

NOTE 1:	For PWS stage 2 capabilities will be specified by CT.

NOTE 2:	The scope will keep alignment with RAN.

********** End of Changes **********