3GPP TSG-SA WG2 Meeting #172	S2-2510678

# November 17 - 21, 2025 Dallas, USA

Source:	Samsung

Title:	Interim agreement update and final conclusion for KI#2

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement update and final conclusions for KI#2 Authorization and Revocation to Support Sensing

# 1. Discussion

TS 23.700-14 v1.1.0 has some agreed principle for KI#2 Authorization and Revocation to Support Sensing Service but there are some other aspects that need to be addressed.

If the ongoing sensing service is completed, the Sensing Function can revoke the ongoing sensing service and update to sensing service consumer.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * First Change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

### 7.1.2	Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorization of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorization of the AF's Sensing Service Request performed by the Sensing authorization function.

NOTE:	In this clause, the Sensing authorization function wording may need alignment according to the conclusion of key issue 1.

b)	Sensing authorization function determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorization information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorization of the sensing service request.

d)	Sensing authorization function can revoke to the previously authorized Sensing Service Request when the authorization criteria are no longer met. Also, the sensing function can revoke when the ongoing sensing service operation is completed.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorization for the gNB as the Sensing Entity, as the gNB is deployed by operator.

* * * Second Change * * * *

# 8	Conclusions

The following conclusions are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorization of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorization of the AF's Sensing Service Request performed by the Sensing authorization function.

NOTE:	In this clause, the Sensing authorization function wording may need alignment according to the conclusion of key issue 1.

b)	Sensing authorization function determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorization information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorization of the sensing service request.

d)	Sensing authorization function can revoke to the previously authorized Sensing Service Request when the authorization criteria are no longer met. Also, the sensing function can revoke when the ongoing sensing service operation is completed.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorization for the gNB as the Sensing Entity, as the gNB is deployed by operator.

* * * End of Changes * * * *