3GPP SA WG2#172	S2-2510708

Dallas, US, 17-21 November, 2025	(Revision of S2-250xxxx)

Source:	OPPO

Title:	Update to solution #5

Spec:	3GPP TR 23.700-30

Agenda item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution is proposed to update solution #5 to resolve the Editor’s Notes.

1. Discussion

In order to resolve the Editor’s Notes in solution #5, the ENs are discussed and resolved as follows:

Editor's note:	DO-A traffic security mechanism will be determined by SA WG3.

Editor's note:	DO-A trigger broadcast signal is further defined in RAN WGs.

Editor's note:	Details for device check for DO-A Data Transfer procedure will be determined by SA WG3.

According to the Rapporteur’s guidance, the cross-WG requirements can be marked as NOTEs instead of Editor’s Note. So ENs above are directly changed into NOTEs.

Editor's note:	Whether and how temp ID can be used to find the AIOTF is FFS.

Since current temp ID does not include AIOTF information, it is proposed to include target AIOTF ID in the D2R message for AIOTF selection.

Editor's note:	Whether inventory procedure for the DL NAS messages transfer is needed is FFS.

Since there is already a DO-A traffic transmission before the downlink ACK is sent to the AIOT device, it is proposed to reuse the resource (including the NGAP ID) to transmit the downlink ACK message if the resource is not released. Otherwise, an inventory procedure is needed.

There are also other changes to support Topology 2. Impacts on services, entities and interfaces are also updated accordingly.

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-30.

* * * * First Change * * * *

## 6.5	Solution #5: DO-A procedure with configured routing information

### 6.5.0	High-level solution Principles

This solution aims to resolve KI#2 to support DO-A Capable AIoT Devices, the following principles are followed in this solution:

-	This solution has reused the architecture defined in clause 4.2.2 of TS 23.369 [3]. This solution is applied to both direct connectivity and indirect connectivity for Topology 1.

-	This solution is applied to RRC based option for Topology 2

-	The AIoT Device is not required to perform the registration procedure first before sending the DO-A data.

-	AIoT Device sends the DO-A routing information along with the DO-A message for AIOTF to find the proper AF.

-	AF subscribes to the DO-A Data notify by sending a Subscribe Request to the AIOTF (optionally via the NEF).

-	AIOTF derives the corresponding AF according to the routing information in the NAS DO-A Data Transfer Request message and the context generated in DO-A data subscribe procedure.

### 6.5.1	Description

This solution resolves KI#2 for support of DO-A Capable AIoT Devices.

This solution has the following assumptions:

1)	AIoT Device is pre-configured or configured by the network with DO-A routing information.

2)	AIoT Device sends the DO-A routing information along with the DO-A message for AIOTF to find the proper AF.

3)	The NG-RAN selects the correlated AIOTF according to the target AIOTF information if  target AIOTF information is included in the D2R AS message.

4)	AF subscribes to the DO-A Data notify by sending a Subscribe Request to the AIOTF (optionally via the NEF). AIOTF updates AIoT DO-A Profile in ADM with the associated AF subscription.

5)	AIOTF receiving the DO-A data can query the ADM to find the DO-A data associated AIOTF(s) and forward the received DO-A data to the associated AIOTF(s).

6)	AIOTF derives the corresponding AF according to the routing information in the NAS DO-A Data Transfer Request message and the associated (AF) Transaction ID used in DO-A data subscribe procedure.

### 6.5.2	Procedures

#### 6.5.2.1	DO-A data subscribe procedure

Figure 6.5.2-1: Figure for DO-A data subscribe procedure

1.	The AF sends the DO-A data subscribe request to NEF with AF ID and target AIoT device information to subscribe for the notification of the DoA data.

2.	NEF performs AIOTF selection based on the target AIoT device information.

3.	The NEF sends the DO-A data subscribe request to the selected AIOTF with a Transaction ID.

4.	The AIOTF authorizes the AF request to check whether the AF is allowed to subscribe to DO-A delivery, based on the AF authorization data stored in the ADM.

5.	If authorization succeeded, the AIOTF updates AIoT DO-A Profile in ADM with the associated AF subscription.

6.	If the AIOT device is not pre-configured, the AIOTF allocates the routing information for the DO-A data transfer and configure the AIOT device via e.g. AIoT Command procedure defined in clause 6.2.3 of TS 23.369 [3]. In this step, the AIOTF may also allocate a Temp ID to AIoT Device for NAS DOA Data Transfer. The AIOTF configures the AIOT device with the allocated Temp ID, routing information and the AIOTF information.

7.	AIOTF sends response to the NEF with a Transaction ID for the DO-A data subscribe request.

8.	NEF sends response to AF with AF Transaction ID for the DO-A data subscribe request.

#### 6.5.2.2	DO-A data transfer procedure

Figure 6.5.2-2: Procedure for DO-A procedure with configured routing information

0.	DoA routing information (e.g. AF ID or AIOTF allocated DO-A association ID) at the AIoT device side or configured by the network as defined in clause 6.5.2.1. Security information for DO-A traffic may also be pre-configured or configured along with the DoA routing information.

NOTE 1:	DO-A traffic security mechanism will be determined by SA WG3.

1.	The AIoT Device decides to trigger a DO-A message (e.g. detecting DO-A trigger broadcast signal, etc.).

NOTE 2:	DO-A trigger broadcast signal is further defined in RAN WGs.

2.	The AIoT Device sends the AS D2R message ( target AIOTF information, NAS DOA Data Transfer Request message (DO-A data, AIoT device ID, routing information, target AIOTF information,)) to the NG-RAN or UE reader. AIoT device ID may be a permanent Device ID or Temp Device ID. The target AIOTF information is also provided in the AS D2R message if the AIOTF has sent an AIOTF information to AIoT Device during or after the Command procedure, or during the previous DO-A Data Transfer procedure. If AS D2R message is sent to a UE reader, the UE reader sends a RRC message including the AIOTF information and DO-A Data transfer request message to the serving NG-RAN.

3.	The NG-RAN allocates an NGAP ID for the AIoT device and forwards the NAS DO-A Data Transfer Request message in Initial DO-A message to the AIOTF. If target AIOTF information is included AS D2R message for topology 1 or RRC message for the topology 2, the NG-RAN can select the AIOTF according to the target AIOTF information. If the target AIOTF information is not included in AS D2R message for topology 1 or the RRC message for topology 2 or the AIOTF derived from the AIOTF information can not be selected, the NG-RAN can select an AIOTF which can serve the AIoT Device. The selected AIOTF can find the DO-A data associated AIOTF(s) according to the target AIOTF information or by querying the ADM. The serving AIOTF forwards the received DO-A data to the associated AIOTF(s).

4.	Upon receiving the Initial DO-A Message, the AIOTF performs a device check with device profile data retrieved from the ADM to validate the AIoT device.

NOTE 3:	Details for device check for DO-A Data Transfer procedure will be determined by SA WG3.

5.	After the successful device check, the AIOTF derives the corresponding AF according to the routing information in the NAS DO-A Data Transfer Request message and the associated Transaction ID received in step 3 of clause 6.5.2.1. The AIOTF sends the Naiotf_AIoT_Data Transfer message including the DO-A data and AIoT Device ID to the NEF.

6.	The NEF invokes the AIoT_Data Transfer service to forward the DO-A data to the AF according to the AF Transaction ID and AF ID used in DO-A data subscribe procedure.

7.	The AF sends the AIoT_Data Transfer_Response to NEF with AIoT device ID and the response to the DoA data, which may include further data or acknowledgments related to the DO-A Data Transfer process.

8.	The NEF invokes AIoT_Data Transfer_Response to AIOTF. The message includes the DO-A data response and related AIoT device ID.

9.	The AIOTF may allocate an NGAP ID for the AIoT device and sends DL NAS message to NG-RAN with the DO-A data response. In this step, the AIOTF may also allocate a Temp ID to AIoT Device NAS DOA Data Transfer Response message. If the resource for transmitting the DO-A traffic (e.g. NGAP ID) is not released, the AIOTF will reuse the resource for DL NAS message transfer without triggering an inventory procedure. Otherwise, the AIOTF will trigger an inventory and command procedure as defined clause 6.2.3 of TS 23.369 [3] for DL NAS message transfer.

10.	The NG-RAN may further send the DL NAS message to a UE reader. The NG-RAN or UE reader sends the AS R2D message to the AIoT device including the NAS message.

### 6.5.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AIoT Device:

-	Support to initiate DO-A message transfer procedure and send routing information along with the DO-A data.

-	Support to send target AIOTF information along with the DO-A data in the D2R message.

AIOTF:

-	Support to route the DO-A message to proper NEF/AF according to the routing information.

-	Support to allocate the routing information and configure the AIOT device with the routing information and AIOT information.

-	Support to store or update the DO-A data subscribing AF information and associated AIOTF in the ADM.

-	Support to forward the DO-A data to associated AIOTF according to the target AIOTF and routing information.

NG-RAN:

-	Support to select the AIOTF based on the target AIOTF information if provided by the AIoT device.

-	Support to establish NGAP connection with AIOTF for DO-A message transfer.

UE reader:

-	Support to receive the DO-A D2R message from the AIOT device and transmit the DO-A message to the NG-RAN.

* * * * End of Changes * * * *