3GPP TSG-SA WG2 Meeting #172	S2-2510732

# October 17-21, Dallas, US

Source:	Xiaomi

Title:	Conclusion for KI#4 Sensing Data and the Associated Information Collection and Transport

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes the conclusion for KI#4 Sensing Data and the Associated Information Collection and Transport.

1. Discussion

TS 23.700-14 v1.1.0 has documented the interim agreements for KI#4 Sensing Data and the Associated Information Collection and Transport:

7.1.4 Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor's note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

Proposal 1: copy all the interim agreements for KI#4 in clause 7.1.4 into clause 8.4 conclusion for KI#4.

Observation 2: in last SA2#171, the majority companies agreed that Sensing Function may have different Endpoints for control messages and for Sensing Data, in regardless of AMF involved or not.

Proposal 2: to add “Sensing Function may have different Endpoints for control messages and for Sensing Data”.

Observation 3: during the study, event triggered Sensing data report to SF is in scope. But during the interim agreement discussion in last SA2#171, it was agreed that Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Since people think one time sensing data report also includes the event triggered sensing data report case.

But in some other case, when the sensing service is initiated, it also allows SE(i.e. gNB) to report the sensing data based on events. It is not one time, or periodical sensing data report.

Proposal 3: Sensing Entity can support one time, event trigger, periodical Sensing Data to the Sensing Function.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

8.4 Conclusion for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

- Sensing Function may have different Endpoints for control messages and for Sensing Data.

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, event trigger, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, event trigger, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

* * * End of Changes * * * *