3GPP TSG-SA WG2#172	S2-2510761

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	FS_Sensing_ARC: Additional principles for KI#2 on Authorization and Revocation to support sensing service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_Arc / Rel-20

Abstract of the contribution: This contribution proposed additional principles for the KI#2 on Authorization and Revocation to support sensing.

# 1.	Discussion

In SA2#171, several general principles were agreed for the KI#2 for FS_Sensing_Arc on Authorization and Revocation to support sensing service to support sensing. This contribution proposed the additional principles based on the architecture discussion in KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.2	Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9]; and the authorization of the AF is performed by the Sensing authorization functionality, i.e. the Sensing Gateway function;

2)	Authorisation of the AF's Sensing Service Request is performed by the Sensing authorization functionality, i.e. the Sensing Gateway function.

NOTE:	The Sensing authorization functionality is the Sensing Gateway function, which can be collocated with the Sensing Function in deployment.

b)	Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing Gateway function for authorisation of the sensing service request.

d)	Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

# >>>>END OF CHANGES<<<<