SA WG2 Meeting #172	S2-2510787

17 – 21 November, 2025, Dallas, US

Source:	Apple

Title:	[WT#1.2, QoS] WT & KI for QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution suggests an update to WT and KI for WT#1.2 QoS Framework.

# Discussion

This contribution is revision of the version discussed in the CC#3 prior to SA2#172. (Pen-holder’s version S2-2509800 which was postponed in SA2#171 and subsequently revised for the CC). As discussed in the CC#3, this version proposes to add a NOTE for bullet 1 to highlight the dependency with WT#3.2 for AI traffic.

**** First Change ****

# Annex A.X WT#1.2.X: QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

Items that have consensus:

- Investigate whether and what new functionality in 6G QoS framework considering, e.g., AI,  the emerging new traffic characteristics and application needs, is required.

NOTE 1:  The QoS enhancements for AI related traffic are to be studied in collaboration with WT#3.2.

- Whether and how to enhance QoS mechanisms to:

- support QoS targets fulfilling application requirements (e.g., dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

- Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

- Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g., traffic pattern, dynamic QoS requirements and awareness in the application/UE of what can be provided by the network, e.g., maximum bitrate.

NOTE 1: Applications typically have UE and DN aspects.

- Whether and how to enhance QoS monitoring, e.g., to determine the end-to-end packet delay between the PSA-UPF and the UE for the verification of the QoS target fulfillment.

- Study whether and what enhancements to traffic detection are needed to enable QoS differentiation, e.g., in case of frequent IP address change of the server.

NOTE 2: Aspects of traffic detection belong to this WT.

- Study QoS related aspects of interworking with 5GS.

NOTE 3: Interworking aspects will be coordinated with WT#2.

NOTE 4: This WT will coordinate with SA WG4 and RAN WGs.

NOTE 5: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

Items for further discussion:

- Study whether and how to minimize the impact from the network status fluctuations on the upper transport layer.

**** End of Changes ****