3GPP TSG-SA WG2 Meeting #172	S2-2510808

17 - 21, November, 2025, Dallas, USA

# Source:	China Telecom

# Title:	New WID on support for slice-based N3IWF selection for 5G ProSe Layer-3 UE-to-Network Relay

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Slice-based N3IWF selection for 5G ProSe Layer-3 UE-to-Network Relay

Acronym:	TEI20_SN3ProSe

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

In Rel-18 work item 5WWC_Ph2, slice-based N3IWF selection for UE accessing non-3GPP is supported. This feature enable the UE to select the correct N3IWF supporting the S-NSSAI that the UE is going to include in the Requested NSSAI in the subsequent Registration procedure.

While until Rel-20, in 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support, it only supports selection of common N3IWF for the 5G ProSe Layer-3 Remote UE. The common N3IWF selection for UE accessing non-3GPP was introduced in Rel-17, but it is not flexible for the N3WIF usage as it can not apply for the scenario where different N3IWF supports different S-NSSAI.

Therefore, it is necessary to enhance the selection of N3IWF for the 5G ProSe Layer-3 Remote UE to support slice-based N3IWF selection.

The standard work is to align the N3IWF selection in 5G ProSe Layer-3 UE-to-Network Relay with slice-based N3IWF selection for UE accessing non-3GPP in 5WWC_ph2（in 6.3.6 of TS 23.501）, including the slice-based N3IWF selection procedure for 5G ProSe Layer-3 Remote UE and Policy/parameters provision for slice-based N3IWF selection.

4	Objective

The purpose of this Work Item is to add the support for slice-based N3IWF selection for 5G ProSe Layer-3 Remote UE.

The following aspects will be specified:

WT#1: To support for slice-based N3IWF selection for 5G ProSe Layer-3 UE-to-Network Relay, N3IWF selection procedure for 5G ProSe Layer-3 Remote UE and Policy/parameters provision for N3IWF selection are enhanced for slice-based N3IWF selection, by aligning with slice-based N3IWF selection for UE accessing non-3GPP in 5WWC_PH2.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CR(s).

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  | 0.5 | No | None |

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.304 | Addition of support for slice-based N3IWF selection for 5G ProSe Layer-3 UE-to-Network Relay | TSG SA#113 (September 2026) |  |

6	Work item Rapporteur(s)

Heng Nie(China Telecom), nieheng@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified yet

9	Supporting Individual Members

Supporting IM name

CATT?

China Unicom?

Huawei?

Nokia?

Qualcomm?

TNO?

vivo?

ZTE?