3GPP TSG-SA WG2#171	S2-2508293

Wuhan, China, 13 – 17 October, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	Proposal on Interim agreements for KI#3

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#3 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Based on TR 23.700-14 V1.0.0, Solutions #1, #3, #6, #13-#17, #35, and #36 are to address KI#3 (Sensing Entity and Sensing Function Discovery and (Re-)Selection). Key functionalities and architectural options are summarized below:

​a) SE discovery and selection by a Sensing Function (SF)​​

A logical function, the Sensing Function (SF), is introduced to perform the discovery and selection of Sensing Entities (SEs). The SF is responsible for selecting appropriate SEs based on the sensing service request. See Solutions #1, #3, #13, #15, #16, #17, #35. The Access and Mobility Management Function (AMF) may assist in SE discovery and selection in certain architectures. See Solutions #13, #14, #17.

​b) SF discovery and selection by a centralized network function​

A centralized network function (e,g, NEF or a specific gateway function) is responsible for discovering and selecting the SF based on the sensing service requirement from the Application Function (AF), either via local configuration or by querying the Network Repository Function (NRF).

​c) SE Registration and Capability Exposure​ (Solutions #13, #15, #16, #35.)

SEs register their capabilities with a network function (SF or AMF) for subsequent discovery. The registered information includes SE Identifier, supported sensing service area, and sensing capabilities (e.g., acting as Transmitter, Receiver, or both).

​d) Support for Direct or Indirect Connection Architectures​ (Solutions #13, #15, #16, #17, #35.)

Two primary connection architectures between the SE and SF are defined:

​Direct Connection Architecture (SE-SF):​​ The SE registers directly with the SF. See Solutions #15, #16, #35.

​Indirect Connection Architecture (SE-AMF-SF):​​ Interaction between the SE and SF is facilitated via the AMF. See Solutions #13, #17, #35.

​e) Discovery and Selection based on Sensing Service Requirements​ (Solutions #1, #4, #13, #14)

The discovery and selection of the SF and SE are based on the sensing service requirements provided by the AF, which include:

Target sensing service area

Required sensing QoS (e.g., accuracy)

Sensing service type (e.g., motion monitoring).

​f) NF Profile Enhancements​ (Solutions #13, #14, #15.)

NF Profiles are enhanced to support sensing-specific capabilities:

​SF NF Profile:​​ Includes supported sensing area, supported sensing service types.

​AMF NF Profile (in indirect architecture):​​ Includes supported sensing area, which is derived from the areas of its connected SEs.

​Observation 1:​​ Convergence can be seen for the above-mentioned features a) to g).

​Proposal 1:​​ Document the principles derived from features a) to g) in the Agreed Principles for KI#3.

Additionally, diverse views are identified regarding the detailed procedures for SE and AMF discovery and selection within the indirect connection architecture:

​SE Discovery/Selection (Indirect Architecture):​​

​	Option 1 (AMF-assisted):​​ The SF queries the AMF for SEs matching a target area. The AMF returns a list of suitable SEs based on pre-registered capabilities. See Solution #13, #17.

​	Option 2 (SF-based):​​ The SE registers its capabilities with the SF (via the AMF) in advance. The SF subsequently selects an SE based on these stored capabilities. See Solution #16.

​AMF Discovery/Selection (Indirect Architecture):​​

​	Option 1 (Area-based):​​ The SF discovers and selects an AMF by querying the NRF using the target sensing service area. See Solution #13.

​	Option 2 (SE-based):​​ The SF first selects an SE and then selects the AMF to which that SE is connected. See Solution #35.

​Observation 2:​​ For the indirect architecture, two options exist for key selection procedures. The choice impacts the required pre-configuration and interaction flows between SF, AMF, and SE.

​Proposal 2:​​ Normative specifications should at least support direct connection architecture and indirect connection architecture can be supported optionally. The dependencies and necessary pre-configuration for each option shall be clearly defined.

2	Proposal

Proposal 1: Document Item a) to g) in the Agreed Principles for KI#3.

Proposal 2: Normative specifications should at least support direct connection architecture and indirect connection architecture can be supported optionally. The dependencies and necessary pre-configuration for each option shall be clearly defined.

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * * Start of Changes * * * *

7.1.Y	Agreed Principles for KI#3

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following interim agreements on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection, are made:

- A logical function, i.e., the Sensing Function (SF), is responsible for the discovery and selection of Sensing Entities (SEs).

- The discovery and selection of the SF and SEs are based on the sensing service requirements (e.g., target sensing service area, required Sensing QoS, required sensing service type) provided by the Application Function (AF).

- The SF registers its NF profile with the Network Repository Function (NRF) to enable its discovery. The NF profile includes the SF ID/address, NF type, supported sensing area, supported sensing service types.

- A centralized network function (e,g, NEF or a specific gateway function) is responsible for the discovery and selection of the SF, based on local configuration or by querying the NRF according to the AF's sensing service requirements.

​Principles for Direct Connection Architecture (SE-SF)​​:

- The SE registers its sensing-related information (e.g., SE ID, supported sensing capabilities including the supported sensing area) with the SF.

Editor's note: The exact procedure for registration (e.g., during an NG-setup equivalent between SE and SF) is FFS.

- The SF discovers and selects an SE based on the SE's registered capabilities and the sensing service requirements from the AF.

- The SF may construct its NF profile (e.g., its supported sensing area) based on the aggregated capabilities of the SEs that are registered with it.

​Principles for Indirect Connection Architecture (SE-AMF-SF) (Optional)​​:

- The SF is responsible for the discovery and selection of the SE, with assistance from the AMF.

- The SE registers its sensing-related information (e.g., SE ID, supported sensing capabilities including the supported sensing area) with the AMF.

- The SF discovers and selects an SE by querying the AMF, providing the target sensing service area. The AMF returns a list of matching SEs.

- The SF is responsible for the discovery and selection of the AMF, based on local configuration or by querying the NRF using the target sensing service area.

- The AMF's NF profile includes its supported sensing area, which is constructed from the aggregated capabilities of the SEs connected to it.

* * * * End of Changes * * * *