SA WG2 Meeting #171	S2-2508347

Wuhan, China, 13 – 17 October 2025

Source:	vivo

Title:	[WT#1.2, QoS Framework] QoS Framework for 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: it proposed the study contents for 6g QoS framework.

1. Justifications

  - General

The QoS framework mechanism of 5G has achieved significant breakthroughs in resource scheduling and performance guaranteed. However, according to arriving new traffic, new terminal, new services (i.e., beyond communication), new resource (computing resource, AI modal resource, etc.), the 5GS QoS framework, capabilities and adaptability still have obvious limitations needing to be overcome. Overall, it is proposed to considerer the following directions in the evolution of the 6G QoS framework:

- #1: QoS for communication for new traffic, e.g., for traffic of Generative AI, collected data, computing offloaded data, etc.

- #2: QoS for new lightweight AR glasses.

- #3: UE centric dedicated QoS.

-  #4: QoS monitoring.

- #5: QoS for new services.

The motivation, GAP analysis and technical impact for each direction are listed as the following chapters.

  - QoS for new media (e.g. generative AI traffic)

1.2.1 Architectural Motivation:

Generative AI: A category of artificial intelligence that can create new content, including text, images, audio, and video, based on patterns learned from training data.  Unlike traditional AI that focuses on recognizing patterns and making predictions, generative AI is designed to produce novel outputs

Figure-1

As described in TR 22.870 Clause 6.25, Figure-1 shows the traffic characteristics are different according in different AI tasks. Some traffic characteristics examples are shown for generative AI.

- For single IP flow, the traffic Characteristics is dynamical change according to different AI task.

- but in the view of any AI task. The Characteristics is determined.

- The DL traffic is bursty.

Figure-2

- Dynamicity: Traffic fluctuates in real-time with AI tasks.

- Bursty: The arrival of service DL traffic is bursty.

- Small-packet: In the process of text Q&A or thinking, there are many small - packet transmissions, which can effectively relieve users' waiting anxiety; however, the actual load of small packets is very small comparing to the protocol header.

- Predictability: Each AI task has its own flow characteristics (text, image, audio, video). According to the user's intent, some traffic characteristics can be predicted.

- Response Latency: Most latency overhead is spent on computing. Latency-sensitive/insensitive pending to different users or different AI tasks. For Latency-sensitive users, the waiting time easily trigger anxiety. They may interrupt the session (e.g., TTFT, RT latency, etc.).

- Streaming Orderly Transmission: From the perspective of streaming, it has a strict order.

- Other partial features are similar to XR: The feature classification of Image, Audio, and Video is similar to XR, and it also supports SLAM - related data. The definition of XR can be reused.

1.2.2 Gap Analysis:

Static Resource Allocation cannot fulfill dynamic and bursty traffic:

- Service flows (such as AI flows) are no longer limited to a single traffic type, its characteristic change dynamically

- Current QoS Update is slow, e.g. need to update UE, RAN and UPF, but the Characteristics of Data Bursts Change Rapidly.

Limitations of Fixed GBR

- If Guaranteed Bit Rate (GBR) is always used to ensure QoS for traffic, it will cause resource waste during periods of low traffic.

- If rely entirely on non-GBR, it will be unable to cope with the sudden high data rate traffic during peak periods, leading to a decline in service quality.

Difficult to balance Between Cost and Service Quality

- Cost Differences Between GBR and non - GBR: GBR services occupy fixed resources, so their costs are generally higher than those of non - GBR services.

- Conflict Between Cost and Demand:

    - If GBR is always enabled, it will cause cost waste due to small - volume data transmission during business intervals.

    - If use non-GBR entirely, it will be unable to meet the occasional high volume services business demands

1.2.3 Technical Impact:

Technical Impact for QoS for communication for new traffic, e.g., for traffic of Generative AI, collected data, computing offloaded data

- Identify the characteristic of new traffic (e.g. AI, Sensing, Data Collection).

- Study QoS control, congestion control, power saving and rate limitation for the new traffic.

- Study how to fulfil the QoS requirement for the dynamical and bursty traffic, e.g. support flexible and elastic QoS adaptive mechanism.

- Consider what XR mechanisms can be reused for the new traffic.

  - QoS for lightweight AI/AR glasses

Figure 1.3-1:  lightweight AR glasses

Architectural Motivation:

AI/AR Glasses are the most promising 6G mobile terminals. They can bring AI/AR applications and AR and AI traffic into mobile environments, which is conducive to expanding the traffic in 6G network pipelines.

However, limited by its size and weight, the amount of power it can carry is relatively limited, making it highly sensitive to power consumption. In the meantime, it is a kind of XR device, generally, XR traffic drain batter faster.

The users are willing to pay for the energy efficiency for the bits of AR glasses and the bits transfer over AR glasses ca n be treated as valuable bits.

Gap Analysis:

  - 5G Connection drain batter faster and the immersive services also drain batter faster.

  - 5GS doesn’t support differentiated Billing and differentiated Energy-Saving Service to different UE

  - Legacy power saving has to sacrifice the service experience, concurrent large bandwidth and Power Saving is new requirement

  - There is no UE like light weight AI/AR glasses before.

Technical Impact:

  - E.g. Via traffic shaping mechanism to make the UE have more sleep time but the throughput is still guaranteed in the required delay budget

  - Suitable for the traffic without extreme delay requirement, e.g. traffic in AI/AR glasses

  - Study how to support to optimize energy efficiency while meeting the experience of large bandwidth transmission.

  - UE Centric dedicated QoS

1.4.1 Architectural Motivation:

Figure-1: Future 6G terminal

The Future 6G Terminals probably has AI capability and well perceive d understand the user needs:

- Terminals are no longer merely "dumb terminals" but rather "intelligent interaction terminals"

- The AI in the terminal is to well perceive and understand the user real needs and assist the user.

- The UE is capable of greater participation in network decisions related to UEs.

- For the use of the same app, the QoS can vary among different users, and personalized needs will become increasingly prominent.

- UE centric will be popular.

1.4.2 Gap Analysis:

- <Business failure for AF centric dedicated QoS>

In 5GS, dedicated QoS requirements is based on the AF's request and can be understood as AF-centric dedicated QoS. However, AF-centric QoS falls in business for the following reasons:

- Although 5GS standard supports AF request QoS for individual UE. But in the real business, the 3rd party has no motivation to request personalized QoS requirements for individual UEs; for example, Gaming vendor may conduct overall optimization for the gaming app only but not personalized optimization for individual users.

- The 3rd party is lack of the UE's wireless environment and connection intentions. For instance, even with the same conference call App, the QoS requirements differ significantly: such as in high-speed rail, office, or outdoor environments—yet the app itself treats them uniformly. But the user experiences distinct much. E.g. the user in the high-speed rail has more will to pay for the dedicated QoS.

  - Even in the same situation, the personal needs are different. e.g. conference call in the high-speed rail, some people treat the conference important and willing to pay for the dedicated QoS guaranteed, but some other are not. The AF doesn’t perceive this personalized requirement.

- The same QoS requirement at the RAN side can actually vary due to the UE's location in the cell (e.g., signal strength), but the 3rd party has no visibility into that.

- <Feasibility of UE Centric dedicated QoS>

- UE is the role closest to users and it should be the one best understands different individual users personalized requirement.

- The UE owns the entire protocol stack, including the App layer, OS, NAS, and AS and the valuable information (wireless parameters and end-to-end performance measurement metrics) in all layers.

- UE evolves from basic mobile phones to intelligent terminals: evolving toward AI capabilities.

- Great number of users are more willing to pay for higher-value data, such as AI traffic and XR traffic on AR glasses. The UE can capture those users’ demands.

- AI capability and valuable information on the UE side help to well perceive and understanding user’s real requirement for the dedicated QoS.

- <Benefit and business of UE Centric QoS>

- For the use of the same App, personalized QoS needs from different individuals can be fulfilled.

- Meeting the personalized needs of UEs can generate additional billing revenue. The number of AFs is far smaller than that of UEs, and the enhanced market value brought by UEs is much greater than the incremental market value from AFs.

- Each bit of data has different value and pricing, creating new revenue streams in the data traffic billing market.

Observation: from AF-centric to UE-centric: UE involvement drives the personalized needs and those personalized needs further enable differentiated billing, thereby increasing revenue for operators.

1.4.3 Technical Impact:

Technical Impact of QoS for lightweight AR glasses:

- Comparing to AF centric QoS negotiation, study how to support UE centric QoS Negotiationnegotiation.

- How the UE is involved into the QoS policy determination

- What information from UE is useful to reflect the user real personized needs and useful for the QoS negotiation

Void (consider in UP)

  - QoS monitoring over UP

1.5.1	Architectural Motivation

In 6G network, an efficient end-to-end QoS monitoring and information exchange mechanism between the UE and UPF has the following benefits:

a)	Reflecting the actual QoS guarantee result and effects: the UE’s application client could make some adjustments beforehand to adapt to the network status according to the QoS monitoring result.

b)	Providing actual QoS evaluation data to the NW for AI model training: in this case, with the actual QoS monitoring data, the network could provide more suitable QoS policy decision for the UE according to the AI model inference.

c)	Improving end-user QoE experience: when the network transmission performance is getting worse, the network can identify the problem and apply mechanisms to improve the QoE experience according to the QoS monitoring result.

d)	Optimizing the terminal’s control efficiency: providing an accurate data support for real-time optimization of terminal service data flows, and improving the end-user service experience.

For example, the end-to-end QoS monitoring for RTT information between the UE and UPF is needed. This parameter denotes the latency information within the mobile network, so the UE may provide this information to the UE’s application client for the codec adjustment in the application layer or apply this information for AI model training and QoS prediction. Furthermore, the RTT information measurement needs to be real-time in order to be timely used in the UE. So an efficient measurement for this information is necessary to improve the end-user service experience.

Observation 1: An efficient end-to-end QoS monitoring and information exchange mechanism between the UE and UPF is needed.

1.5.2	Gap Analysis

The 5G network supports the QoS monitoring for the specific QoS parameters, e.g., round trip delay, congestion information, available bitrate, etc. The current QoS monitoring mainly focuses on the following two aspects:

a)	RAN information reporting to the UPF via GTP-U header extension, e.g., congestion information, available bitrate;

b)	For the end-to-end network performance, e.g., round trip delay, the measurement includes two parts: the measurement on N3/N9 interface and the measurement for radio interface, wherein:

1)	the measurement on N3/N9 interface can be supported via GTP-U header extension field(s).

2)	the measurement on radio interface is based on the RAN implementation.

Observation 2: QoS measurement between the UE and UPF is not efficiently supported in 5G network.

For example, the QoS monitoring for round trip delay defined in 5G system is divided into following two parts:

a)	Measurement of the delay between the RAN and PSA UPF (i.e., T[RAN<-> UPF]).

This measurement relies on the enhancements on the GTP-U header to include the time stamp information.

b)	Measurement of the delay in the radio interface between the UE and RAN (i.e. T[UE<-> RAN]).

T[UE<-> RAN] is determined by RAN and relies on the RAN implementation. Especially for the UM mode at RAN, it would be hard for the RAN to obtain the value T[UE<-> RAN]. It doesn’t support to include the time stamp information at radio interface.

Then the PSA UPF gets the round trip delay according to the two parts (i.e. Rond Trip Delay = T[ UE<-> RAN] + T[ RAN<-> UPF]). However, the measurement of the delay at radio interface depends on RAN implementation and brings the uncertainty for the measurement of round trip delay.

Figure 1.2-1: QoS monitoring in 5G network

If the QoS monitoring for certain parameters can be measured between UE and UPF directly, it would be more efficient and minimize the impact on the RAN.

1.5.3	Technical Impact

This aspect has the following impact:

-	UE and UPF interaction for QoS monitoring, including:

-	what parameters can be supported by QoS monitoring between UE and UPF (e.g., RTT measurement).

-	procedures and protocol to support UE and UPF interactions.

  - QoS for new services

Figure-1 different dimensional resources and different level QoS

Architectural Motivation:

- The essence of QoS is used to guide optimal allocation of resources. The resources of 6GS are not limited to communication resources alone, but also involve computing resources, model resources, and data resources.

- A new service may need different kinds of resources. In particular, the optimal adaptation of multiple resources is essential to provide integrated end-to-end service guarantees to the Client.

- Such QoS across different types of resources is referred to as task-level QoS, which is an important component of the 6GS QoS framework.

GAP Analysis:

- Legacy 5GS only deploy communication resource and doesn’t deploy the other kind resource.

- The is no task before requires multiple kind resources

Technical Impact:

- Support Task level QoS to fulfill the requirement for multiple resources and meet task-level performance.

- Support the optimal adaptation of multiple resource combinations and the overall performance optimization. For computing: e.g., computing power + communication.

- Support the performance requirement for single resource, to meet distinctive performance requirements: e.g. computing resource only

Dependency: WT#4, #6

Proposal: with regard this direction has dependency with new services in WT#5, 6. It can be postponed until the basic logic definition completion of the new services and then take the QoS for new services into account of the 6G QoS framework.

  - Summary the target for 6G QoS framework:

Figure-1: 6G framework

6G QoS framework targets to build a flexible, efficient, and intelligent QoS system：

- On the one hand, it can meet the distinctive QoS requirements of new traffic (such as AI applications traffic, computing offloading traffic, and data collection traffic, etc.).

- On the other hand, it can efficiently adapt to increasingly variable and bursty service attributes.

- In scenarios involving lightweight terminals, it can optimize energy efficiency while satisfying the experience of large bandwidth transmission.

- In terms of the real personalized QoS requirement, which reflect different individual users’ needs, to support UE involvement in QoS negotiation.

- In terms of the integration of communication and new services, it will demonstrate integrated performance across different kinds of resources and cooperate to provide the optimal QoS experience

- In terms of the allocation of communication resources, it will adapt to meet communication and perception needs, maximizing the commercial value of wireless resources.

- In terms of QoS monitoring, it will realize the real E2E monitoring between UE and UPF to make consumer understand the real QoS performance, and to close the cycle of QoS guaranteed. It can also support the training the NW modal and realized the AI-driven QoS strategies and resource scheduling

- Achieved precise QoS control from multiple dimensions such as flow level, PDU set, and packet.

- A unified, smart, and modularized framework compatible to both communication and on demand deployed beyond communication services.

**** First Change ****

Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements.

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

  - UE provided information to the network for QoS collaboration

  - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

- 6.	Study how to support charging control for the 6G QoS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs. But finally, QoS originating from new services will be revisited into this WT in order to realize a unified QoS framework.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** Second Change (All new text)****

5.X. Key Issue #X: 6G QoS framework

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The study for 6G QoS framework includes the following aspects:

- #1: Study how to support QoS for new traffic, including the following detail:

-   Identify the traffic characteristic for new traffic (such as AI applications traffic, computing offloading traffic, and data collection traffic, etc.).

-   Study the requirement for QoS control, congestion control, power saving for different new traffic.

- #2 Whether and how to efficiently adapt to increasingly variable and busty service attributes. e.g., a flexible and adaptive mechanism will be developed to adjust QoS in real-time.

- #3 Whether and what to monitor in relation to QoS Framework, including:

-  How to enable the QoS monitoring

-  What to measure between UE and UPF to reflect the effort of QoS fulfilment

NOTE 1: QoS monitoring over UP will coordinate with the study of UP architecture.

-#4 Whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

-  UE provided information to the network for QoS collaboration to reflect the user real personized needs and useful for the QoS negotiation

-  Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

-#5 Study how to ensure interworking with 5GS.

-#6 Study how to support charging control for the 6G QoS. NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs. But finally, QoS originating from new services will be revisit into this WT in order to realize a unified QoS framework.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** End of Changes ****