SA WG2 Meeting #171	S2-2508415

Wuhan, China, October 13 - 17, 2025

Source: 	vivo

Title: 	Interim agreements on KI#2: Authorization and Revocation to Support Sensing Service

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides interim agreements on KI#2: Authorization and Revocation to Support Sensing Service.

1	Discussion

It is proposed to provide interim agreements on KI#2.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

-	Sensing service authorization:

-	If untrusted AF is the sensing service consumer, the NEF performs authorization for accepting the sensing service request. If trusted AF is the sensing service consumer, the NRF performs authorization for accepting the sensing service request. The details for authorization for accepting the sensing service request will be coordinated with SA3 and will be decided in normative phase.

-	The SF is enforcement point for the sensing service (SS) authorization based on per-SS authorization profile and sensing requirements, and after successful SS authorization, the SF triggers sensing configures.

-	The SF may adjust the sensing configurations based on the result of SS authorization, e.g. configures the allowed service area, and excludes the disallowed service area from the requested sensing area.

-	The SF pre-configures with the per-SS authorization profile.

-	The per-SS authorization profile is criteria of specific sensing service, which may include:

-	The SS’s allowed/unallowed service area

-	The SS’s allowed/unallowed service time

-	Sensing entity authorization:

-	The SF is enforcement point for the sensing entity (SE) authorization based on per-SE authorization profile and sensing requirements, and after successful SE authorization, the SE can be selected as sensing entity.

-	The SF follows the same procedure with sensing entity discovery and selection as described in 7.1.3 to retrieves per-SE authorization profile.

-	The per-SE authorization profile is criteria of specific sensing entity, which may include:

-	The SE’s allowed/disallowed service area

-	The SE’s allowed/disallowed service time

NOTE:	Coordination with SA3 on sensing service authorization and/or sensing entity authorization is needed.

-	Revocation:

-	The sensing requirements changes may cause revocation of authorization:

-	The AF sends request to the SF to revoke the requested sensing service, and the SF cancels the sensing service.

-	The AF sends request to the SF to update the sensing requirements, and the SF cancels the sensing service if the network cannot fulfil the new sensing requirements, the SF reselects sensing entity or reconfigures sensing configuration if the network fulfils the new sensing requirements.

-	The per-SE authorization profile or current status changes may cause revocation of authorization:

-	Upon the update of per-SE authorization profile or changes of current status, e.g. the current time is not within the SE’s allowed service time, the SF reselects sensing entity to fulfil the old sensing requirements, the SF cancels the sensing service if the network cannot fulfil the old sensing requirements.

-	The per-SS authorization profile or current status changes may cause revocation of authorization:

-	Upon the update of per-SS authorization profile or changes of current status, e.g. the changed SS’s allowed service area is out of the requested sensing area, the SF reconfigures the sensing configuration or reselects sensing entity to fulfil the old sensing requirements, the SF cancels the sensing service if the network cannot fulfil the old sensing requirements.

# * * * * End of change* * * *