3GPP TSG-SA WG2 Meeting #171	S2-2508506

# Wuhan, China, October  13th – 17th, 2025				       		      (revision of S2-2506276)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6072 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Support of MoQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following cases have not been studied and do not appear to be fundamental to have a workable R19 MoQ deployment - The case where the UE acts as a MoQ PUBLISHER - Different MoQ application servers can correspond to different MoQ relays that are deployed on the same PSA UPF Following wording is misleading as operarors are not really doing implementations The mappings between the MoQ Metadata and the PDU Set Information can be based on local configuration, or operator-determined implementations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the overall set of features that are or that are not specified - In this release, how to deal with the case where the UE acts as a MoQ PUBLISHER is not specified. - In this release, how to deal with the case where the UPF acts as a content cache is not specified. Conversely - The mappings between the MoQ Metadata (carried as defined in draft-ietf-moq-transport [201] and in TS 29.561 [132]) and PDU Set Information is specified in TS 29.561 [132] Clarify - Different MoQ relays each corresponding to a different network instance can be deployed on the same PSA UPF. Instead of following incorrect text - Different MoQ application servers can correspond to a MoQ relay deployed on a PSA UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear MoQ functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of 3096 -4637 NOT HANDLED TWICE |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.37.9.2	Usage of Media Over QUIC in order to Handle end-to-end encrypted XR and media flows

When a PSA UPF that supports MoQ relay functionality has been selected, encrypted Metadata associated with traffic can be transported between the UPF and the AS via Media over QUIC (MoQ), IETF draft-ietf-moq-transport [201].

A PSA UPF supporting the MoQ relay functionality can be selected by SMF with the consideration of the DNN and S-NSSAI provided by UE.

NOTE 1:	The application client in the UE uses MoQ Transport protocol and establishes a MoQ connection using the address of MoQ Relay between the application client and MoQ relay in UPF as defined in MoQ, IETF draft-ietf-moq-transport [201].

The SMF may get the address of MoQ Relay from the UPF during N4 session management procedure as defined in the clause 4.4.1 of TS 23.502 [3]: the SMF indicates the PSA UPF to provide MoQ relay address in N4 Session Establishment/Modification Request and the PSA UPF returns MoQ relay address in N4 Session Establishment/Modification Response.

NOTE 2:	The application client can obtain the MoQ relay address via application layer-based schemes.

In the case the EASDF based DNS procedure as described in clause 6.2.3.2.2 of TS 23.548 [130] is used, when the SMF has determined that the received FQDN in the DNS message report from EASDF is for MoQ, the SMF gets the address of the MoQ relay in a suitable PSA UPF and then the SMF provides back a DNS message handling rule with the address of MoQ relay to instruct EASDF to return the address of MoQ Relay in PSA UPF by setting the Forwarding Action as Respond directly to the DNS request according to TS 23.548 [130]. In this case a local PSA UPF supporting MoQ Relay maybe selected by SMF.

NOTE 3:	It is assumed an application client can send a DNS query to resolve FQDN.

NOTE 4:	The FQDN can be the FQDN for MoQ traffic and preconfigured in SMF or it can be the FQDN of the MoQ Relay and preconfigured in SMF and application client. It is assumed there is SLA between operator and application service provider for using MoQ relay functionality provided by operator.

NOTE 5:	In some other case, the application client can also obtain the MoQ relay FQDN or IP address from the MoQ application server via the query during the MoQ session establishment between the application client and the MoQ application server as in MoQ, IETF draft-ietf-moq-transport [201], which is out of the scope of 3GPP. The MoQ application server can get the MoQ relay FQDN or IP address via local configuration.

NOTE 6:	Different MoQ relays each corresponding to a different network instance can be deployed on the same PSA UPF.

The AF provided Protocol Description indicating Media over QUIC Transport, IETF draft-ietf-moq-transport [201] can be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45]. Based on the PCC rule, the SMF instructs the PSA UPF to perform PDU Set Information identification. Upon receiving the MoQ SUBSCRIBE message from UE application client, the MoQ relay can establish n upstream MoQ subscription to the MoQ application server if there is no existing subscription yet.

For the downlink service data flow using MoQ Transport, the PSA UPF can identify the PDU Set Information from the MoQ Metadata that is extracted by the MoQ relay and provide the available PDU Set Information to the RAN in the GTP-U header.

NOTE 7:	The mappings between the MoQ Metadata (carried as defined in draft-ietf-moq-transport [201] and in TS 29.561 [132]) and PDU Set Information is specified in TS 29.561 [132].

NOTE 8: In this release, how to deal with the case where the UE acts as a MoQ PUBLISHER is not specified.

NOTE 9: In this release, how to deal with the case where the UPF acts as a content cache is not specified.

# * * * * End of changes * * * *