3GPP TSG-SA2 Meeting #171	S2-2508645

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0388 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS broadcast support for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.247 contains the term “NR NTN Intended Service Area” and states that related encoding and procedures are defined in TS 38.413 and TS 38.331. However, those specs use the terminology “MBS Intended Service Area List”: #### TS 38.413 defines #### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. MBS Intended Service Area List; 0..<maxnoofIntendedAreasforMBS>. >Intended Service Area Coordinates; M; 9.3.1.273. #### 9.3.1.273 Intended Service Area Coordinates This IE provides information on the Intended Service Area Coordinates for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Intended Service Area Coordinates; M; OCTET STRING; Includes the areaCoordinates contained in the IntendedServiceAreaInfo IE as defined in TS 38.331 [18]. #### TS 38.331 defines #### – SIBXX SIBXX contains the information of the intended service area(s) of an MBS broadcast service(s) in an NTN cell. SIBXX information element -- ASN1START -- TAG-SIBXX-START SIBXX-r19 ::= SEQUENCE { intendedServiceAreaList-r19 IntendedServiceAreaList-r19 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } IntendedServiceAreaList-r19 ::= SEQUENCE (SIZE (1..maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19 IntendedServiceAreaInfo-r19 ::= SEQUENCE { intendedServiceAreaId-r19 MBS-IntendedAreaID-r19, areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { center-r19 ReferenceLocation-r17, radius-r19 INTEGER(0..65535) } } } MBS-IntendedAreaID-r19 ::= INTEGER (1..maxNrofMBS-Area-r19) -- TAG-SIBXX-STOP -- ASN1STOP SIBXX field descriptions. intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.. polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.. |
| Summary of change: |  | Clarify that “NR NTN Intended Service Area” denotes the MBS Intended Service Area List as defined in TS 38.413 and 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misallignment with RAN terminology may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6,21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.21	Support of Broadcast MBS Session over NR non-terrestrial networks

TS 38.300 [9] specifies the additional NG-RAN procedures to support Broadcast MBS Session over NR non-terrestrial network (NTN).

Specifically for NR NTN, an MBS service area in the 5GC may be denoted with an NR NTN Intended Service Area in combination with a TAI list and/or cell ID list, for instance to denote an area smaller than a cell and the AMF may provide the gNB with the MBS service area information as a TAI list, cell ID list, and/or MBS intended service area list as defined in TS 38.413 [15]. The NR NTN Intended Service Area is used in NG-RAN to further restrict the MBS service area denoted by TAIs and/or cell IDs as defined in TS 38.300 [9].

Based on local configuration (e.g. related to the received geographical area from an AF), the NEF may determine that an MBS session is targeting a NR NTN network and may be configured to include the NR NTN Intended Service Area derived from the geographical area information in an MBS service area, and to provide this information to the MB-SMF as part of the MBS session creation procedure (see clause 7.1.1.2) and the MB-SMF then forwards the MBS Service Area during MBS Session Start procedures for Broadcast (see clause 7.3.1) towards the AMF. The AMF provides the MBS service area information towards NG-RAN. NG-RAN delivers the MBS Intended Service Area List within the NR NTN Intended Service Area to the UE, and the UE handles this information as specified in TS 38.331 [28].

NOTE 1:	It is recommended that based on service agreement the AF uses a geographic area in an encoding compatible with TS 38.413 [15] for NR NTN.

NOTE 2:	The NR NTN Intended Service Area is transparently transmitted in 5GC, i.e. the MB-SMF selects the AMF based on TAIs, and the AMF also selects NG-RAN nodes in the same manner. (Cell IDs are provided together with related TAIs to enable this selection).

NOTE 3:	Location-dependent MBS services in Clause 6.2 for broadcast communication over NR NTN are not supported in this Release.