3GPP SA WG2 Meeting #171	S2-2508912

13-17 October 2025, Wuhan, China

Source:	Qualcomm

Title:	Modification to KI "voice services for 6G"

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / R20

Abstract of the contribution: This contribution propose a delta for the KI of "voice services for 6G" based on the latest "pen holder" document in S2-2508128.

# 1.	Discussion

In SA2#170 and the subsequent conference call it was discussed whether "voice fallback" to 5GS/EPS for normal and emergency voice calls should be handled as part of WT#1.4 (and the related KI) or instead as part of the interworking and migration WT/KI.

In this paper, we argue that voice fallback should be covered as part of WT#1.4 because the solutions for voice fallback may diverge from normal interworking scenarios, which are triggered due to UE mobility.

The fundamental difference is that mobility procedures are usually governed by radio condition measurements performed by the source RAN node as a basis for triggering a handover or an RRC redirection. This means for the mobility scenario, even if the handover fails, it is reasonable to expect that in most cases the UE will "stay" at the target cell, e.g., in IDLE mode, since the radio conditions are typically better or at least sufficient at the target cell.

For EPS fallback as it was designed for 5G, the trigger to initiate a handover or an RRC redirection works differently: handover or RRC redirection are triggered based on the existence of a QoS Flow with 5QI-1 that is used for voice. In case of EPS fallback, if the handover fails, the target cell may however not be the optimal cell from radio conditions’ point view. This is because the UE at AS or NAS layers is not aware why the handover was triggered. From our implementation and testing experience, the error handling of the 5G EPS fallback solution therefore resulted in unnecessarily complex "cross-layer" interactions.

For 6G, if voice fallback is pursued either to EPS or 5GS, we propose to allow the option to study solutions where the UE is aware of the fallback, e.g. by triggering the voice fallback through a NAS procedure. This can be similar to the 5G procedure for emergency fallback, where the UE is aware at least at NAS layer that a fallback is triggered.

In summary, solutions for voice fallback may differ from general mobility procedures. To allow for such targeted solutions to be discussed and studied, we prefer that voice fallback is are handled as part of WT 1.4.

# 2.	Proposal

WT#1.4 and KI "voice services for 6G" to study the need and potential solutions for voice fallback for normal and emergency calls. A text proposal shown as "delta" on top of S2-2508128 is proposed below.

*** First Change *** (changes on top of S2-2508128)

# 5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (using 5GS as the starting point for discussion) – the following aspects will be studied:

-	How a UE supporting 6G RAN selects and connects to a network for voice service.

-	Whether and how to update the IMS specification to support an IPCAN in the 6GS.

-	How the 6GS supports IMS based voice over the 6G access network.

-	Whether and how to support fallback for voice services to 5GS and EPS.

Editor’s Note: It is TBD if voice services for 6G NTN will be studied as part of this Key Issue or by a Key Issue related to WT7.

Editor’s Note: It is still to be determined if other capabilities will be considered as part of this KI.

# 5.Q. Key Issue #Q: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services for the 6G system using the capabilities defined for voice service in Key issue P.

Additionally, this key issue will study:

-	How to enable emergency voice service for 6GS.

-	How to establish emergency session via 6GS for the emergency voice service.

-	How to identify emergency services in 6GS such that the required priority for an emergency call is applied.

-	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

-	Whether and how to support fallback for emergency voice services to 5GS and EPS.

Editor’s Note: It is still to be determined if other capabilities will be considered as part of this KI.

*** End of Changes ***