3GPP TSG SA2 Meeting #171	S2-2508929

13 – 17 October, 2025, Wuhan, China

Source:	Ericsson

Title:	KI#2 - Interim agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: This contribution proposes some updates to the interim agreements.

# 1. Discussion

This PCR proposes some interim agreements.

One principle that may be derived from the architectural assumption indicating that how the UPF detects wrong or malformed or duplicated packets is that the UPF can detect that certain source address is sending abnormal traffic and is performed based on the existing Packet Detection Rules, e,g containing an application detection filter that extends the inspection and considers the dynamics of the flows. Once the abnormal traffic is detected using the application detection filter in the PDR, the existing N4 rules can be applied. Therefore, the following principle is proposed:

- Principle 1: UPF can detect abnormal traffic and take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

In addition, some statistics and predictions can be provided by NWDAF, that are not specific to a UPF, related to the user plane from an application or from a UE that is causing abnormal traffic, as such an application may generate a large amount of user plane traffic in the network, although it may appear as “normal” traffic in one UPF. Therefore, the following principle is proposed:

- Principle 2: NWDAF provides statistics and predictions to identify abnormal traffic in the network, type of anomaly and the source of the traffic based on the input provided by the UPFs in the network.

Then, when it comes to the consumer of an AnalyticsID that reports that certain source causes abnormal traffic and the type of anomaly, there are proposal that SMF (and PCF) or UPF are the consumers of this Analytics, some observations about the consumer of this AnalyticsID:

- The actions to improve the user plane performance are not unique, may be different depending on the e.g. detected anomaly or even for the same detected anomaly may be different depending on the source of the anomaly or the target user, for example, selection of a UPF may be an action taken based on the analytics result, that is taken by the SMF, while dropping packets from a source application is performed by the UPF based on the N4 rules provided by SMF.

Observation: NWDAF produces statistics and predictions, as such it does not know what the action to be performed is, only the consumer knows the action to be taken.

- The UPF receives authorized N4 rules to enable detection of application traffic for services that are authorized for the PDU Session, those reside in the PCF, as such the NWDAF does not know which application traffic is authorized for a user and take the risk to create inconsistencies at the UPF that receives information about applications that cause anomalies that may not be authorized for the PDU Session, and for those how to treat them do not depend on analytics but on operator policies in the PCF.

  - Principle 3: The action to be performed as result from analytics are part of the decisions made by the PCF or by the SMF for the PDU Session, therefore the PCF or the SMF are the only two consumers. The UPF is not a consumer of this Analytics.

  - Principle 4: To minimize the signalling between SMF and UPF, mitigation actions can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

# 2. Text Proposal

It is proposed to capture the following principles into TR 23.700-04:

- Principle 1: UPF can detect locally abnormal traffic and take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

- Principle 2: Analytics provided by NWDAF can be used to identify abnormal traffic that are not local to the UPF but in the area of the network covered by the NWDAF including type of anomaly and the source of the traffic. The NWDAF identification is based on the input provided by the UPFs in its area of the network.

- Principle 3: The action to be performed as result of analytics are part of the decisions made by the PCF or by the SMF for the PDU Session, therefore the PCF or the SMF are the only two consumers. The UPF is not a consumer of these analytics.

- Principle 4: To minimize the signalling between SMF and UPF, mitigation actions can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

*** Start of changes ***

*** Next change (all new text) ***

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information for input data filtering to UPF. Other actions that can be taken to reduce load when reporting input data to NWDAF are support the provisioning of the skip reporting instruction information in the subscription request, then the UPF do not send the event reports if they meet the skip reporting instruction, or UPF may bundle event reports of multiple subscriptions in a same Notify request, see TS 29.564 [x].

#### 7.1.2.2	Agreed Principles for Use Case #1

Principles for detection of abnormal traffic locally at the UPF:

- As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 1: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

Principles for detection of abnormal traffic in the area covered by NWDAF:

- The NWDAF provides statistics and predictions that identifies abnormal traffic in its area, type of anomaly (so called exception) and the source of the abnormal traffic based on the input provided by the UPFs in the network area.

- The service consumer of the analytics service may be SMF, PCF, OAM and AF. The consumer may take the exceptions reported by NWDAF into account. Possible actions are determined by the consumer; examples are blocking traffic from the source of the abnormal traffic at the UPF when the exception indicates too many fragmented packets (to avoid excessive degradation in UPF performance).

NOTE 2: The UPF is not a consumer of this Analytics ID.

-

-	To derive the output analytics, the following input data may be collected from the UPF, using existing UPF event exposure services for example extending “USAGE DATA MEASUREMENTS” with new measurement types, to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic.

- 	type of identified anomality, i.e. exceptions.

- 	source of the abnormal traffic.

Principles to minimize the signalling between SMF and UPF when applying mitigation actions applicable to any UE:

- To minimize the signalling between SMF and UPF, mitigation actions can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

*** Next change***

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-

-

-	Consumer actions are FFS.

*** End of changes ***