SA WG2 Meeting #171	S2-2509154

Wuhan, China, October 13 – 17th, 2025

Source:	NEC

Title:	WT#1.2, Update to Network Sharing in the 6G system

Document for:	Discussion / Approval

Agenda Item:	WT#1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to update Network Sharing in the 6G system KI more specially related to providing non-3GPP access services under INS coverage.

# 1. Justifications

S2-2506096 includes WT#1.2

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

The discussion paper discusses following aspects support network sharing aspect more specifically indirect network sharing

Architectural Motivation:

In the 3GPP 5G System (5GS), indirect network sharing is an advanced method where multiple operators can share a single Radio Access Network (RAN) 3GPP Access without having direct connections to it. Instead, communication is routed through the core network of one of the operators, known as the hosting operator. Participating Operator offers services to its customers using the shared RAN but does not have a direct connection. when a customer of a participating operator connects to the shared RAN, their signaling and data traffic are first routed through the hosting operator's 5G Core (5GC). The hosting operator's core then forwards this traffic to the participating operator's core network. Essentially, this model functions similarly to a home-routed roaming scenario, but it's designed specifically for sharing infrastructure within the same country to improve coverage and reduce costs, especially in rural or hard-to-reach areas.

The UE supporting both 3GPP access and non-3GPP access in the HPLMN shall attach to  the N3WIF of the HPLMN and both accesses shall be connected to the same AMF.

Gap Analysis:

Currently when the UE is in coverage of INS network sharing can not access non-3GPP services at the same time. This brings serious problem that the user can not access services through non-3GPP access while the UE is registered to the 3GPP access. This will have serious implications on the user experience.

.

**** First Change ****

### A.1.2.1	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- 	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

NOTE 1: non-3GPP access services should be possible when the UE is in the INS coverage and operator wants to deploy non-3GPP access.

Editor's note:	It is FFS whether other aspects of network sharing in 6G can be added in this clause.

**** End of Changes ****