SA WG2 Meeting #S2-172	S2-2509863

17 - 21 November, 2025, Dallas, USA

3GPP TSG-RAN WG1 Meeting #122bis	R1-2508096

Prague, CZ, 13th – 17th October, 2025

Title:	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

Response to:	(R1-2506707) LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

Release:	Release 20

Work Item:	FS_5GSAT_Ph4_ARC

Source:	RAN1

To:	SA2

Cc:	RAN2, SA4, CT1, SA3, SA1

Contact person:	Alberto Rico

albertor@qti.qualcomm.com

# 1	Overall description

RAN1 has discussed the following question from SA2:

Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

RAN1 reply:

Assuming line-of-sight condition for the duration of the call, no change in large scale parameters (e.g. fixed shadowing), and using an NTN TDL-C channel model (which is a channel model previously used by RAN1 and SA4 evaluations), the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions (e.g. up to 10% packet error rate)

For the case of line-of-sight not being guaranteed for the duration of the call, an intermittent blockage (e.g. according to the land mobile satellite channel model in TR 38.811) may still occur in practice, in which case the probability of having 16 or 64 packets consecutively lost may not be negligible. RAN1 has not reached consensus on whether to consider this case.

# 2	Actions

To SA2:

ACTION: 	RAN1 asks SA2 to take the above information into account.

# 3	Dates of next WG RAN1 meetings

RAN1#123	 2025-11-17    -    2025-11-21	 Dallas, US

RAN1#124	 2026-02-09    -    2026-02-13	 Gothenburg, Sweden