3GPP TSG-WG SA2 Meeting #172	S2-2509886

Dallas, US, 17 - 21 Nov 2025	(revison of S2-250)

Source:	OPPO

Title:	Proposed way forward on contentious issues

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes way forward on contentious issues identified on SA2#171.

# 1	Discussion

Some open issues are identified during the discussion of interim agreements of KI#1-4 on SA2#171 for further discussion, including:

- Whether to introduce a Gateway SF?

- Whether to have single endpoints or 2 endpoints between gNB and SF?

- Whether to transport the control signalling directly between gNB and SF or indirectly via AMF?

- Whether to transport the sensing data directly between gNB and SF or indirectly via AMF?

- Which transport layer protocol is used to carry sensing layer protocols (e.g. LCS-UPP) between gNB and SF?

- What are the format and semantics for the “sensing data” and the “associated information” provided by the gNB to the SF for result generation?

- Whether event triggered sensing data reporting is supported by the gNB?

- What are additional parameters that gNB expects to receive from the SF to perform the sensing operation and provide Sensing Data with the expected characteristics?

- What are the sensing capabilities of a gNB acting as a Sensing Entity and how they are provided?

Below are evaluations and proposed way forward:

A. Whether to introduce a Gateway SF?

A Gateway SF (GSF) is a SF supporting a sub-set of SF functionalities as shown in the table below with the comparison with SCF, SPF, NEF and GSF.

| SF functionalities | SCF | SPF | NEF | GSF |
| --- | --- | --- | --- | --- |
| Authorization of untrusted AF |  |  | Y |  |
| Authorization/revocation of sensing service request | Y |  |  | Y |
| Storage of sensing authorization information | Y |  |  | Y |
| Configuration of sensing data collection | Y |  |  | Y |
| Sensing result generation |  | Y |  |  |
| Sensing result storage |  | Y |  |  |
| Sensing result exposure | Y | Y |  | Y |
| SF/SPF discovery | Y |  |  | Y |
| SE discovery | Y |  |  |  |

Key Benefits of Introducing the GSF:

- The GSF acts as a central gateway for AF requesting sensing services.

- By handling authorization/revocation and storing authorization information centrally, the GSF ensures consistent policy enforcement across the entire network. This is similar to the role of a GMLC in eLCS.

- Decouples NEF from Service Logic: NEF handles generic, service-agnostic authorization for untrusted AFs. The GSF then handles the sensing-specific authorization and service logic.

- The GSF's centralized role allows the SPF to be deployed locally at the edge, close to gNB. By processing sensing data locally (at the edge SPF), the architecture avoids the need to transport massive, unprocessed data streams from the gNB to a central cloud. Only the refined, smaller sensing results are sent to the central GSF for exposure to the AF. This dramatically reduces bandwidth usage and latency.

- The architecture allows for independent scaling of different functions.

  - The central GSF can be scaled to handle a growing number of service requests and AF connections.

  - The edge SPFs can be scaled based on the sensing processing load in specific geographical areas.

Observation 1: Introducing the GSF allows centralized management with distributed, low-latency processing. GSF is an instance of SCF that is deployed as a gateway SCF. If both GSF and SCF are introduced, the separation of their functionalities should be clarified.

Proposal 1: GSF is introduced as an instance of SCF. When both of them are deployed, the functional split between SCF and GSF can be discussed during normative phase.

- Whether to have single endpoints or 2 endpoints between gNB and SF?

- Whether to transport the control signalling directly between gNB and SF or indirectly via AMF?

- Whether to transport the sensing data directly between gNB and SF or indirectly via AMF?

Those 3 questions can be evaluated based on the table below.

| Feature | Direct SF-gNB Connection | Communication via AMF (like NRPPa) |
| --- | --- | --- |
| Latency | Low (Shorter path, fewer hops) | Higher (Longer path, AMF processing delay) |
| AMF Load | No Impact (Traffic bypasses AMF) | Heavy Impact (Sensing signaling/data loads the AMF) |
| Data Throughput | High (Dedicated, optimized data pipe) | Limited (Uses general-purpose signaling channels) |
| Reliability & Availability | High (Independent of AMF failures) | Dependent on AMF (AMF outage disrupts sensing) |
| Deployment Flexibility | High (Ideal for edge computing) | Lower (Centrally routed via AMF) |

Observation 2: The core advantages of the direct connection stem from efficiency, performance, and flexibility, which are critical for the high-bandwidth, low-latency demands of sensing services.

Proposal 2: Both the control signalling and sensing data are transported directly between gNB and SF, separate endpoints can be used for control signalling and sensing data delivery, i.e.

- 2 endpoints are between gNB and SF

- Control signalling are transported directly between gNB and SF

- Sensing data is transported directly between gNB and SF

- Which transport layer protocol is used to carry sensing layer protocols (e.g. LCS-UPP) between gNB and SF?

2 Options can be considered on direct data connection establishment between Sensing Entity (e.g. a gNB) and the data processing function.

-	Option 1: TLS

-	Option 2: GTP-U

For the determination of which option to be adopted for 5GA R20 sensing, the following factors will be considered:

- Impact to SF deployment: SF may be split into SCF (centrally deployed without sensing layer) and SPF (distributed with sensing layer supported). In such case, an interface may be introduced between SCF and SPF for sensing service control and management. Sensing layer is introduced for the interaction between gNB and SF for sensing service operation (e.g. NRPPa alike protocol).

- Impact to gNB: To be evaluated by RAN WGs

- Impact to CT: How much stage 3 work is expected.

- Future proof to 6G: consistent solution for sensing data collection from gNB and UE is preferred

|  | Impact to SF deployment | Impact to gNB | Impact to CT | Future proof to 6G |
| --- | --- | --- | --- | --- |
| Option 1 | Easily support | TBD | High | Yes |
| Option 2 | Complicated for SCF-SPF interaction | TBD | Low | No |

NOTE: Items marked with TBD require further evaluations, especially the coordination with RAN WGs

Observation 3: Option 1, i.e. TLS based approach has more advantages. Coordination with RAN3 is needed.

Proposal 3: TLS is used as the transport layer protocol for the direct connection between gNB and SF. Include Questions E into the RAN LS and align with RAN for solution development in normative phase.

- What are the format and semantics for the “sensing data” and the “associated information” provided by the gNB to the SF for result generation?

- Whether event triggered sensing data reporting is supported by the gNB?

- What are additional parameters that gNB expects to receive from the SF to perform the sensing operation and provide Sensing Data with the expected characteristics?

- What are the sensing capabilities of a gNB acting as a Sensing Entity and how they are provided?

Observation 4: Questions F to I are subject to RAN WG to answer.

Proposal 4: Include Questions F to I into the RAN LS and align with RAN for solution development in normative phase.

# 2	Proposal

Proposal 1: GSF is introduced as an instance of SCF. When both of them are deployed, the functional split between SCF and GSF can be discussed during normative phase.

Proposal 2: Both the control signalling and sensing data are transported directly between gNB and SF, separate endpoints can be used for control signalling and sensing data delivery, i.e.

- 2 endpoints are between gNB and SF

- Control signalling are transported directly between gNB and SF

- Sensing data is transported directly between gNB and SF

Proposal 3: TLS is used as the transport layer protocol for the direct connection between gNB and SF. Include Questions E into the RAN LS and align with RAN for solution development in normative phase.

Proposal 4: Include Questions F to I into the RAN LS and align with RAN for solution development in normative phase.