SA WG2 Meeting #172	S2-2509887

13 – 17 Sep, 2025, Wuhan, China

Source:	OPPO

Title:	[WT#5] Discussion on terminologies, use cases and updates to WT

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper is to propose terminologies and use cases of data framework to facilitate the agreement on WT#5 with the proposed updates.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

#### Terminology proposals

Data framework: A structured system that orchestrates the data operation, including data collection from producer, data transfer to a storage or processing entity for result generation and data exposure to consumer.

Data producer: An entity (e.g., UE, gNB, CN NF, AF) that originates or provides data by performing data collection based on a provided configuration.

Data consumer: An entity (e.g., UE, gNB, CN NF, AF) that utilizes data from the framework.

Data initiator: An entity (e.g., UE, gNB, CN NF, AF) that triggers data exposure procedures to request data from the framework. Data consumer and data initiator can be the same entity.

Data collection: The set of procedures performed by a data producer to log or measure data from its source, which is initiated based on a provided data collection configuration and leads to the invocation of data transfer.

Data transfer: The set of procedures for the delivery of collected data from a data producer to a data storage or processing entity, configured and controlled by the data control, storage, and processing entities based on a data transfer configuration.

Data exposure: The set of procedures triggered by a data consumer or a data initiator for one or multiple data consumers to access, retrieve, or receive processed data from a data control, storage, or processing entity within the framework.

Data storage: An entity responsible for the retention and management of data within the framework after it has been transferred.

#### Use case examples

#### UC1: Sensing

- Data type: sensing data

- Result data: sensing result

- Data producer: UE, gNB

- Data consumer: UE, gNB, CN NF, AF

- Data initiator: UE, gNB, CN NF, AF

- Data collection: Based on determined sensing mode, UE and/or gNB performs sensing measurements based on configurations from SF to collect sensing data for result generation.

- Data transfer: Sensing data is transferred from UE/gNB to SF for result generation.

- Data exposure: The sensing service request can be triggered and used by UE (e.g. for ADAS), gNB/CN NF (e.g. for sensing assisted communication) or AF (e.g. for generating high definition 3D map)

- Data storage: The sensing data / sensing result can be stored that can be accessed by SF for future usage or for usage of historical data

In this 6G sensing use case, devices like smartphones (UEs) and base stations (gNBs) act as data producers, collecting sensing data based on instructions from Sensing Function (SF).

The sensing data is transferred to the SF, which processes it into useful results. These results are stored in a data storage system, allowing for historical analysis and future use.

The process is initiated by a data initiator, which can be any 6GS entity or application. Finally, the processed information is exposed to data consumers. For example, a vehicle's navigation system (UE) uses it for collision avoidance, the network (gNB) uses it to improve signal quality, and a mapping service (AF) uses it to create live 3D maps. This creates a seamless loop where the network's sensing capability is a shared service for both communication enhancement and new user applications.

#### UC2: AI assisted positioning

- Data type: positioning data

- Result data: positioning AI model

- Data producer: UE, gNB

- Data consumer: LMF or NWDAF

- Data initiator: LMF or NWDAF

- Data collection: LMF may require positioning AI model from NWDAF or trained by itself. UE and/or gNB performs positioning measurements based on configurations from LMF to collect positioning data for AI model training.

- Data transfer: Positioning data is transferred from UE/gNB to LMF/NWDAF for AI model training.

- Data exposure: The positioning AI model is used by LMF to improve positioning performance. If the AI model is trained by NWDAF, the NWDAF transfers the positioning AI model to the LMF.

- Data storage: The positioning data or positioning AI model can be stored that can be accessed by LMF for future usage or for usage of historical data

This use case demonstrates an AI-enhanced positioning mechanism within a 6G network. The process is initiated by LMF or NWDAF, requesting a positioning AI model to improve location accuracy.

UE and gNBs, then perform Data Collection. They conduct precise positioning measurements based on configurations provided by the LMF. This collected positioning data is used as the training dataset.

This measurement data is transferred from the UE and gNBs to the LMF or NWDAF. Here, the positioning AI model is trained or refined. The Data Storage function retains both the historical and current positioning data, creating a valuable repository for continuous model training and improvement.

Finally, the key output is Data Exposure, where the trained and refined positioning AI model is provided to the LMF. The Data Consumer (the LMF) then utilizes this intelligent model to significantly enhance the accuracy and efficiency of its positioning services for end-users and network applications. This creates a closed-loop, self-improving system for network-based positioning.

#### Exposure of dataset collected from UE

Some reginal regulations (e.g. GDPR (General Data Protection Regulation), Data Act, PIPL (Personal Information Protection Law), etc.) require lawful basis for processing, with explicit consent being a cornerstone, and enforce core principles of purpose limitation, data minimization, and transparency. Individuals are granted powerful rights, including access, correction, and deletion of their data. Crucially, the laws impose strict security obligations on data processors and heavily regulate cross-border data transfers, requiring specific legal mechanisms like adequacy decisions, standard contractual clauses, or security assessments to ensure protection continues after the data leaves its region of origin.  Individuals are given control over their data, placing the accountability and liability for safe data exposure squarely on the organization. These laws are not merely bureaucratic hurdles but are legally enforceable frameworks designed to safeguard personal privacy, prevent the misuse of sensitive information, and maintain individual control over personal data.

Observations: The exposure of datasets collected from UE must strictly follow related regulations to ensure the fundamental rights and freedoms of individuals are protected. Non-compliance can result in severe financial penalties, reputational damage, and legal action.

Proposals:

- Seriously consider the regulation alignments to evaluate whether to adopt the related use cases in the data framework

- Remove “exposure of dataset collected from UE” related use case from the example list in the WT and KI objectives

- Enhance “standardization”, “visibility” and “controllability” of the data

# 2. Proposals

**** First Change (based on S2-2509794) ****

# A.X 	Work Task X: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

- Identify high level use cases (e.g. AIML in the core, Sensing, Positioning) potentially subject to the data framework.

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

- Study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected standardized data (including data distribution) )

- data labelling/metadata handling

- data processing (e.g.data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 4:	 Coordination with SA5 may be required for charging aspects.

NOTE 5:  Consideration on MNO and UE’s requirements of full visibility and controllability during data handling processes is required.

NOTE 6:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

- Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE 7:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** End of Changes ****

# 3. References

General Data Protection Regulation (GDPR) https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=celex:32016R0679

Personal Information Protection Law (PIPL) https://flk.npc.gov.cn/detail?id=ff8081817b6472a3017b656cc2040044&fileId=&type=&title=%25E4%25B8%25AD%25E5%258D%258E%25E4%25BA%25BA%25E6%25B0%2591%25E5%2585%25B1%25E5%2592%258C%25E5%259B%25BD%25E4%25B8%25AA%25E4%25BA%25BA%25E4%25BF%25A1%25E6%2581%25AF%25E4%25BF%259D%25E6%258A%25A4%25E6%25B3%2595

Data Act https://www.eu-data-act.com/