3GPP SA WG2 Meeting #172	S2-2509936

17-21 November 2025, Dallas, Texas, USA	(was S2-250xxxx)

Source:	NEC

Title:	[WT#1.3] Scope and key issue on Non-3GPP Access

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This contribution proposes scope and key issue to Work task 1.3 of the 6G study on Non-3GPP Access.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #1.3 of the approved 6G SID (SP-250806):

WT#1.3: Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

The contribution builds on the baseline S2-2509804, which was postponed at the end of SA2#171.

1.1 Architectural Motivation

There are common interests that 3GPP core network can be accessed via Non-3GPP Access, e.g. WiFi, Wireline. The WiFi calling based on IMS has been commercially deployed in the real market. 6G system should consider whether and how non-3GPP Access can access to the core network of 6G system.

1.2 Gap Analysis

3GPP has specified multiple non-3GPP Access solutions in different generations, i.e. Interworking-WLAN(3G), Trusted/untrusted(ePDG) non-3GPP Access(4G), N3IWF based and Wireline access(5G). For the new generation 3GPP system i.e. 6G, it is suggested to study whether and how a new non-3GPP access solution can be specified, while the different already specified solutions may be compared and re-considered including to simplify the network architecture and UE implementation.

During the SA2#170 discussion, there is an opinion that only WiFi is considered as the non-3GPP access in the first release of 6G system, other non-3GPP accesses may be considered in the following releases(s).

The service continuity between 3GPP access and untrusted non-3GPP access may not be needed in all cases, it is suggested to study whether the service continuity between 3GPP access and untrusted non-3GPP access is needed.

1.3 Technical Impact

There are some opinions the current non-3GPP Access solutions are too complicated to be deployed, it is suggested to simplify the network architecture and UE implementation for non-3GPP Access.

Based on the gap analysis, WT#1.3 scope should address the following key issues:

- Whether only one or some specific non-3GPP access-es are considered in the first release of 6G system.

- Compared with previous generations, how to simplify the network architecture and UE implementation for non-3GPP Access connected to the core network of 6G system.

2. Proposal

This contribution proposes to update the annex of TR 23.801-01 to include the scope of WT#1.3 and the related Key Issue.

**** First Change – (based on S2-2509804) ****

# X. WT#1.3

It proposes to add several sub-WTs to address the non-3GPP access scenario and multi-access data connections.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G between 3GPP access and non-3GPP access.

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

NOTE 0:  At minimum, IMS based operator voice service over untrusted non-3GPP access (i.e. WiFi calling over IMS) needs to be supported.

2.	Study whether and how to support wireline access in 6G System architecture.

NOTE 1:	The work to support wireline access will not depend on BBF or CableLabs;

3.	Study whether and how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

4.	How to support URSP provided to UE for Non-Seamless WLAN Offload (NSWO).

NOTE 2:  The Authentication of NSWO is not within the scope for this bullet.

5.	How to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 3:  WT#1.2 aspects related to QoS, Policy, Interworking and UP applicable to non-3GPP access are studied under this WT and coordinated with corresponding WTs during the study.

NOTE 4: The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access present.

NOTE 5: Coordination with SA3 for authentication aspects will be needed for this WT.

For the multi-access:

6.	Study whether and how to support the multi-access data connectivity (e.g. traffic steering and switching) between 3GPP access and non-3GPP access for different types of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 6: This bullet will be studied only if the architecture for bullets 1 and 2 is stable, and the study will be conducted on the basis of that architecture, if time allows.

**** Second Change ****

# 5.X. Key Issue #X: Support of non-3GPP access and multi-access

For non-3GPP access, this KI will study:

1.	How to support untrusted non-3GPP access in 6G System architecture.

NOTE 0:  At minimum, IMS based operator voice service over untrusted non-3GPP access (i.e. WiFi calling over IMS) needs to be supported.

2.	Whether and how to support wireline access in 6G System architecture.

NOTE 1:	The work to support wireline access will not depend on BBF or CableLabs;

3.	Whether and how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

4.	How to support URSP provided to UE for Non-Seamless WLAN Offload (NSWO).

NOTE 2:  The Authentication of NSWO is not within the scope for this bullet.

5.	How to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 3:  WT#1.2 aspects related to QoS, Policy, Interworking and UP applicable to non-3GPP access are studied under this WT and coordinated with corresponding WTs during the study.

NOTE 4: The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access present.

NOTE 5: Coordination with SA3 for authentication aspects will be needed for this WT.

For multi-access access, this KI will study:

6.	Whether and how to support the multi-access data connectivity (e.g. traffic steering and switching) between 3GPP access and non-3GPP access for different types of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 6: This bullet will be studied only if the architecture for bullets 1 and 2 is stable, and the study will be conducted on the basis of that architecture, if time allows.

**** End of Changes ****