3GPP TSG-WG SA2 Meeting #172	S2-2509948

# Dallas, USA, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0761 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AI/ML positioning Case 3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The LMF functionality of providing input data to NG-RAN for NG-RAN node assisted positioning with gNB-sided model is missing from the general description. - Clarifications in the data collection for gNB-sided models for UEs under positioning procedure are needed, e.g. if user consent is not granted, the following steps 3-4 shall be skipped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add to LMF functional description that the LMF provides input data to the NG-RAN for NG-RAN node assisted positioning with gNB-sided model. And editorial modification to LMF functional description. - Clarification and editorial modification are made in the data collection for gNB-sided models for UEs under positioning procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and obscure specification on the functionality and procedure of the LMF supporting NG-RAN node assisted positioning with gNB-sided model. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8, 6.23.1, 6.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods, interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information and optionally interacts with NWDAF and/or OAM to retrieve ML model(s).

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information. If the indication of requiring reliable UE location information is received, the LMF selects positioning method as per the TS 33.256 [51].

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support cancelation of a 5GC-MT-LR or 5GC-MO-LR during UE mobility from 5GS to EPS with N26 interface.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Support determination of indoor or outdoor for a location estimate.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF over TLS.

NOTE 2:	How the LMF uses the received UE user plane positioning capability for SUPL [49] is left to the implementation.

-	Determine whether to use an ML Model for UE positioning.

-	Support requesting ML model provision from NWDAF containing MTLF for LMF-based AI/ML Positioning.

-	Support ML model provisioning by OAM for LMF-based AI/ML Positioning subject to operator's policy using the procedures and information models applicable to the scenario defined in clause 4a.2 of TS 28.105 [53].

-	Determine whether to use an ML Model for UE positioning. -	Support data collection for training of the ML Model and for monitoring of the performance of the ML Model.The LMF retrieves user consent for data collection for the purpose of ML model training from UDM as defined in TS 23.502 [19].

NOTE x:	The LMF retrieves user consent for data collection for the purpose of ML model training from UDM as defined in TS 23.502 [19].

-	Support training ML model locally using the training data collected from UE and/or gNB for UE positioning.

-	Monitoring the performance of the ML model used for UE positioning.

-	Support providing requested data to the NG-RAN as input data for ML model training and performance monitoring by the NG-RAN for NG-RAN node assisted positioning with gNB-sided model.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message and removes the PRU information after PRU Disassociation.

-	LMF keeps PRU information for PRUs which are in OFF state.

NOTE y:	PRU ON/OFF states indicate temporarily availability of the PRU functionality of a UE at the serving LMF (e.g. PRU OFF due to other high priority tasks/energy saving at the UE, or the UE temporarily loses network coverage).

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU indication via NF profile update if PRU is stationary PRU.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	If the LMF determines to use UE based positioning and to use PRU to improve the positioning result, it supports to provide the assistance data including location measurements from the PRU, the associated timestamp and the PRU location to the target UE which is responsible to calculate location based on the assistance data.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

-	Support to determine UE location by considering target UE location measurements and PRU location measurements in the time window(s). In the case that the serving LMF of target UE and the serving LMF of PRU are different LMFs:

-	If network assisted positioning is used by the serving LMF of target UE, it provides the positioning method, time window(s) and the identifier of TRPs for the measurements to the serving LMF of PRU. The serving LMF of PRU triggers network assisted positioning based on the received information and returns the PRU location measurements to the serving LMF of target UE.

-	If UE assisted positioning is used by the serving LMF of target UE, it provides the positioning method and time window(s) to the serving LMF of PRU. The serving LMF of PRU triggers UE assisted positioning based on the received information and returns the PRU location measurements to the serving LMF of target UE.

NOTE z:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR or MWAB based on location and velocity of the MBSR or MWAB and the timing of the location estimations for the target UE and MBSR or MWAB.

-	For a regulatory location service, support reporting of multiple INTERMEDIATE location estimates to GMLC.

* * * Next Change * * * *

6.23.1	General

This clause describes the procedure for data collection by the NG-RAN for input data to train or monitor the performance of the ML model for NG-RAN node assisted positioning with gNB-sided model.

6.23.2	Data Collection for NG RAN-sided models for UEs under positioning

Figure 6.23.2-1 shows a procedure of data collection for UEs under positioning by the NG-RAN, in order to train or monitor the performance of the gNB-sided ML model for NG-RAN node assisted positioning.

Figure 6.23.2-1: Data Collection for NG RAN-sided models for UEs under positioning

Precondition: There is an ongoing positioning session for the UE/PRU, and the network assisted or network based positioning is used.

1.	The procedure in Figure 6.11.2-1 can be used for data collection by the NG RAN for input data to support NG-RAN node assisted positioning with gNB-side model.

The steps 1-6 of network assisted positioning procedure are used as defined in clause 6.11.2, and the information to indicate that data collection is needed for the UE being positioned is included by the NG-RAN node in the Network Positioning message in steps 5-6 as defined in NRPPa signalling in TS 38.455 [15].

2.	Before the LMF provides the data for the UE being positioned to the NG-RAN, the LMF performs the user consent check by interaction with UDM as defined in step 4 of clause 6.22.3. If user consent is not granted, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15] and the following steps are skipped.

3-4.	The LMF determines the data requested by NG-RAN based on the ground truth data of the UE. The LMF sends the requested data via the AMF to the NG-RAN node in a Network Positioning message as defined in TS 38.455 [15]. If the LMF is unable to provide the positioning information, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15].

* * * End of Changes * * * *