3GPP TSG-SA WG2 Meeting #172	S2-2509950

Dallas, USA, November 17 – 21, 2025

Source:	CATT

Title:	KI#3 interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim agreements for KI#3.

1. Introduction

Based on the interim agreements for KI#3 “NF (re)selection and/or UP path adjustment based on energy related information” in clause 7.1.3, conclusions for KI#3 are proposed.

In addition, it is proposed to add “DNAI (re)selection for UP path adjustment taking into account of energy related information” in the conclusion, for the following reasons:

Compared to UPF (re)selection based on the NF profile, which only takes into account of the energy related information of one UPF, UP path adjustment considering DNAI (re)selection takes into account of the overall energy related information (e.g. energy consumption, energy efficiency, and/or renewable energy consumption ratio) of all the UPFs along the UP path for transmitting the service data flows towards the DNAI, so that the most suitable DNAI from energy point of view can be selected for the specific service(s). This can improve energy related performance for supporting services/applications, e.g. when multiple DNAIs can be selected in local networks for the same service(s)/application(s).

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

# * * * * First change (all new text) * * * *

8.3	Conclusions for KI#3

The conclusions for KI#3 are as follows:

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.1.3-1.

Table 7.1.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	UP path(s) selection and reselection may also be triggered via DNAI (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow.

# * * * * End of change * * * *