SA WG2 Meeting #172	S2-2509983

17 - 21 Nov, 2025, Dallas, US	revision of S2-2509836/9791

Source:	China Mobile, ZTE

Title:	[WT#3.1] AI for 6G architecture

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document summarizes the proposals for WT#3 regarding the use of AI for 6G architecture. It is based on the postponed version in SA2#171.The further proposed text is highlighted in yellow.

**** First change ****

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs include intent

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.2	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.3	when the requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations)

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.4	design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of intelligencebased on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not AI technologies can interwork with home network using or not AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring,

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

**** Second change (all new text) ****

5.x	Key Issue #xx: AI for 6G architecture

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs include intent

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.2	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.3	when the requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations)

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.4	design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of intelligencebased on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not AI technologies can interwork with home network using or not AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

**** End of changes ****