3GPP TSG-WG SA2 Meeting #172	S2-2509991

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#3 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#3. The conclusions are drafted based on the principles agreed in the interim agreements for KI#3 in Clause 7 of 3GPP TR 23.700-14 V1.1.0.

The following conclusions are arrived at based on the interim agreements with the following updates,

- The NF profile parameters for SeMF registration with NRF, are expanded.

- Additional parameters for SeMF selection have been provided

- The re-selection of SeMF and Sensing Entity(ies) are added, as it is part of the scope.

- Apart from the sensing service request, Sensing Entity(ies) may be selected by SeMF based on the information about the Sensing Entities, e.g. its supported sensing area.

- Removal of NOTE

- Depending on the architecture, the sensing capabilities of gNB can be provided to SeMF through a direct connection or via AMF. The exact sensing capability information will be based on RAN3 input which has been captured as NOTE

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.a	Conclusion for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed as conclusions for KI#3 and is recommended for normative work:

-	The NEF/AF performs SeMF selection based on local configuration or NRF query.

-	The SeMF registers itself to the NRF, with its NF profile that may contain NF Type, supported Sensing Service Area, supported Sensing Service Type(s) and supported Sensing QoS.

-	SeMFis selected based on at least one of, its supported sensing area, Target Sensing Area, QoS parameters and Sensing Service Type.

-	SeMF re-selection may be performed, when necessary, e.g. due to unavailability of Sensing Function to serve a specific target sensing area during a periodic sensing procedure

-	SeMF selects one or more Sensing Entity(ies), i.e. gNBs, based on the sensing service request and capabilities of Sensing Entity(ies), e.g. its supported sensing area.

NOTE: The sensing capabilities of gNB to be given to SeMF will be decided by RAN3 and is required in the normative phase.

-	The sensing capability information of Sensing Entity(ies) may be provided to SeMF either directly or through the AMF.

-	The SeMF may perform re-selection of Sensing Entity(ies), when necessary, e.g. dynamic load conditions of gNB(s).

# * * * * End of changes * * * *