3GPP TSG-WG SA2 Meeting #172	S2-2509992

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#4 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#4. The conclusions are drafted based on the principles agreed in the interim agreements for KI#4 in Clause 7 of 3GPP TR 23.700-14 V1.1.0.

The following conclusions are arrived at based on the interim agreements with the following updates,

- An event-triggered type of sensing should be supported by the gNB for certain use-cases, such as sensing of a region based on the movement of object in/out of an area. Hence, the event-triggered type of sensing has been added to the list of sensing types.

- The Editor’s Note is converted to a NOTE and updated to incorporate all the inputs required from RAN3

- Based on KI#1, there are two methods to transfer sensing data from Sensing Entity to SeMF i.e., a direct connection or an indirect connection via AMF. Considering the various use-case requirements, we feel both the methods should be supported in the system

- In the indirect connection, the AMF facilitates the transfer of sensing data from the Sensing Entity to SeMF and does not interpret the sensing data

- In point 4, instead of having “information” it would be clear if the text is “sensing information”. For specific use-case requirements, if the sensing information is obtained from multiple gNBs, then the SeMF should support aggregation of information to generate the sensing result

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.d	Conclusion for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed as conclusions for KI#4 and is recommended for normative work:

-	Based on the received sensing service requirements from the SeMF, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one-time, periodic, and event-triggered types, to transfer Sensing Data to the SPF(s) in SeMF.

-	Sensing data can be collected from one or multiple gNBs by SeMF for the same sensing service request, either directly or via AMF.

NOTE: The sensing service type (i.e., one-time, periodic, event-triggered) and the content of Sensing Data along with associated information, that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	The SeMF can generate sensing results based on the aggregated sensing information provided by the gNB(s).

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# * * * * End of changes * * * *