SA WG2 Meeting #172	S2-2510004

17 – 21 November, 2025, Dallas, US

Source:	Apple

Title:	KI#1 Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: KI#1 conclusion proposal based on agreed interim agreements.

# 1.	Discussion

This contribution proposes KI#1 conclusion proposal based on agreed interim agreements agreed at SA2#171.

Changes are highlighted in this section for clause 7 in track changes mode and proposed as all new text for clause 8 afterwards to avoid unnecessary discussion on two different clauses with the same contents.

### 7.1.1	Key Issue #1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

NOTE 4: The Sensing control functionality contains the sensing service request authorization and other sensing gateway functionality.

Principle 2: Direct interfaces between the SF and Sensing Entity (SE) must be supported for control, configuration, and sensing data transport.

NOTE 5: The definition of such an interface should account for the possible deployment split of SCF and separated SPF(s).

Principle 3:  No dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 4: In this study, the gNB is the only entity that acts as the SE.

NOTE  6:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.700-14.

**** First Change (ALL NEW TEXT) ****

## 8.X	Key Issue #1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

NOTE 4: The Sensing control functionality contains the sensing service request authorization and other sensing gateway functionality.

Principle 2: Direct interfaces between the SF and Sensing Entity (SE) must be supported for control, configuration, and sensing data transport.

NOTE 5: The definition of such an interface should account for the possible deployment split of SCF and separated SPF(s).

Principle 3: No dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 4: In this study, the gNB is the only entity that acts as the SE.

NOTE  6:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

**** End of Changes ****