SA WG2 Meeting #172	S2-2510007

17 – 21 November, 2025, Dallas, US

Source:	Apple

Title:	KI#5 Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: KI#5 conclusion proposal.

# 1.	Discussion

This contribution proposes KI#5 conclusion proposal.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.700-14.

**** First Change (ALL NEW TEXT) ****

## 8.X	Key Issue #5: Sensing Result Exposure

The following principles are concluded for KI#5:

Principle 1: The Sensing Function (SF) is responsible for exposing the Sensing Result to the Sensing Service Consumer (SSC).

-	For an external AF, the exposure of the Sensing Result is performed via the NEF.

-	For a trusted AF, the exposure may be performed directly from the SF.

Principle 2: The exposure of Sensing Results shall support one-time, periodic, and event-triggered reporting, based on the Sensing Service Consumer's request.

Principle 3: The exposed Sensing Result may contain information about the detected object(s) (e.g. location, velocity, type) and/or characteristics of the environment. The result may also be expanded with sensing contextual information (e.g. timestamp, confidence level, accuracy).

NOTE:	To account for the possible deployment split of Sensing control functionality (SCF) and Sensing processing functionality (SPF), the SPF may directly expose the Sensing Result(s) to the Sensing Service Consumer.

**** End of Changes ****