3GPP TSG SA WG2 Meeting #172	S2-2510033

17 - 21 November, 2025, Dallas, USA	                      (revision of S2-2508773)onr20xxxx)

Source:	China Mobile

Title:	EN removal of Solution#27

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: EN removal of Solution#27 on support of user plane management analytics.

1. Discussion

- It is up to the UPF’s implementation to detect traffic hijacking and traffic spoofing, which is out of 3GPP scope. The following NOTE can be removed.

Editor's note: It is FFS whether and how the UPF can detect traffic hijacking and traffic spoofing.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

6.27.2.2	Input Data

The NWDAF supporting UP management analytics shall be able to collect information from AF, OAM and 5GC NFs.

Table 6.27.2.2-1: Input data related to UPF performance

| Information | Source | Description |
| --- | --- | --- |
| UPF info | UPF/ SMF/NRF | Identifying the UPF(s) that is associated to the collected information, e.g. the NF instance ID(s) or NF set ID(s) of the UPF(s), or the IP address or FQDN of the UPF. |
| UPF load | NRF | Load information indicates the current load of the UPF, e.g. CPU, memory, or percentage of load information. |
| DL/ UL Throughput | UPF/ AF | The throughput UL/DL over the measurement period, .e.g. peak value or average value. |
| UPF service area | UPF/ NRF | The service area information of the corresponding UPF(s). |
| Capacity GTP | UPF/ OAM | The maximum achievable UL and/or DL GTP transmission rate between PSA UPF and NG-RAN and between PSA UPF and UE, e.g. as defined in clause 6.3.11 of TS 28.554 [16]. |
| Data transmission over N3 interface | UPF/ OAM | Data packets or traffic volume of incoming and/or outgoing GTP data packets on the N3 interface (from (R)AN to UPF), e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Data loss over N3 interface | UPF/ OAM | Incoming GTP Data Packet Loss or Rejection in UPF and/or outgoing GTP Data Packet Loss that are not successfully received at gNB over N3, e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Throughput of N3 interface | UPF/ OAM | Upstream and/or downstream throughput at N3 interface, e.g. as defined in clauses 6.3.3 and 6.3.4 of TS 28.554 [16]. |
| Data transmission over N6 interface | UPF/ OAM | The link usage of N6 interface, including the usage of incoming traffic and/or outgoing traffic, e.g. as defined in clause 5.4.2 of TS 28.552 [15]. |
| N4 interface information | SMF/ OAM | The N4 interface information may include the number of requested, successful or failed N4 session establishments and/or N4 session reports, e.g. as defined in clause 5.4.3 of TS 28.552 [15]. |
| Data transmission over N9 interface | UPF/ OAM | The Number of incoming and/or outgoing GTP data packets on the N9 interface associated to the PSA UPF, e.g. as defined in clause 5.4.4.2 of TS 28.552 [15]. This may include the overall, successful or failed data packets transfer. |
| Delay | SMF/UPF/ OAM | The UL/DL/ round trip delay between UPF and NG-RAN, or between UE and UPF, e.g. as defined in clauses 5.4.7, 5.4.8 and 5.4.9 of TS 28.552 [15] or collected from UPF/ SMF. The delay could be the values of average, distribution, maximum, or variance. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information. |

Table 6.27.2.2-2: Input data related to the service flow via user plane

| Information | Source | Description |
| --- | --- | --- |
| Application info | UPF/ AF | Identifying the application associated to traffic/ service flow, e.g. application ID, IP address or FDQN of the application. The data transmission might be from or to the application server. |
| UE IDs | SMF/ UPF/ AF | Identifying the UE(s) (e.g. SUPIs, GPSIs) that are associated to the corresponding data transmission from/to the application server(s). |
| Transport protocol information | SMF/UPF/AF/ | The transport protocol associated to the traffic transmission, e.g. RTP, SRTP, TCP, UDP, DCCP, SCTP, QUIC, etc. |
| IP Packet Filter Set or PDR | UPF/ AF | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 [2] or the PDR for traffic detection at UPF. |
| Data communication type | SMF/UPF/AF | Identify the data communication type, e.g. IP type, Ethernet type, or unstructured type data communication. |
| Traffic characteristics | UPF/ AF | The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), packet delay, loss rate, throughput, etc. within the measurement period. |
| Variation of traffic characteristics | UPF/ AF | The variation or standard deviation of the parameters belong to traffic characteristics within the measurement period, e.g. the variation of the traffic volume or data burst size, etc. within a given time period. |
| (Abnormal) data packet information | UPF | For example, malformed packets, duplicate packets, packet retransmission, traffic hijacking and traffic spoofing. The UPF may report the abnormal data packet base on operation's pre-configuration or information in consumer's request. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information |

NOTE:	UPF reports the specific data based on subscription from NWDAF by using, e.g. analytics filter, to reduce UPF reporting load.

# * * * * End of Changes* * * *