3GPP TSG-SA2 Meeting #172	S2-25010045

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Source: 	SK Telecom, AT&T, China Mobile, China Telecom, Deutsche Telekom, KDDI, Verizon, CATT, ETRI, ZTE, Samsung, Nokia

Title: 	KI#2 Interim Agreements/Conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes the update for KI#2 Interim Agreements/Conclusion of AIML_CN KI#2.

1	Discussion

In TR 23.700-04 V1.1.0 (2025-10), the recent UC#1 confirm that UPF can be a consumer of NWDAF analytics (e.g. Sol#23(S2-2509576), #24(S2-2509315), #30(S2-2509316), #31(S2-2508478)). Therefore, the agreed principles are kept and clarified: a) retain the consumers including UPF, b) add a concise role boundary that UPF, as a consumer, takes only local, non‑Session/QoS actions, with SMF/PCF remaining the owners of N4/PCC policy, and c) add low‑impact clarifications on Input/Output and load‑control.

Example options

For example, UPF as a consumer subscribes per‑UPF and/or per‑interface and received Analytics could performs local, non‑Session/QoS actions, whereas SMF/PCF consumes per‑UE, per‑QoS, per‑application analytics (N4/PCC). Since abnormal UP traffic can be in any form of data/packets (unknown, malformed, fraud, temporal behaviour, malicious, etc) that may not belong to any Sessions, it is more useful to allow the UPF to act as a NWDAF consumer enables fast, localized mitigation by subscribing per-UPF and/or per-interface analytics, letting the UPF handle any abnormal UP traffic behaviours through simple, non-Session/QoS actions such as operators’ defined shaping, mirroring, or auditing.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288 [5]) for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	When the UPF acts as a consumer, it performs operator‑configured local packet handling only (e.g. allow, deny, mirror, audit, shape, DLDR pacing, etc). Session/QoS state changes remain under SMF/PCF control. When the UPF acts as a NWDAF consumer, its analytics subscription scope is limited to per-UPF or per-interface level.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	Volume, rate or burst size of abnormal traffic.

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s).

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	Time window (e.g. start, duration) and UPF ID, interface when applicable.

-	UP Traffic Pattern information containing pattern type (e.g. small‑packet burst, malformed, unknown, duplicate, fragmented packets, etc) and overload type (e.g. CPU/resource saturated, interface queue full/N6 congestion) when such data are available from analytics.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

-	From UPF/SMF:

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.).

-	Traffic flow filters.

-	Identification of corresponding UEs.

-	Information of traffic characteristics.

-	Information of UP pattern data elements if available from UPF, e.g. pattern type (including malformed, unknown, duplicate, fragmented), overload type, Top‑N fraud/offenders, and application traffic info (IP packet filter set) as labels.

NOTE: Existing UPF event exposure can be used or extended to collect usage/throughput/delay inputs (e.g. user‑data usage measures/trends, QoS monitoring, etc); per‑data‑flow reports commonly include flow descriptors and AppId when available. Additional UPF statistics (e.g. packet‑anomaly indicators, buffer/queue occupancy, per‑flow discard reasons, UPF resource load, etc) may be obtained via existing means or considered in normative phase. Details about applicable events and parameters for the subscription and reporting are to be determined in the normative phase.

- 	The event subscription can Target any UE or specific UEs and provides information about targeted UP pattern data and thresholds for the occurrence/volume/burst The UPF only reports when corresponding traffic patterns are detected.

-	The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501 [2]).

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations.

-	UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or pps limitations).

-	adjust packet processing resources.

NOTE 1:	The UPF shall not subscribe for analytics requiring Session/QoS-flow, or application specific context, as such information resides with the SMF/PCF. Any analytics subscription or output at the UPF shall therefore be used only for local, non-Session/QoS behaviours (e.g. interface-level traffic statistics, anomaly trends, Top-N offenders, or overload indications, etc).

-	The PCF as consumer of NWDAF analytics may for instance take the following actions:

-	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2:	This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF. The PCF may take the analytics outputs into account to derive the QoS for the target application(s).

-	The NWDAF may provide the output analytics on traffic pattern information, including:

-	Maximum Burst Size/ Maximum Data Burst Volume, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity.

-	Corresponding information of the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s).

-	Identifiers of affected UEs, e.g. UE ID(s).

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow(s), e.g. packet filter, application ID of the traffic flow.

-	Information identifying the associated UEs.

-	Information related to the data flow characteristics, including, N6 delay, congestion information, indication that UPF drops packets and packet drop rate, data volume, packet delay, data rate, periodicity, and packet interval.

NOTE 1:	The indication of UPF dropping packets and packet drop rate is only applied to the abnormal scenarios and the scenarios when UPF dropped buffered data.

NOTE 2:	Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined will be determined in the normative phase.

-	The consumer actions of the PCF may take the output Analytics into account, e.g. to generate or update PCC rules by adjusting the QoS.

-	OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

-	An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

-	The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc.

7.2	Topics for further consideration

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether and how to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	The need and details of type/identification of traffic pattern is FFS.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-	For UC1, the input from AF, possibly via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

8	Conclusions

KI#2 UC#1:

The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	When the UPF acts as a consumer, it performs operator‑configured local packet handling only (e.g. allow, deny, mirror, audit, shape, DLDR pacing, etc). Session/QoS state changes remain under SMF/PCF control. When the UPF acts as a NWDAF consumer, its analytics subscription scope is limited to per-UPF or per-interface level.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	Volume, rate or burst size of abnormal traffic.

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s).

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	Time window (e.g. start, duration) and UPF ID, interface when applicable.

-	UP Traffic Pattern information containing pattern type (e.g. small‑packet burst, malformed, unknown, duplicate, fragmented packets, etc) and overload type (e.g. CPU/resource saturated, interface queue full/N6 congestion) when such data are available from analytics.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

-	From UPF/SMF:

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.).

-	Traffic flow filters.

-	Identification of corresponding UEs.

-	Information of traffic characteristics.

-	Information of UP pattern data elements if available from UPF, e.g. pattern type (including malformed, unknown, duplicate, fragmented), overload type, Top‑N fraud/offenders, and application traffic info (IP packet filter set) as labels.

NOTE: Existing UPF event exposure can be used or extended to collect usage/throughput/delay inputs (e.g. user‑data usage measures/trends, QoS monitoring, etc); per‑data‑flow reports commonly include flow descriptors and AppId when available. Additional UPF statistics (e.g. packet‑anomaly indicators, buffer/queue occupancy, per‑flow discard reasons, UPF resource load, etc) may be obtained via existing means or considered in normative phase. Details about applicable events and parameters for the subscription and reporting are to be determined in the normative phase.

- The event subscription can Target any UE or specific UEs and provides information about targeted UP pattern data and thresholds for the occurrence/volume/burst The UPF only reports when corresponding traffic patterns are detected.

-	The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501 [2]).

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations.

-	UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or pps limitations).

-	adjust packet processing resources.

NOTE 1:	The UPF shall not subscribe for analytics requiring Session/QoS-flow, or application specific context, as such information resides with the SMF/PCF. Any analytics subscription or output at the UPF shall therefore be used only for local, non-Session/QoS behaviours (e.g. interface-level traffic statistics, anomaly trends, Top-N offenders, or overload indications, etc).

-	The PCF as consumer of NWDAF analytics may for instance take the following actions:

-	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2:	This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

-	OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

-	An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

End of Changes