SA WG2 Meeting #S2-172	S2-2510073

17 – 21 November, 2025, Dallas, USA

Source: 	Interdigital

Title: 	Proposal for KI#6 conclusion

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for KI#6 conclusion.

# 1	Introduction

This contribution provides the update on interim agreements and conclusion on KI#6 on Configuration of Parameters for Sensing Entities.

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.1.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.6	Agreed Principles for KI#6

- The Sensing Function configure parameters for sensing operation at the selected Sensing Entity(s) while it sends a request for sensing operation.

- The configuration parameters include the target sensing area, the requested sensing service type, sensing report type(e.g., one time, periodic, event triggered), requested sensing QoS (e.g., accuracy, granularity, resolution, etc), and sensing operation ID.

NOTE1: The content of the configuration parameters for Sensing Entity(s) will be coordinated with RAN.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.6 Conclusion on KI#6

- The Sensing Function configure parameters for sensing operation at the selected Sensing Entity(s) while it sends a request for sensing operation.

- The configuration parameters include the target sensing area, the requested sensing service type, sensing report type(e.g., one time, periodic, event triggered), requested sensing QoS (e.g., accuracy, granularity, resolution, etc), and sensing operation ID.

NOTE1: The content of the configuration parameters for Sensing Entity(s) will be coordinated with RAN.

# * * * * End of change* * * *