3GPP TSG-SA2 Meeting #172	S2-2510075

# Dallas, USA, November 17 – 21, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0123 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AIOT Device Profile Data and AIoT Security Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 sent LS (S3-253683) to SA2 to clarify the storage of the device credentials: SA2 has specified in TS 23.369 that UDR may store AIoT data including AIoT device profile data and AF authorization data. However, SA3 has specified that device credentials can be stored only in the ADM. From SA3, the device credentials are stored in the ADM only while the AIoT Device Profile Data may be stored in the UDR as specified in TS 23.369. Also, the service operations in Nadm_Sec specified by SA3 uses the device credentials, while the service operations in Nadm_DM specified by SA2 uses the AIoT Device Profile Data. For the Nadm_DM_Query operation, the device credentials are not expected to be included, which are to be accessed by the service operations in Nadm_Sec. The device credentials should not be regarded as a part of AIoT Device Profile Data. And thus, the ADM needs to manage device security data, as specified in TS 33.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce device security data management in ADM. - Update the description to separate device credentials from the AIoT Device Profile Data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incorrect description of the device credentials. - Missing function description of ADM. - Misalignment with SA3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.9, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>BEGINNING OF CHANGES <<<<

### 4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

-	Management of AIoT device security data as defined in TS 33.369 [9].

-	Management of AF authorization data.

The AIoT device profile data and AF authorization data used by the ADM may be stored in the UDR.

>>>> NEXT CHANGE <<<<

## 5.5	AIoT Device Profile Management

The ADM may hold AIoT Device Profile Data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is stored in the ADM, otherwise the corresponding profile data is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in clause 5.2.3 of TS 23.502 [4], it is stored in the ADM  network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

NOTE:	In addition to the AIoT device profile data, the ADM also manages AIoT device security data (i.e., device credentials) as specified in TS 33.369 [9].

>>>>END OF CHANGES<<<<