3GPP TSG-SA WG2 Meeting #172	S2-2510083

Dallas, USA, November 17 - 21, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim Agreements for KI#2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes interim agreements for KI#2.

1. Introduction

In TR 23.700-30, there are 19 solutions for KI#2 included. This contribution discusses and proposes interim agreements from different aspects listed below.

Initial Registration

In general, those solutions can be classified as two categories:

- Active solution: the DO-A capable devices can register towards the network autonomously

- Passive solution: There is a trigger from AF to trigger the registration for the DO-A capable devices.

In our view, in the KI description, it is to study:

How the AIoT Device informs the network of its presence autonomously (e.g. an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

The DO-A capable AIoT devices are assumed to be able to inform the network of its presence autonomously. It does not require the trigger from the AF.

The motivation of the passive solutions are to reuse Rel-19 solution framework. It can reuse the existing mechanism developed in Rel-19 to enable the AIoT devices to obtain radio resources to perform random access. The active solutions take the assumption that there is a method for the readers to deliver information to the DO-A capable AIoT devices (e.g., broadcast SIB-like messages), so that the DO-A capable AIoT devices can perform random access in the similar way as UEs.

In our view, the Rel-19 solution framework is customized for the passive devices. Applying such solution framework brings some limitations towards the active devices: It depends on AFs whether a DO-A capable device can register to the network or not within an area. If a device is not within the AIoT Identification Informations from the AFs which have sent the request, it cannot register towards the network. If a device moves out of the expected coverage of the AF, it cannot register towards the network as well.

It also requires the NG-RAN node to be able to coordinate different requests from quite many AFs, which is unncessary complexity towards the NG-RAN.

[Proposal-1] It is proposed to agree on the principle that the DO-A capable devices support Initial Registration.

Mobility Registration

The mobility registration will enable the network to be updated with the location of the AIoT device. For the downlink message, with the location of the AIoT device, the message delivery can be performed effectively. There are no needs for the network to page the AIoT device within an area to find the device.

[Proposal-2.1] It is proposed to agree on the principle that the DO-A capable devices support Mobility Registration.

However, mobility registration requires the readers to be able to deliver area information to the AIoT devices, so that the AIoT device can determine whether it is within the area or not. If it moves out of the area, it performs Mobility Registration.

[Proposal-2.2] Check with RAN2 on whether the area information is delivered by the readers.

In some solutions, the area information is the AIoT Registration Area. It can be understood as similar concept as Registration Area for the UE. However, considering the AIoT Area introduced in Rel-19, some further clarification is required on the relationship between the AIoT Registration Area and the AIoT area.

[Proposal-2.3] Further clarification is needed on AIoT Registration Area,

The Rel-19 focuses on the indoor deployment, it can be understood the AIoT area may be significantly smaller compared with the cell and tracking area. For outdoor deployment, what are the area concepts to be used? Are they still AIoT Area and AIoT Registration Area? Will they be tracking area and Registration Area? And such decision requires collaboration with RAN3.

[Proposal-2.4] Further clarification is needed on the area and registration area for outdoor deployment.

Periodic Registration

In our view, periodic registration is important for DO-A capable devices as well. Periodic registration allows the network to be updated for the presence of the devices. It also allows the network and the AIoT devices to maintain up-to-date contexts. Without periodic registration, the network will lack the method of deregistering those unreachable devices to free up resources.

[Proposal-3.1] It is proposed to agree on the principle that the DO-A capable devices support Periodic Registration.

There are some concerns on the DO-A capable devices cannot maintain a sufficient long timer.

In our view, if the DO-A capable devices can maintian the timer, the most easiest way is to let the devices to trigger the periodic registration on their owns. Otherwise, the assistance from the network will be needed (e.g., the network broadcast timestamp information to let the device to compare with its local stored one, or the network perodically triggers the device to perform the registration update).

Anyway, SA2 does not have the expertise to decide whether the DO-A capable devices are able to maintain the timer to trigger the Periodic Registration.

[Proposal-3.2] SA2 can check with RAN1 on whether the DO-A capable devices are able to maintian the timer to trigger the Periodic Registration.

Temporary ID

Regarding the device ID to be used from the DO-A capable devices towards the network, quite many solutions further propose the temporary ID should be used. According to our understanding, the temporary ID is NOT the T-ID in Rel-19. The temporary ID is a CN allocated device ID like 5G-GUTI, which can be included in the uplink messages from the AIoT devices towards the network to identify the device. And in our understanding, there is AIOTF ID in the temporary ID, which can be used for the message routing from the NG-RAN towards the AMF and from the AMF to the AIOTF. Furthermore, the temporary ID is expected to be updated frequently like the 5G-GUTI for privacy protection reason.

In our view, the temporary ID is a similar mechanism like 5G-GUTI for the UEs. It is a mature solution to be used in communication systems.

[Proposal-4.1] It is proposed to agree on the principle that the temporary ID which is allocated by the AIOTF, is included in the uplink messages from DO-A capable devices. The temporary ID includes the AIOTF ID for message routing.

In Rel-19, SA2 delegates the development of T-ID to SA3. And considering the security protection is under the remit of SA3, it is recommended to ask SA3 to verify the usage of the temporary.

Also, the temporary ID is allocated by AIOTF, there needs to be a mechanism for the very first initial registration message to identify the device, which also needs to be routed to the proper AIOTF. This aspect hasn’t been covered by the existing solutions. We can understand that a SUCI-like identification could be possible. It is also recommended to check with SA3 about this device ID in the device ID to be used in very initial Initial Registration when the temporary ID hasn’t been allocated by AIOTF.

[Proposal-4.2] The usage of Temporary ID needs confirmation from SA3. And check with SA3 on the device ID when the temporary ID hasn’t been allocated by the AIOTF.

DO-A Data Delivery from the AIoT device to the AIOTF

For DO-A data delivery, most of the solutions proposed that the DO-A capable AIoT device sends a DO-A Delivery Request NAS message (or other similar message names) to the AIOTF containing the DO-A data and the device ID.

[Proposal-5.1] It is proposed to agree on the principe of using NAS message for DO-A data delivery.

Regarding the acknowledgement, there are several options: no acknowledgement, the acknowlegement from the AF, the acknowledgement from the AIOTF.

In the active RFID systems, there are also Acksnolwedged Mode and Unacknowledged Mode for the event or alert messages from the active tags.

Many industrial or high-reliability active RFID systems are designed to confirm the receipt of event data by the readers. That is, a response message (ACK) is almost always part of the protocol to ensure the reliability of the data delivery.

And for some simpler applications, the alert of the event message may be one-way. And those simpler active tags usually do not “register” towards the reader.

In our view, the registration process in communication systems inherently indicates a high reliability requirement. Therefore, it is recommended to introduce acknowledgement from the network to the DO-A capable devices for the event or alert messages. And considering the AIOTF is the NAS termination point, the AIOTF can be responsible for the acknowledgement of the event or alert messages from the DO-A capabile devices, like the readers in active RFID systems.

[Proposal-5.2] It is proposed to agree on the principe of using the AIOTF acknowledges for DO-A data delivery from the DO-A capable devices in NAS.

DO-A Data Delivery from the AIOTF to the AF

Regarding the DO-A data delivery from the AIOTF to the AF, the solutions can be classified as two options:

- Option-1: AIoT device includes AF information (e.g., AF ID) in the DO-A Delivery Request message. Based on AF information, the AIOTF can determine the target AF address, e.g., by local configuration.

- Option-2: The AIOTF obtains the target AF address from the AIoT Device Profile Data, which is written by the network by handling the subscribe request or configuration request from the AF.

The comparsion of those two options can be performed from the following aspects:

- Radio resource efficiency:

  - In Option-1, the AF information needs to be included in the DO-A Delivery Request message, which increases the overhead to some extent.

  - In Option-2, such overhead can be avoided.

  - Option-2 is more efficient compared with Option-1.

- Configuration efforts:

  - Option-1 requires the AF information to be configured in the AIoT device and the target AF address associated with the AF information to be configured in the AIOTF. If the target AF address needs to be updated, the reconfiguration in the AIOTF is needed. If the AF information needs to be updated, the reconfiguration (e.g., by Write Command) in the device is required.

  - In Option-2, the updates of the AF adress can be achieved by subscribe/unsubscribe request or the configuration/reconfiguration request form the AF. And there are no needs to update AF information in the AIoT device.

  - The configuration efforts of Option-1 is higher than the one of Option-2.

- NEF involvement:

  - For the AF outside trusted domain, the DO-A data need to be sent to the NEF firstly and the NEF should forward them to the AF. It is not clear on how the AIOTF can select the proper NEF which is reachable by the AF. It is also not clear on how the NEF is prepared to receive data on its notification endpoint, as there are no interactions between the NEF and the AIOTF prior to the DO-A data delivery. It is also not clear on how the AF is prepared to receive data on its notification endpoint, as there are no interactions between the AF and the NEF prior to the DO-A data delivery.

  - In Option-2, the NEF is determined by the handling of subscribe request or the configuration request from the AF. Such handling can ensure the proper NEF is determined. Before sending the request to the NEF, the AF needs to provide its notification endpoint (i.e., to be prepared to receive data on its notification endpoint). When handling the request, the NEF needs to take the similar action and provide its notification endpoint towards the AIOTF.

  - Considering the NEF invovlement, Option-2 is a less complex, clearer on how the solution works compared with Option-1.

[Proposal-6.1] It is proposed to endorse Option-2 as the way forward.

Within Option-2, there are also two methods proposed:

- Option-2.1: the AF subscribes to the DO-A data delivery towards the AIOTF. The AIOTF performs AF authorization and update the target address (the notification endpoint of the NEF or the AF depends on whether the AF sends the request to the AIOTF via the NEF or directly) in the AIoT device profile data.

- Option-2.2: the AF sends configuration request towards the NEF. The NEF updates the target address (the notification endpoint of the NEF). The ADM or the NEF may perform AF authorization?

Comparing those two sub options:

- SBI framework:

  - Option-2.1 uses the “subscribe-notify” mechanism within the NF service framework, which aligns SBI principle very well. The “subscribe-notify” applies to the interactions between the AF and the NEF, as well as between the NEF and the AIOTF.

  - In Option-2.2, for the interactions between the AF and the NEF, there is no explicit subscription which triggers the notification, the “configuration request” can be interpreted as an implicit subscription. For the interactions between the NEF and the AIOTF, there is neither explicit subscription nor implicit subscription from the NEF to the AIOTF, unless the ADM update can be interpreted as an implicit subscription.

  - From SBI principle perspective, Option-2.1 is a cleaner option.

- AF authorization:

  - Option-2.1 follows the AF authorization solution in Rel-19. That is, not to overload the NEF, the AIOTF is responsible for AF authorization for AIoT specific services, and the “Allowed service operations” is already defined in the AF authorization data.

  - In Option-2.2, it is unclear on which NF performs the AF authorization. If it is the NEF, it brings extra complexity to the NEF which we want to avoid in Rel-19. If it is the ADM, for the “update” operation of the AIoT Device Profile Data, the ADM needs to check the AF authorization data, which is a cross data set operation. From implementation perspective, it is also less good.

  - From AF authorization perspective, Option-2.1 aligns with Rel-19 solution better.

[Proposal-6.2] It is proposed to adopt Option-2.1.

Update after CC on November 3rd:

- In CC, there were some comments on whether the NAS ACK (i.e., a NAS response message for DO-A Data Delivery is needed or not. There was similar discussion in NB-IoT/CIoT regarding the AS ACK, NAS ACK and Application ACK. And if the AS ACK is utilized, the link is considered sufficiently reliable and there is no need to have NAS ACK which will lower efficiency.

We can see the benefits of the NAS ACK:

  - A reliable transmission is provided in NAS layer

  - Positive response (ACK) and negative response with cause code can be aligned. Without the response message, if the failure cause code needs to be informed to the AIoT device, the mechanism may be complex.

And we also agree that those comments do make sense. If the acknowledgements are applied in every layer, the transmission efficiency will be impacted.

We noticed that in Rel-19, the NAS messasge is utilized as an acknowledgement of the previous AS message. In TS 38.391, even though not very explicitly stated, the MAC layer uses NAS PDU as an implicit acknowledgement in AS layer.

In R2D Upper Layer Data Transfer message:

  - Recevied Data Size can be included, to instructs the AIoT device to perform a retransmission of the whole D2R message or some segments.

  - Data SDU can be included which is the NAS PDU, which implicitly acknowledges the D2R message.

We tend to think, if the similar mechanism applies, the NAS ACK will be needed to simplify the AS design which aligns with the MAC layer in Rel-19 for passive AIoT devices.

[Proposal-7] It is proposed to update [Proposal-5.2] under the dependency on RAN2.

- In CC, there were some comments on the temporary ID. It may be too early to draw the conclusion on the 5G-GUTI like temporary ID.

We also tend to agree on this. In our view, it is important that the AIOTF ID (and perhaps other information needed for message routing) should be included in the temporary ID. It can be up to SA3 to determine whether the temporary ID is 5G-GUTI like or not. We are open if a new temporary ID which is quite different from 5G-GUTI is decided by SA3.

[Proposal-8] It is proposed to rephrase the EN in [Proposal-4.2] about the coordination with SA3.

- In CC, there were some discussions on the AIOTF relocation or the request redirection, if a request is sent to a new AIOTF which does not hold the device context.

In our view, just like AMF relocation, the AIOTF relocation (i.e., the new AIOTF fetches the device context from the old AIOTF and updates ADM to make itself to be the serving AIOTF) is the most straightforward solution, in the non roaming scenario. It is more efficient compared with the request redirection option. The AMF relocation for UE is mature, and the AIOTF relocation can learn from that.

[Proposal-9] It is proposed to endorse AIOTF relocation for cases a request is sent to the AIOTF that does not hold the device context.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change (All texts are new) * * * *

### 7.1.2	Agreed Principles for KI#2

Following principles have been agreed for KI#2:

- DO-A capable AIoT Devices support Initial Registration to inform the network of its presence autonomously.

- DO-A capable AIoT Devices support Mobility Registration.

Editor’s note:		It depends on RAN2 whether the reader can broadcast area information to DO-A capable AIoT Devices.

Editor’s note:		The AIoT Registration Area is FFS.

Editor’s note:		The area concept and registration area for outdoor deployment is FFS. And the decision requires colaboration with RAN3

- DO-A capable AIoT Devices support Periodic Registration.

Editor’s note:		It depends on RAN1 to decide whether DO-A capable AIoT Device is able to maintain a timer for Periodic Registration.

- Temporary ID is allocated by the AIOTF and provided to the DO-A capable AIoT device in Registration Response message. The Temporary ID contains the AIOTF ID.

- The DO-A capable AIoT Device includes the Temporary ID in the NAS request message. The AIOTF ID included in the Temporary ID is used for message routing.

Editor’s note:		The Temporary ID requires coordination with SA3.

Editor’s note:		It is up to SA3 to determine the device ID in the Initial Registration when the Temporary ID hasn’t been allocated.

- If a NAS message is routed to a new AIOTF which does not hold the device context (i.e., not the serving AIOTF), the AIOTF relocation is performed.

- DO-A capable AIoT Devices send DO-A data to the AIOTF via NAS request message. The NAS request message contains DO-A data and Temporary ID.

- Editor’s note:		It depends on RAN2 to determine whether a NAS response message is needed as an acknowledgement in MAC layer.

- The AIOTF obtains the target address(es) from the AIoT Device Profile Data and notifies the DO-A data and the AIoT Device Permanent ID to NEF or the AF(s).

- The AF subscribes to the AIOTF for the DO-A data delivery of AIoT Devices directly or via the NEF.

- The AIOTF performs AF authorization for the subscribe request from the AF.

* * * End of Change * * * *