SA WG2 Meeting #172	S2-2510084

November 17 – 21, 2025, Dallas, USA

# Title:	[Draft] LS on FS_Sensing_ARC-related aspects requiring coordination

# Response to:	-

# Release:	Rel-20

# Work Item:	FS_Sensing_ARC

Source:	Ericsson [to be SA2]

To:	RAN3

CC:

Contact person:	Aleksejs Udalcovs

aleksejs.udalcovs@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

SA2 has started consolidating agreed principles for key issues in FS_Sensing_ARC study (see clause 7 in TR 23.700-14). To further progress the work, SA2 would like to request feedback on the following aspects identified so far.

- KI#3 on NG-RAN discovery and selection: SA2 considers that the Sensing Function selects an NG-RAN node for a requested sensing service based on the information about the area where NG-RAN node can perform sensing measurements. SA2 assumes this information is sufficient to make the selection.

SA2 asks:

1.1	Could RAN3 confirm this assumption?

- KI#4 on NG-RAN sensing information processing and reporting to CN for CN sensing result generation: SA2 considers that NG-RAN nodes could support, for instance, one-time and periodic sensing result reporting to the Sensing Function so that the Sensing Function could perform further processing for the determination of a final sensing result and exposure to the sensing service consumer.

SA2 asks:

2.1	Whether RAN3 considers the one-time and periodic sensing result reporting as part of their study?

2.2	What is the content of sensing reports that NG-RAN nodes report to the Sensing Function?

2.3	What level of processing is performed at NG-RAN nodes and whether this level of processing allows, for example, to detect an object or obtain its characteristics (e.g. Position and 3D velocity) when the object is present in an area where sensing measurements are performed.

- KI#5 on sensing result exposure: In order to specify a geographic area for which a sensing service consumer request a sensing service, SA2 considers definition of the Target Sensing Area concept as described in S2-2510090. This Target Sensing Area will be used to select NG-RAN node and sent in the sensing service request message. SA2 assumption is that the provided Target Sensing Area will be used by the NG-RAN to determine an area in which to perform sensing measurements. SA2 assumes that tracking area concept and cell IDs cannot be used to describe Target Sensing Area information.

SA2 asks:

3.1	Could RAN3 provide the feedback on the SA2 assumption?

3.2	In which format Target Sensing Area information needs to be provided to NG-RAN nodes?

- KI#6 on Configuration of parameters for NG-RAN nodes: SA2 assumes that sensing entities (i.e., NG-RAN nodes) can determine how to perform the relevant sensing measurements to collect sensing data based on the sensing service requirements that the Sensing Function includes inside sensing service request message. The following examples of the sensing service requirements have been considered so far: Target Sensing Area, sensing type (e.g. ‘Object Detection’, ‘Object Tracking’), reporting mode (e.g. one-time or periodic reporting). A final list of sensing service requirements that can be provided to Sensing Entities is subject of alignment with RAN WGs.

SA2 asks:

4.1	Could RAN3 confirm the above SA2 assumption on need for signalling sensing requirements during the sensing request to assist NG-RAN in performing the measurement.

4.2	Is there any other parameter (except for the sensing service requirements) that needs to be provided from the Sensing Function to the NG-RAN node?

[Other questions, if any, can be further added during the meeting]

# 2	Actions

To TSG RAN3

ACTION: 	SA2 kindly asks RAN3 to provide feedback on the above questions.

# 3	Dates of next SA WG2 meetings

SA2#173	February 9 – 13, 2026	Goa, India

SA2#174	April 13 – 17, 2026	MT, Malta