3GPP TSG-WG2 Meeting #172	S2-2510085

November 17 – 21, 2025, Dallas, USA		(revision of (unhandled) S2-2508858)

Source:	Ericsson

Title:	Interim agreement for KI#5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides the agreed principles and conclusions for KI#5.

# 1. Discussion

This document proposes interim agreement for KI#5.

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.5	Agreed Principles for KI#5 on Sensing Result Exposure

In order for a 3GPP network to expose a sensing result to the sensing service consumer based on the authorized sensing service request, the following main principle apply:

NOTE X1:	It is assumed that a sensing service requester and a sensing service consumer is the same entity.

-	A sensing result is exposed by the network to the authorized sensing service consumer in relationship with the invoked sensing service request.

-	Sensing service consumer can include the following parameters in the request: AF Identifier, information about the Target Sensing Area, sensing type to perform (e.g. ‘Object Detection’, ‘Object Tracking’), and Sensing Result Detail Level (e.g. ‘Notify result only’, ‘Notify result with associated information (time of capture, area map, etc.)’).

-	Additionally, the sensing service consumer may request, for instance, a specific reporting mode (e.g. one-time or periodic reporting of the sensing result), sensing characteristics of service (Sensing CoS such as accuracy/confidence, temporal resolution, spatial resolution), and time of day when the sensing result can be exposed.

NOTE X2:	A full set of sensing service requirements that consumers can include in a request will be determined in the normative phase.

-	The Sensing Function includes the relevant sensing service requirements in the sensing request message sent to the NG-RAN for performing sensing measurements.

NOTE X3:	Parameters that the Sensing Function includes in the sensing service message to the selected NG-RAN nodes are given in KI#6 agreed principles.

-	After obtaining the sensing measurements, the NG-RAN apply a certain level of processing prior providing the resulting sensing data to the Sensing Function.

NOTE X4:	‘Sensing Data’ is referred to a sensing report that contains sensing measurement results after NG-RAN nodes applies a certain level of processing (i.e., a sensing result from RAN perspective). The level of processing that NG-RAN node applies to the obtained sensing measurements prior providing them to the Sensing Function is RAN WGs decision. Further terminology alignment with RAN WGs can be required.

-	The Sensing Function may further use the NG-RAN provided sensing report to determine a sensing result.

NOTE X5:	‘Sensing result’ is referred to a set of received ‘Sensing Data’ from NG-RAN node after a certain level of processing by the Sensing Function.

-	After the sensing result has been determined, the Sensing Function provides the determined sensing result to the sensing service consumer via the NEF using the Nnef_SensingEvent_Notify and Nsf_SensingEvent_Notify service operations.

NOTE X6:	The exact names of service operations can be further revised.

>>>> Next Change <<<<

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.5	Conclusions for KI#5 on Sensing Result Exposure

TBD

>>>> End of Changes <<<<