3GPP TSG-SA2 Meeting #172	S2-2510209

# Dallas, USA, November 17 – 21, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6471 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AUSF Selection with Default Routing Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AUSF Selection section specifies that … When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer can select any AUSF instance within the home network for the UE. This covers mainly the scenario where SUPI privacy feature is activated in a deployment (i.e. SUCIs are generated with protection scheme other than null-scheme for all or some of the SUPIs) where a single group of AUSF instances manages authentication requests for ALL SUPIs in the HPLMN. This assumes that in a deployment where different groups of AUSF instances are deployed to handle the authentication requests of different sets of SUPIs within the HPLMN, the AUSF instances in the different AUSF groups and the USIM cards of ALL SUPIs are configured with appropriate Routing Indicator values (i.e. different from the default value) to be able to route authentication requests towards the appropriate AUSF group managing each SUPI. It may be however not possible to comply with this assumption in all cases. For example, in network transformation scenarios where a deployment that makes use of SUPI privacy feature evolves from a single group of AUSFs to multiple groups of AUSFs, the operator may not manage to configure appropriate routing indicator values for all the USIM cards previous to the introduction of the additional groups of AUSFs. In this case, authentication requests for UEs that make use of USIMs configured to apply SUPI privacy but not configured with an appropriate Routing Indicator value, may not be routed to the appropriate group of AUSF instances managing the corresponding SUPI for the SUCI generated by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the AUSF selection for SUCIs using the default value of the Routing Indicator is completed so that … - If the SUCI uses null-scheme then the AMF can derive the SUPI from the SUCI and then select the AUSF based on the SUPI. This is already covered in TS 23.003 (clause 2.2B, NOTE 2). - If the SUCI uses a protection scheme other than null-scheme, then the AMF selects an AUSF instance as follows … - If AMF discovers a group of AUSF instances that explicitly indicates support for the default value of the routing indicator within the NF profile registered in NRF, then the AMF selects one of these instances. This is, the operator can leave the subscriptions for which it has not been possible to provide an appropriate routing indicator in a given AUSF group and characterize these AUSF instances to support explicitly the default value of the routing indicator. The AMF, then uses the default value of the routing indicator as any other value during AUSF selection. - Otherwise, the AMF selects any AUSF instance within the home network as defined in the baseline. The proposed updates do not require any stage 3 change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AUSF selection in AMF can fail, in scenarios where different groups of AUSFs manage authentication requests for different groups of SUPIs in the HPLMN, for UEs that make use of USIMs configured to apply SUPI privacy but not configured with an appropriate Routing Indicator value. Not possible to deploy in practice different groups of AUSFs to manage authentication requests for different groups of SUPIs, when SUPI privacy is active but corresponding USIM cards are not assigned an appropriate Routing Indicator value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Principles of the CR applies to previous releases as well. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 6.3.4	AUSF discovery and selection

In the case of NF consumer based discovery and selection, the following applies:

-	The AMF and the NSWOF perform AUSF selection to allocate an AUSF Instance that performs authentication between the UE and 5G CN in the HPLMN. The AMF and the NSWOF shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on AMF and on NSWOF. The AUSF selection function in the AMF and in the NSWOF selects an AUSF instance based on the available AUSF instances (obtained from the NRF or locally configured in the AMF).

-	The UDM shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on UDM. The UDM selects an AUSF instance based on the available AUSF instance(s) obtained from the NRF or based on locally configured information and information stored (by the UDM) from a previously successful authentication.

AUSF selection is applicable to both 3GPP access and non-3GPP access.

The AUSF selection function in AUSF NF consumers or in SCP should consider one of the following factors when available:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI (by an NF consumer in the Serving network) along with the selected NID (provided by the NG-RAN) in the case of SNPN, Routing Indicator and optionally Home Network Public Key identifier (e.g. in the case that Routing Indicator is not enough to provide SUPI range granularity).

NOTE 1:	The UE provides the SUCI, which contains the Routing Indicator and Home Network Public Key identifier as defined in TS 23.003 [19], to the AMF during initial registration and to the NSWOF during NSWO authentication. The AMF can provide the UE's Routing Indicator and optionally Home Network Public Key identifier to other AMFs as described in TS 23.502 [3].

NOTE 2:	The usage of Home Network Public Key identifier for AUSF discovery is limited to the scenario where the AUSF NF consumers belong to the same PLMN as AUSF.

NOTE 3:	In the case of SNPN and if the UE provides an IMSI type SUCI to the AMF and the SUCI provided by UE or the SUPI derived from the SUCI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from IMSI) or the Routing Indicator provided by the UE within the SUCI for selection of AUSF. In the case of SNPN and the UE provides an NSI type SUCI to the AMF, the AMF uses the Home Network Identifier and Routing Indicator of SUCI/SUPI for selection of AUSF.

When using ‘non null-scheme’ as privacy protection scheme to generate the SUCI and the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer selects an AUSF instance that explicitly indicates support for the default value of the Routing Indicator in its NF profile registered in NRF, if any; otherwise any AUSF instance within the home network for the UE.

When using ‘null-scheme’as privacy protection scheme to generate the SUCI and the UE's Routing Indicator is set to its default value, the AUSF NF consumer can derive the SUPI from the SUCI and use the SUPI for AUSF selection as defined in TS 23.003 [19].

2.	AUSF Group ID the UE's SUPI belongs to.

NOTE 4:	The AMF can infer the AUSF Group ID the UE's SUPI belongs to, based on the results of AUSF discovery procedures with NRF. The AMF provides the AUSF Group ID the SUPI belongs to other AMFs as described in TS 23.502 [3].

3.	SUPI; e.g. the AMF selects an AUSF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for AUSF discovery.

NOTE 5:	In the case of Onboarding via ON-SNPN, AUSF instances supporting UE onboarding can be registered in NRF or locally configured in the AMF. The AMF in ON-SNPN can discover and select AUSF instance(s) supporting UE onboarding based on the MCC and MNC or realm part in Home Network Identifier of the SUCI/SUPI provided by the onboarding UE.

In the case of delegated discovery and selection in SCP, the AUSF NF consumer shall send all available factors to the SCP.

# *** End of Changes ***