SA WG2 Meeting #172	S2-2510221

17 – 21 November 2025, Dallas, USA

# Title:	New WID on Energy Efficiency and Energy Saving Phase2

# Source:	Lenovo, China Telecom

# Document for:	Approval

# Agenda Item:	30.2

Abstract:	It is proposed to introduce a new normative phase work for EnergySys_Ph2 based on the respective conclusions.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Energy Efficiency and Energy Saving Phase2

Acronym:	EnergySys_Ph2

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X |  |  |  |
| Don’t know |  |  | X |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| EnergyServ Phase2 | SA1 | 1030044 | Energy Efficiency as service criteria Phase2 |
| FS_EnergySys_Ph2 | SA2 | 1070058 | Study on Energy Efficiency and Energy Saving Phase2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940037 | Enhancements of EE for 5G Phase 2 | The Rel-18 WI in SA5 may be potentially related to the proposed SA2 study. |
| 870022 | Enhancements on EE for 5G networks | The Rel-17 WI in SA5 may be potentially related to the proposed SA2 study. |
| 810023 | Energy efficiency of 5G | The Rel-16 WI in SA5 may be potentially related to the proposed SA2 study. |
| 981137 | Network energy savings for NR | The Rel-18 work in RAN1 and RAN2 may be potentially related to the proposed SA2 study, depending on the agreed Rel-19 scope. |
| 1050112 | Energy Efficiency and Energy Saving | The Rel-18 WI in SA2 may be potentially related to the current proposed SA2 study. |

# 3	Justification

This work item is to address the new requirements from the SA1 EnergyServ Phase 2 and continue the enhancements from the previous release in the system perspective for further improvement on the network energy efficiency and energy saving.

Release 19 EnergySys has defined a basic set of functional enhancements, including the EIF (Energy Information Function) for supporting energy efficiency and energy saving in the network. Some features that were left over from the previous release, together with new identified ones, will need to be addressed in Release 20.

It is proposed to complete the design of stage 2 solutions and document the normative specifications, based on the conclusions in TR 23.700-67 of the FS_EnergySys_Ph2 study.

# 4	Objective

This work aims to specify required enhancements on 5GS to improve energy efficiency and to support energy saving in the network, for the following aspects, based on the conclusions in the TR 23.700-67:

WT#1: The objective of this WT is to enhance the collection, determination and exposure of energy related information in the 5GS;

1）The following additional energy related information is exposed by EIF:

A) The exposure of additional information in association to the exposure of energy consumption information at a desired granularity, specifically,

a)	This additional information, if requested by the consumers of the exposure services of the EIF, is provided together with the Energy Consumption information at the desired granularity and is the result of postprocessing of the information the EIF already has collected (i.e. no new information is required beyond what the EIF already has collected from OAM and SMF).

b)	Energy consumption per bit: this is defined as the calculated energy consumption of the desired granularity divided by the measured data volume of the granularity used to calculate the energy consumption.

c)	"Energy consumption category" that is a scalar with multiple values that are determined by the EIF using PLMN-operator defined rules (e.g. 5 categories of percentile) and the rules and values associated to each category are not in scope of 3GPP. This category information is applicable to the same scope (e.g. regional, PLMN-wide) as the scope of the information that the EIF is collecting for the same granularity for other UEs that are e.g. associated to ongoing exposure subscriptions. The scope of the information an EIF collects is implementation and deployment specific.

B) 	Per UE application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session with the following restriction:

a)	The application ranking related information exposed to the AF is only about the applications that the AF has permission/authorization with the applications and for which has requested the ranking. This permission/authorization is performed by the NEF. The EIF performs the ranking only of the ongoing applications subject to exposure.

NOTE 1:	The application ranking related information is exposed only to AF(s) related to those applications.

NOTE 2:	It is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF depending on SA5 feedback.

WT#2: The objective of this WT is to support service adjustments for the UE to enable energy saving;

1)	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 3:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 4:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

2)	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

A)	SM policy control.

NOTE 5:	Based on operator policy, it should have the balance between energy saving and user experience.

B)	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

C)	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

3)	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

4)	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

NOTE 6: Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 7:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

WT#3: The objective of this WT is to support NF (re)selection and/or UP path adjustment based on energy related information.

1)	For NF selection, the NF profile is enhanced to include the new parameters, e.g., Energy Priority Information.

2)	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

3)	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

4)	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA5 Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- | --- |
| WT #1 | 2.5 | 2.5 | Maybe | Yes | - |
| WT #2 | 2.5 | 2.5 | Maybe | Yes | WT#1 |
| WT #3 | 1 | 1 | No | Maybe | WT#1 |

Total TU estimates for the study phase: 6

Total TU estimates for the normative phase: 6

Total TU estimates: 12

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | System architecture for support of Energy Efficiency and Energy Saving | TSG SA #113 (Sept., 2026) |  |
| 23.502 | Procedures for support of Energy Efficiency and Energy Saving | TSG SA #113 (Sept., 2026) |  |
| 23.503 | Policy control for support of Energy Efficiency and Energy Saving | TSG SA #113 (Sept., 2026) |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Konstantinos Samdanis (Lenovo), ksamdanis@lenovo.com

Secondary Rapporteur: Zhuoyi Chen, China Telecom, chenzy34@chinatelecom.cn

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN issues are covered by RAN WGs (e. g. RAN2, RAN3)

Collaboration with SA5 for collection and possible charging impact.

# 9	Supporting Individual Members

Supporting IM name

Lenovo

China Telecom

AT&T?

BT?

CEWiT?

CATT?

China Mobile?

Deutsche Telekom?

ETRI?

Ericsson?

Futurewei?

Hisilicon?

Huawei?

Intel?

KDDI?

KPN?

LG Uplus?

MATRIXX Software?

MediaTek?

NEC?

Nokia?

NTT DOCOMO?

Rakuten Mobile?

Samsung?

SK Telecom?

Telefonica?

vivo?

ZTE?