SA WG2 Meeting #172	S2-2510239

17 – 21 November, Dallas, TX, USA	(revision of S2-250xxxx)

Source:	Ericsson

Title:	[WT#1.3] WT and KI for non-3GPP access

Document for:	Approval

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper discusses the detailed scope for WT#1.3 non-3GPP access in SA2 6G SID. Based on the discussion, a WT update and a new KI are proposed.

# 1. Justification

Using non-3GPP access has become an important complement to 3GPP access for many operators. We propose a WT for non-3GPP in 6G based on the following justifications and considerations:

- 	The main use case deployed so far for non-3GPP access (in 4G/5G) is to support untrusted access primarily for voice over WiFi. Therefore, the WT should include similar support for 6G, i.e. support for untrusted non-3GPP access.

- 	It is not clear to us that wireline access should be specifically included in the first 6G release. For example, there is at present no coordination with BBF or Cablelabs. It may also be more suitable to stabilize the 6G System first before discussing with BBF/Cablelabs. In addition, it could be studied if the solution for untrusted non-3GPP access can be re-used also for wireline access, to avoid multiple solutions as we have seen in 5GS. This may be more suitable to do when the solution for 6GS and untrusted non-3GPP access are available.

- 	It also seems suitable to not include multi-access in the first 6G release. In our understanding multi-access will build on the general non-3GPP support studied as part of this WT, and therefore it may be more suitable to start the multi-access work once the basic 6G architecture and non-3GPP solution are available.

# Proposal

It is proposed to approve the following WT scope and key issue in TR23.801-01.

**** First Change ****

### Annex A.1.3. WT#1.3 – non-3GPP access

This WT includes separate sub-WTs to address the non-3GPP access scenario and NSWO.

WT#1.3. Study whether and how to support and/or enhance non-3GPP access in 6G as well as session continuity between 3GPP access and non-3GPP access.

NOTE 1: 	Coordination with SA3 for authentication aspects will be needed for this WT.

WT#1.3.1. For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

NOTE 2:	At minimum, operator voice service over untrusted non-3GPP access needs to be supported. Other services are not excluded.

2.	How to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	How to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 3: 	Coordination with WT#1.4 and WT#2 is needed.

NOTE 4:	WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT and coordinated with corresponding WTs during the study.

NOTE 5: The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access present.

WT#1.3.2. For Non/Seamless WLAN Offload

1.	How to support Non-Seamless WLAN Offload (NSWO).

NOTE 6: 	This bullet is about URSP provided to UE for Non-Seamless WLAN Offload. The Authentication of NSWO is not within the scope for this bullet.

**** Next Change ****

## 5.X. Key Issue #X: Support of non-3GPP access

<<<< Copy WT text as KI text >>>>

**** End of Changes ****