SA WG2 Meeting #172	S2-2510240

17 – 21 November 2025, Dallas, USA

Source:	Lenovo

Title:	Update of the agreed principles for KI#3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes updates for the agreed principles for KI#3.

# Discussion

As a result of SA2#171 meeting, the agreed principles for KI#3 were captured in TR 23.700-67v1.1.0 enhances the NF profile in the NRF introducing the Energy Priority Information. The energy priority aims to establish a value that reflects various aspects of energy performance, considering renewable energy use, energy consumption, and energy efficiency, among others. The primary evaluation criteria and the method for determining this value are operator specific.

However, energy priority cannot indicate the NF’s current energy saving operational state, but it can support energy specific service selection.

In other words, the Energy Priority Information in the NRF does not reflect the NF’s actual energy saving state.

Proposal: It is proposed to add the EnergySaving/not EnergySaving state and the corresponding schedule in the NF profile in table 7.1.3-1.

# Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

## 7.1.3	Agreed principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.1.3-1.

Table 7.1.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

- For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

- The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

- A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

- Update the heartbeat timer of the NF to avoid triggering restoration procedures once an NF enter an energy saving state. The value of the heartbeat needs to match the time instance in where an NF is expected to be repowered on, i.e., return from an EnergySaving state to a not EnergySaving state.

* * * * End of Changes * * * *