SA WG2 Meeting #172	S2-2510242

17 - 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source:	ZTE

Title:	KI#2, New Sol: DO-A Data Transfer Procedure for Topology 2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The paper adds new solution for key issue of supporting DO-A capable AIoT devices in TR 23.700-30.

1. Discussion

In TR 23.700-30, the following key issue is to be studied:

5.2	Key Issue #2: Support of DO-A Capable AIoT Devices

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

The paper addresses the key issue of supporting DO-A capable AIoT Devices.

This solution proposes the procedure of DO-A data transfer which is used to transmit the AIoT DO-A data from the AIoT Device to the AF. The preconditions for the DO-A data transfer include: the UE Reader registers to the network; the AIoT Device performs registration-like procedure; and the AF subscribes to the AIoT Device's DO-A data.

2. Proposal

It is proposed to capture the following changes into TR 23.700-30.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #5 |  | X |
| #6 |  | X |
| #7 |  | X |
| #8 |  | X |
| #9 |  | X |
| #10 |  | X |
| #11 |  | X |
| #12 |  | X |
| #13 |  | X |
| #14 |  | X |
| #15 |  | X |
| #16 |  | X |
| #17 |  | X |
| #18 |  | X |
| #19 |  | X |
| #X |  | X |

* * * * Second Change (All new text) * * * *

6.X	Solution #X: DO-A Data Transfer Procedure for Topology 2

6.X.0	High-level solution Principles

This solution addresses the Key Issue #2 "Support of DO-A Capable AIoT Devices", especially the following aspects:

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

This solution proposes the call flow for DO-A data transfer from the AIoT Device to the AF.

6.X.1	Description

The Ambient IoT Devices of type 2b/c may initiate communication independently with DO-A traffic. This solution is applicable to the UE type AIoT Reader for Topology 2.

The principles of this solution are as follows:

-	The procedure for DO-A data transfer which is used to transmit the AIoT DO-A data from the AIoT Device to the AF is proposed.

-	The AIOTF receives the DO-A data from the AIoT Device and sends it to the AF.

-	The ADM is used to store the subscription relationship between the AIoT Device and the AF.

-	The preconditions for the DO-A data transfer include: the UE Reader registers to the network; the AIoT Device performs registration-like procedure; and the AF subscribes to the AIoT Device's DO-A data.

6.X.2	Procedures

The procedure of DO-A data transfer from the AIoT Device to the AF for Topology 2 is depicted in Figure 6.X.2.1-1.

Figure 6.X.2.1-1: Initiating message and data transfer procedure

0.a The UE Reader registers to the core network indicating its AIoT Reader capability. The AMF checks whether the UE Reader is authorized to act as an AIoT Reader based on the subscription of the UE; if so, the AMF indicates the authorization of the UE Reader to the NG-RAN.

0.b The AIoT Device performs registration-like procedure and the Device ID is provided to the core network. After this step, the AIoT Device information and its serving AIOTF are updated in the ADM.

NOTE 1:	The security aspects for this registration-like procedure and the usage of AIoT Device ID or Temporary ID needs coordination with SA WG3.

1.  The AF subscribes to the AIoT Device's DO-A data. The AF sends the subscription request for the AIoT Device DO-A data, and the subscription request includes the Device ID, the AF ID, and the notification address. After this step, the ADM stores subscription information from the AF targeting the AIoT Device.

2.  When triggering by the pending DO-A data from the AIoT Device, the AIoT Device and the UE Reader perform random access and the AS connection between them is established.

3.  The AIoT Device sends the DO-A data transfer request to the UE Reader including the Device ID information and the DO-A data; the DO-A data transfer request is included in the AIoT NAS between the AIoT Device and its serving AIOTF.

4.  The UE Reader sends the DO-A data transfer message to the NG-RAN based on the RRC connection between the UE and the NG-RAN.

5.  The NG-RAN sends the DO-A data transfer message to the AIoT Device's serving AIOTF; the message is transferred by the AMF serving the UE Reader.

6.  The AIOTF sends the DO-A data transfer response to the NG-RAN to confirm the DO-A data is received.

7.  The NG-RAN sends the DO-A data transfer response to the UE Reader based on the RRC connection between them.

8.  The UE Reader sends the DO-A data transfer response to the AIoT Device.

9.  The AIOTF interacts with the ADM to fetch the information of the AF which subscribes to the DO-A data from the AIoT Device sending this data.

10.	The AIOTF sends the data transfer message to the NEF which serves the AF.

11.	The NEF transmits the data transfer message to the AF.

12.	The AF sends back the data transfer response to the NEF.

13.	The NEF transmits the data transfer response to the AIOTF.

6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Receive and transmit the DO-A data of AIoT Devices.

-	Interact with the ADM to fetch the subscription information.

NG-RAN:

-	Route the messages between the AIoT Device and the AIOTF.

ADM:

-	Store the AIoT Device information.

-	Store the subscription information between the AF and the AIoT Device DO-A data.

AIoT Device:

-	Register to the network.

-	Send DO-A data transfer messages.

NEF:

-	Route the messages between the AF and the AIOTF

* * * * End of Changes * * * *