SA WG2 Meeting #172	S2-2510243

17 - 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source:	ZTE

Title:	KI#2, Sol#8: Update

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The paper updates the solution 8 to address the editor’s notes and adds some description for AF subscription.

1. Discussion

In the procedure of solution 8, the following editor’s notes exist:

Editor's note:	Whether using registration like procedure instead of this initiating message procedure is FFS.

Editor's note:	The details of AIoT Device or Temporary ID used in the initiating request/response message is FFS.

The paper addresses the above editor’s notes.

This paper also adds the notification address in the request when the AF subscribes to the AIoT Device DO-A data; and clarifies than the AIOTF receiving the subscribing request may not be the serving AIOTF of the AIoT Device.

2. Proposal

It is proposed to capture the following changes into TR 23.700-30.

* * * * Start of Changes * * * *

6.8.2	Procedures

6.8.2.1	Initiating message and data transfer procedure

Figure 6.8.2.1-1: Initiating message and data transfer procedure

0.	The AIoT Device decides to trigger an initiating message based on some conditions, e.g. the AIoT Device needs to report AIoT data such as sensing data detected, the time for periodic reporting arrives, or the AIoT Device moves to a new position. In this stage the AIoT Device has no serving AIOTF available.

NOTE 1:	The initiating message procedure is very similar to the registration-like procedure performed by the AIoT Device; after this procedure the information regarding the AIoT Device is updated in the ADM and the serving AIOTF is discovered.

1.	The AIoT Device sends the initiating message to the NG-RAN. The initiating message includes the AIoT Device ID information, optional pending data transfer indication.

2.	The NG-RAN selects the serving AIOTF for the AIoT Device, taking into account the Device ID information, the PLMN ID and NID and/or the AIoT area which this AIoT Device belongs to.

3.	The NG-RAN sends the initiating message to the selected AIOTF directly (for direct connectivity option) or via an AMF (for indirect connectivity option); the initiating message includes the AIoT Device ID information.

4.	The AIOTF discovers the serving ADM and interacts with the ADM to fetch the subscription information and may update the AIoT Device information stored in the ADM.

5.	The ADM updates the AIoT Device information and the Device's serving AIOTF information.

6.	The AIOTF sends the initiating message response with the serving AIOTF ID to the NG-RAN directly or via the AMF.

NOTE 2:	The details of AIoT Device ID will be determined in the conclusion part of this study; whether and how the Temporary ID is used will coordinate with SA3 WG.

7.	The NG-RAN sends the initiating message response with the serving AIOTF ID to the AIoT Device. The AIoT Device stores the serving AIOTF ID

8.	The AIoT Device decides to trigger an AIoT data transfer message when the AIoT Device needs to report AIoT data such as sensing data detected, or the time for periodic reporting arrives.

9.	The AIoT Device sends the data transfer message to the NG-RAN. The data transfer message includes the AIoT Device ID information, the AIoT data and its serving AIOTF ID.

10.	The NG-RAN sends the data transfer message to the indicated AIOTF.

11.	The AIOTF sends the data transfer message response to the AIoT Device through NG-RAN.

12.	The AIOTF finds the AF subscribing to the AIoT data of the AIoT Device and then transmits the data transfer message to the NEF which serves the AF. The AF subscribing procedure is described in clause 6.8.2.2.

NOTE 3:	The ADM may store the information of the subscription relationship between an AF and the AIoT Device; in this case, the AIOTF interacts with the ADM to fetch this subscription information.

13.	The NEF transmits the data transfer message to the AF.

14.	The AF may send back the data transfer message response to the AIOTF through the NEF.

6.8.2.2	AF subscription procedure

Figure 6.8.2.2-1: AF subscription procedure

1.	If an AF decides to subscribe the AIoT data from a specific AIoT Device, the AF sends a subscription request to the NEF. The subscription request includes the AIoT Device ID, AF ID, and notification address.

2.	The NEF discovers an ADM based on the AIoT Device ID and interacts with the selected ADM to fetch the serving AIOTF for this AIoT Device.

NOTE 4:	The ADM may store the information of the subscription relationship between the AF and the AIoT Device. In this case, the AIOTF interacts with the ADM to create/update/remove the subscription information in the ADM.

NOTE 5:	If the serving AIOTF is not discovered, the NEF may choose another AIOTF to handle the subscription request.

3.	The NEF transmits the subscription request to the AIOTF serving the AIoT Device. The subscription request includes the AIoT Device ID, AF ID, and notification address.

4.	The AIOTF stores the information of the subscription relationship between the AF and the AIoT Device in the ADM.

5.	The AIOTF sends the subscription response to the NEF to indicate the subscription request successes or fails.

6.	The NEF transmits the subscription response to the AF.

* * * * End of Changes * * * *