SA WG2 Meeting #S2-172	S2-2510291

Dallas, USA, November 17 – November 21, 2025

Source:	Samsung

Title:	KI#4 Interim conclusion update

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC /Rel-20

Abstract of the contribution: This contribution proposes updates to FS_Sensing_ARC KI#4 interim agreements.

# Proposal

It is proposed to adopt the following changes into TR 23.700-14.

*** Start of the change ***

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor’s Note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	Sensing data and and associated inforamtion may be provided from the Sensing Entity/ gNBs to the Sensing Function via direct connection or via indirect connection through AMF.

NOTE y: the protocol to be used for the direct connection requires coordiantion with RAN WGs.

-	The establishment of the direct connection for sensing data and associated inforamtion transfer might be based on etablished associasation, e.g. using the IP address/FQDN of SF and SE.

-	The establishment and modification of the direct connection migth be triggered by the SF or SE.

-	For the establishment of the indirect connection, AMF will facilitate end point addresses of SE and SF.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

*** End of the change ***