3GPP SA WG2#172	S2-2510327

Dallas, USA, 17-21 November, 2025

Source:	Tejas Networks Limited

Title:	Draft interim Agreements for KI#2

Spec:	3GPP TR 23.700-30

Agenda item:	20.5.1

Work Item / Release: FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution proposes interim agreements for KI#2 of release-20 Ambient IoT study.

1. Introduction

KI#2 described in TR 23.700-30 is aimed at supporting DO-A capable Ambient IoT Devices in Topology 1 and Topology 2. The scope of KI#2 is provided below:

To address KI#2, at present TR 23.700-30 contains 19 documented solutions. This paper discusses the approved solutions and proposes interim agreements for the following topics :

- AIoT device registration for DO-A

[Observation-1] :  The accepted solutions within the TR broadly follow three approaches. The first approach as documented in Solutions  #1, #2, #9, #10, #11, #14, #18, and #19 involves the DO-A capable device registering with the network either on power-on or whenever it has data to transmit. The second method advocates a network initiated registration using various mechanisms (Solution #3, #4, #13). The third method mentioned in Solution #5, #6, #7 does not require any registration and uses pre-configured resources.

In our view, it is preferable that the Ambient IoT device registers with the network. This approach does not mandate the pre-requisite that the device should have responded any prior messages e.g., Inventory or command to be able to communicate with the network.

Moreover, some of the solutions support both periodic registration and mobility registration #11, #13, #14 from the device. The mobility registration can occurs when the AIoT device transitions across AIoT areas. Periodic registration occurs when a device timer expires.

These approaches involve the additional complexity of supporting a timer as well as keeping track of AIoT areas. However, we support the concept of periodic and mobility registration as it may aid in choosing an appropriate AIoT reader for a device and would also help in locating the device within the network with finer granularity.

[Proposal-1.1]:  DO-A capable AIoT devices perform initial registration with the network autonomously.

[Proposal-1.2]:  Registered DO-A capable AIoT devices also perform periodic registration with the network at the expiry of a periodic timer.

[Proposal-1.2]: Registered DO-A capable AIoT devices also perform mobility registration with the network on moving to a new AIoT area.

- DO-A Procedure initiation and data routing aspects

  - Authentication of requests from AIoT device :

[Observation-2a] Solutions #1, #2  propose that the AIOTF authenticates the Registration request sent by the AIoT device. Once the device is authenticated and authorized, the AIoT device profile data is updated and a context is created, detailing registration status and access permissions.

In addition, solutions also propose that once an AIoT device is registered, the AIOTF may skip the authorization checking and NEF discovery processes. IDs such as AIoT device Temp ID /session ID can be used for identification in the subsequent messages with the network.

[Proposal-2.1]:  AIOTF authenticates the Registration Request sent by the AIoT device.

[Proposal-2.2:   Once an AIoT device is registered, AIoT device profile data is updated and device context is created in ADM.

[Proposal-2.3]:  For registered DO-A capable AIoT devices, authorization for any subsequent requests may be skipped.

  - Support for reporting periodic/event triggered DO-A data report

[Observation-2b] Solutions #3 #15 support periodic /event triggered data collection from the device. This involves pre-configuration of device or configuration provided by the AF. The pre-configurations include the sensing start time, offsets, duration etc.

Also,  DO-A data traffic can be enabled within the network for a group of devices/within a certain area through an activation request (Solution #9, #17, #19). The above scenarios would occur whenever a multitude of sensors are deployed and the AF may expect to collect data during a specific time duration.

Additionally, devices may indicate that they have DO-A Data to transmit via a data ready indication to initiate the DO-A traffic (Solution#16).

[Proposal-2.4]:  Periodic/ event triggered data collection from AIoT devices can be supported via device pre-configuration or AF configuration.

[Proposal-2.5:   DO-A data traffic can be enabled for a subset of devices in the network/in a certain area through a message from the AF.

[Proposal-2.6]:  DO-A capable device can support transmission of an indication message whenever they have data to transmit..

- Interface/ network function enhancements

[Observation-3] Several enhancements  have been proposed to in various the network functions to support DO-A traffic.

For example, ADM needs to support the storage of following details in the AIoT device profile viz., registration status (solutions #1, #4, #11, # 13,#14), access permissions, AIOTF ID (solution #4 ), registration area (Solution #11), DO-A capability (Solution #3, #4, #19).

Similarly, AF authorization data has be enhanced to support indication of whether AF is allowed to accept DO-A delivery (Solution #5, #10).

[Proposal-3.1]:  AIoT device profile stored in the ADM includes device registration status, AIOTF ID, access permissions, registration area, DO-A capability.

[Proposal-3.2]:   AF authorization data includes a parameters to indicate if AF can accept DO-A data delivery.

In conclusion, a text proposal summarizing the proposed enhancements is provided in the succeeding section.

2. Text Proposal

It is proposed to adopt the following text within TR 23.700-30.

* * * * First Change (All New Text) * * * *

### 7.1.2	Agreed principles for KI#2

The following principles are agreed for KI#2 to support DO-A traffic in the network:

- Ambient IoT devices that are DO-A capable register autonomously with the network.

- The devices register with the network upon power-on by sending a Registration Request (NAS message) to the AIOTF. The registration request contains the AIoT device ID.

Editor’s note:	Other parameters that are exchanged during Registration Request are FFS.

Editor’s note:	How the AIOTF is chosen for DO-A registration is FFS.

- The AIOTF authenticates the Registration Request and responds to the AIoT device using a Registration Accept message. The Registration Accept message contains a Temporary ID for the device, AF ID to which subsequent data is to be routed.

Editor’s note:	Other parameters that may be included in the Registration Response are FFS.

- On successful registration, AIoT device profile data is updated and device context is created in ADM.

- The device profile data contains registration status, AIOTF ID, access permissions, registration area, DO-A capability.

Editor’s note:	Other enhancements to AIoT device profile data are FFS.

- Authorization of subsequent messages from the registered device may be skipped. To identify devices that have been registered, parameters such as Temp ID, session request etc may be included in the subsequent signalling exchanges from the AIoT device.

Editor’s note:	Detailed set of parameters that are sent by the AIoT device are FFS.

.

- After a DO-A capable AIoT device performs initial registration, it also performs periodic registration.

Editor’s note:	Further details regarding the periodicity of the registration are FFS.

- After a DO-A capable AIoT device performs initial registration, it performs mobility registration when it crosses over to a different AIoT area.

Editor’s note:	Further details regarding the AIoT area are FFS and need coordination with RAN WGs.

- DO-A capable device can support transmission of an indication message whenever they have data to transmit..

Editor’s note:	Further details regarding the indication message are FFS.

- Periodic/ event triggered data collection from AIoT devices can be supported via device pre-configuration or AF configuration.

- DO-A data traffic can be enabled for a subset of devices in the network/in a certain area through a message from the AF.

- AF authorization data includes a parameters to indicate if AF can accept DO-A data delivery.

Editor’s note:	Further enhancements to AF authorization data are FFS.

* * * * End of Changes * * * *