3GPP SA WG2 Meeting #172	S2-2510341

17 - 21 November, 2025, Dallas, USA	(revision of S2-25XXXXX)

Source: 	LG Electronics

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusion for KI#1 in TR 23.700-19.

1	Discussion

In SA2#171 meeting, following interim conclusion for Key Issue#1 was agreed.

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

This paper proposes other conclusions for KI#1.

1.1	Number of PDN Connection

There are some solutions using a single PDN connection for both IMS signalling and voice, and other solutions using separate PDN connections for IMS signalling and voice packet.

For using separate PDN connections, two APNs needs to be deployed for an IMS service, each PDN connection requires different address, the voice setup procedure requires another PDN connection for voice media packet which requires additional call setup delay. Also, the PDN connection for voice media only support default bearer with Non-GBR bearer type, which cannot support QoS differentiation for IMS voice.

Proposal 1.

-	A single PDN connection shall be used for both IMS signalling and IMS voice.

1.2.	IMS Signalling over CP vs UP

Based on the current architecture, all the SGi PDN connections of a UE shall either use S11-U or S1-U at any point in time. Therefore, during the voice call (which is decided to use user plan for voice packet), the signalling bearer for IMS PDN connection also need to use user plane.

The UE triggering IMS voice call over control plane can save call setup time because the control plane message can contain the INVITE message when the UE is in idle state. The time for 4 messages between the UE and network (2 of NAS service request/accept, 2 of AS security, 2 of RRC connection reconfiguration, and the RRC message can include a piggybacked NAS message), which is about 1.1s, (assuming the propagation delay of 285ms) can be saved to trigger the call. This can reduce call setup time by up to 1.1s out of the total call setup time of 9.7s ~ 15s when B2BUA is used (based on the estimated call setup time in S2-2508341). In this case, control plane exceptionally can be used to carry SIP INVITE message, and at the same time, this procedure can also be used to request to establish S1-U data transfer for voice call initiation for both signalling and voice.

Proposal 2.

-	The IMS signalling packets shall be transported over user plane except for the following case:

-	When the UE initiate voice call in ECM-IDLE, the UE transports SIP INVITE message over control plane while requesting to establish S1-U bearer.

1.3.	IP vs Non-IP

Based on the Reply LS from RAN1 (S2-2509863), ROHC can be used in typical operating conditions assuming LOS. Satellite access is mainly targeted at remote and open areas, and typically requires LOS. Therefore, IP type with ROHC can be used for IMS voice packet to reduce IP overhead.

Also, based on the current architecture, dedicated bearers are not supported for the Non-IP data, and Non-IP data transfer requires PGW enhancements for Non-IP to IP conversion. Therefore, supporting Non-IP data to transfer voice packet cause more impact to existing architecture. Since one PDN Type is used for a PDN connection (IP or Non-IP), both the IMS signalling and voice need to use either IP or Non-IP.

To minimize impact, we propose to use IP type PDN connection for both IMS signalling and IMS voice packet

Proposal 3.

-	The PDN type of IMS PDN connection for voice call shall be IP type.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-19.

* * * * Start of 1st Change * * * *

8.1	Conclusions for Key Issue #1

The following conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made to progress normative work:

-	The IMS voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	A single PDN connection shall be used for both IMS signalling and IMS voice.

-	The IMS signalling packets shall be transported over user plane except for the following case:

-	When the UE initiate voice call in ECM-IDLE, the UE transports SIP INVITE message over control plane while requesting to establish S1-U bearer.

-	The PDN type of IMS PDN connection for voice call shall be IP type.

* * * * End of Changes * * * *