3GPP SA WG2 Meeting #172	S2-2510344

Dallas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6473 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing in the case of NTN access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skylo Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The existing text in Clause 5.18.3 for INS specifies a rejection case based on identity (rejecting a UE that is not from a participating operator). However, this needs to be functionally expanded to cover rejections based on geographical policy enforcement. For example, a UE from a valid participating operator may attempt to access the network at a location where the hosting operator is not regulatorily authorized to provide service for the participating operator (e.g. the sharing agreement is limited to a specific country, region, or area in the case of shared NTN RAN). The hosting network should reject the UE in this case. Furthermore, when the shared RAN is NTN, the location verification may be used for AMF to make such a decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new sentence to mandate that the hosting network rejects a UE from a participating operator id it determines the UE’s current location is outside the authorized shared service area. in clause 5.18.3 and R.1, Add clarification that when a share RAN is NTN, the location verification procedure may be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The lack of this CR would leave a gap in policy enforcement integrity and regulatory compliance for any Indirect Network Sharing deployment involving geographically constrained service or NTN access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.3, R.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.18.3	Network selection by the UE

NOTE 1:	This clause applies to UEs not operating in SNPN access mode. Network selection for UEs set to operate in SNPN access mode is described in clause 5.30.2.4.

A UE that has a subscription to one of the sharing core network operators shall be able to select this core network operator while within the coverage area of the shared network and to receive subscribed services from that core network operator.

Each cell in shared NG-RAN shall in the broadcast system information include the PLMN IDs concerning available core network operators in the shared network.

When a UE performs an Initial Registration to a network, one of available PLMNs shall be selected to serve the UE. UE uses all the received broadcast PLMN IDs in its PLMN (re)selection processes which is specified in TS 23.122 [17]. UE shall inform the NG-RAN of the selected PLMN so that the NG-RAN can route correctly. The NG-RAN shall inform the core network of the selected PLMN.

NOTE 2:	For Indirect Network Sharing as specified in clause 5.18.1, the broadcast PLMN IDs include the PLMN IDs representing participating operators and a UE from a participating operator selects a PLMN ID representing the participating operator as specified in TS 23.122 [17].

NOTE 3:	The serving AMF of hosting operator's network acquires the relevant UE subscription data from the participating operator's network during the Registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [3].

In the case of Indirect Network Sharing, in this Release of the specifications, if hosting operator's network identifies that a UE is not from a participating operator when the UE selects a PLMN ID representing the participating operator, then the hosting operator network rejects the UE with an existing cause value during the registration procedure. In addition, if the hosting operator’s network determines that a UE from a participating operator is not authorized to be served at the current geographical location (e.g. for the case where the shared RAN is NTN) based on policy or regulatory restrictions, the hosting operator network shall reject the UE with an existing cause value during the registration procedure.

NOTE X: For the case where the shared RAN is NTN, if the User Location Information (ULI) received from the RAN (e.g., TAI) does not provide sufficient granularity for the AMF to enforce geographical policies (e.g., Mobility Restrictions or regional restrictions) for the participating operator’s UE, the AMF shall utilize location verification as specified in clause 5.4.11.4 of TS 23.501to ensure that the geographical requirements are met.

As per any network, after Initial Registration to the shared network and while remaining served by the shared network, the network selection procedures specified in TS 23.122 [17] may cause the UE to perform a reselection of another available PLMN.

UE uses all of the received broadcast PLMN IDs in its cell and PLMN (re)selection processes.

SECOND CHANGE

Annex R (Informative):
5GS support for Indirect Network Sharing deployment

# R.1	General

This annex provides guidance on how 5GS features and capabilities can be used to support Indirect Network Sharing deployment option which is specified in clause 6.21 of TS 22.261 [2].

The Indirect Network Sharing deployment option is described in clause 5.18 and further illustrated in the Figure 5.18.1-2.

In the geographical areas with Indirect Network Sharing deployment, the existing mobility restriction mechanisms specified in clause 5.3.4.1 can be applied. This can be controlled by the UE subscription data related to mobility restriction and Indirect Network Sharing policies in the serving AMF.

For the mobility management, the related procedures (e.g. handover, mobility registration update) specified in clause 4.23 of TS 23.502 [3] can be reused.

For the case where the shared RAN is NTN, if the User Location Information (ULI) received from the RAN (e.g., TAI) does not provide sufficient granularity for the AMF to enforce geographical policies (e.g., Mobility Restrictions or regional restrictions) for the participating operator’s UE, the AMF shall utilize location verification as specified in clause 5.4.11.4 of TS 23.501to ensure that the geographical requirements are met.

End of CHANGES