3GPP SA WG2 Meeting #172	S2-2510345

Dallas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6474 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing for disaster condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skylo Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been agreed, in CR6424 Rev 3 proposed in S2-2509816, that the Indirect Network Sharing can be used for the UE of the participating operator with the disaster condition to reduce the service interruption. In CR6424 Rev 3, the current "Editor's note" regarding overload and traffic distribution among multiple hosting operators is addressed by a Note clarifying that: How to distribute the UE access to different hosting operator networks in the disaster area to avoid overload is up to operator deployment and policy. Existing 3GPP mechanisms, such as EAB, rejection cause codes, and UE retrial control mechanisms, are sufficient for this purpose and no specific standardized solution is required for Indirect Network Sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NOTE to resolve the EN in the case of multiple hosting operators in INS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN cannot be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.18.x	Indirect Network Sharing for disaster condition

Subject to operator policy and national/regional regulations, the Indirect Network Sharing can be used by UEs of the PLMN with the Disaster Condition to minimize the service interruption.

NOTE 1:  With Indirect Network Sharing for Disaster Condition, the UE is not aware of disaster condition and the UE behaviour is same as the normal case of Indirect Network Sharing as specified in clause 5.18.3.

NOTE 2:	How the hosting PLMN is made aware of another PLMN (as the participating operator network) is a PLMN with disaster condition and how the hosting PLMN is made aware of the area where the disaster condition applies is out of scope of 3GPP.

When the disaster condition arises, the AMF of the hosting operator’s network receives the NAS Registration Request message sent by the UE of the participating operator. The AMF identifies that the UE is registering to the network via the Indirect Network Sharing for Disaster Condition by checking if the UE is from a participating operator that can use Indirect Network Sharing for Disaster Condition and that the UE is registering to the network for the Indirect Network Sharing and the UE is in the area of Disaster Condition. If the above conditions are satisfied, comparing to the normal Indirect Network Sharing case, the AMF provides the Disaster Roaming service indication to AUSF and UDM during the registration procedure.  In addition, the AMF also provides this indication to V-SMF during the PDU session related procedure and the V-SMF provides it to the H-SMF. The NFs of the participating operator network (e.g. AUSF, UDM and SMF) are able to identify which UE(s) are using the Indirect Network Sharing for Disaster Condition based on the Disaster Roaming service indication and the PLMN ID of the hosting operator.

NOTE 3:  The Disaster Roaming service indication is specified in the clause 5.40.4.

NOTE 4:  Per-PLMN unified access control can be used to reduce load in the hosting operator’s network and allow the access priority for UEs with special access classes during the disaster condition.

NOTE 5: How to distribute the UE access to different hosting operator networks in the disaster area to avoid overload is up to operator deployment and policy. Existing 3GPP mechanisms specified in TS 23.122, e.g. EAB, rejection cause code, and UE retrial control mechanisms can be used for this purpose.

End of CHANGES