3GPP TSG-WG SA2 Meeting #172	S2-2510408

17 - 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	Interim Agreements for Key Issue 2

Document for:	Approval

Agenda item:	20.5.1

Spec:	3GPP TR 23.700-30

Version:	0.2.0

Work Item / Release:	FS_AmbientIoT-ARC_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim conclusion for KI#2 to TR 23.700-30.

1. Introduction

Release-20 Ambient IoT study includes the support of DO-A capable Ambient IoT Devices in Topology 1 and Topology 2 and there are 19 solutions for KI#2 included in TR 23.700-30.

This contribution proposes interim agreements for the aspects related to the initial registration and the data transfer for the DO-A capable Ambient IoT Devices.

Following are the essential aspects for key issue 2 from our point of view:

1.	Registration required before autonomous operation.

-	The network establishes authentication and authorization before DO-A operation.

-	Only the registered devices can send DO-A traffic.

2.	Network-requested radio resource allocation for DO-A operation.

-	Timing and scope for DO-A operation can be provided to the NG-RAN by the AIOTF based on the AF subscription or local configuration.

3.	Subscription-based DO-A data delivery.

-	The DO-A traffic from the Ambient IoT Devices are routed by the network to the subscribed/designated AFs

3. Conclusion and proposal(s)

It is proposed to agree the following proposals in TR 23.700-30.

* * * * Start of 1st Change (All New Texts) * * * *

7.1.X	Agreed Principles for KI#2

Following principles are agreed for key issue 2:

Radio Resource for DO-A operation:

-	The AIOTF selects NG-RAN and AIoT Reader based on local configuration or AF subscription to trigger registration of AIoT Devices.

-	The AIOTF requests the selected NG-RAN and Reader to allocate radio resources for DO-A capable AIoT Device.

-	The NG-RAN allocates radio resource for the AIoT Reader to broadcast access parameters for DO-A procedure (i.e. registration and data transfer) based on the request from the AIOTF.

NOTE 1:	Both Topology 1(RAN Reader) and Topology 2(UE Reader) are supported and coordination with RAN WGs is required for radio resource allocation.

AIoT Device Profile Data for a DO-A capable AIoT Device:

-	The AIoT Device Profile Data is extended with the following additions:

-	The DO-A capability is pre-configured for a DO-A capable AIoT Device.

-	The Registration status, which is updated via Registration procedure.

-	The Target AF(s) for data routing can be pre-configured or updated during based on AF request.

Initial Registration:

-	The DO-A capable AIoT Device shall perform registration to the network and get accepted to send DO-A traffic.

DO-A data transfer:

-	The registered DO-A capable AIoT Device sends DO-A traffic to the AIOTF via AIoT Reader.

-	The AF subscribes for the DO-A data transfer to the AIOTF directly or via the NEF.

-	The AIOTF supports routing of DO-A data to the target AF(s) based on the Target AF(s) for data routing in the AIoT Device Profile Data.

NOTE 2:	Security aspects including authentication, NAS message protection, privacy protection, and the use of temporary ID mechanisms require coordination with SA WG3.

* * * * End of Changes * * * *