3GPP TSG-WG SA2 Meeting #172	S2-2510413

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5: Conclusion proposal

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#5 on Sensing Result Exposure is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, the architectural conclusion of KI#1 in S2-2509659, this paper provides the conclusion on KI#5 on Sensing Result Exposure.

The proposed text is a revision based on the draft merging version prepared by Nokia but was not discussed during last meeting.

The following aspects are clarified:

-	Input parameters;

-	Parameters included in the Sensing Result sent to AF as the Sensing Service Consumer;

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.5	Agreed Principles for KI#5 Sensing Result Exposure

The following principles are agreed for KI#5:

a)	SF generates, optionally stores and provides the Sensing Results to the NEF and then NEF provides the Sensing Results to the AF and for trusted AF the SF communicates to the AF directly as currently.

NOTE 1:	It is assumed that the AF as the Sensing Service Consumer sends the sensing service request.

NOTE 2:	In case of SCF and SPF deployment, the Sensing Result is exposed by Sensing Control Function.

NOTE 3:	In case of SCF and SPF deployment, how the SPF provides the sensing result to SCF is implementation-based.

b) 	The trigger and preparation of sensing results are based on the Request Type of the sensing service request received from the AF as the Sensing Service Consumer, i.e., one time, event-based, or periodic. The event trigger may include object detection, object in/out of a defined area, velocity threshold, path deviation.

NOTE Y: The conditions for event triggered sensing report are to be specified during normative phase.

c) 	Sensing request from the AF includes AF Identifier (optionally address), Sensing Service Type, Target Sensing Service Area, Sensing Service Requirement, Sensing Results Configuration (one-time, event-based, subscription-based, periodically).

d)	Based on the requested Sensing Service Type, the Sensing Result to the Sensing Service Consumer can be one of the followings:

-	The list of objects, with containing the characteristics (e.g., location, velocity) of the object within the Target Sensing Service Area.

-	Information of the event, i.e., Object In/Out of the Target Sensing Service Area, Path deviation (including the Path information).

e) 	The results exposed can permit for the Application Function to request a subsequent sensing service (e.g., tracking) associated to at least one object that is contained in a Sensing Result.

NOTE 4: The details of the parameters in the Sensing Request and in Sensing Response including Sensing Result will be further defined during normative phase.

NOTE 5: The parameters used between the AF and the SF may be updated during normative phase considering the input from RAN WGs.

NOTE 6: Privacy and security aspects related to sensing result exposure needs the coordination with SA3.

NOTE 7: Charging and OAM implications of exposing sensing results needs the coordination with SA5.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.5 are agreed as the conclusion.

# * * * * End of changes * * * *