SA WG2 Meeting #172	S2-2510423

17 - 21 Nov, 2025, Dallas, TX, USA

Source:	Samsung

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control signalling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope of Work task 1.1.

1. Proposal

As per pre-SA2#172 offline discussion, the WT scope highlighted in green below can be considered as stable.

One of the proposal during the offline discussion was to separate the WT scope discussion and KI discussion based on the two different contributions. We believe that the proposal is reasonable and we only covers the WT scope in this paper.

One issue raised during the offline discussion was that it needs to be clear that WT#1.1 bullet 2a only covers services that are not in control of MNOs. The proposal here is to add the NOTE highlighted in yellow to resolve the issue.

Proposal 1: Add the NOTE below

NOTE 4: This WT1.1 bullet 2a covers services that are only controlled by MNO. MNO shall have full controllability over operator services being covered by the WT1.1 bullet 2a.

Another issue was on the definition of NAS, especially, whether it is the highest stratum of control plane between UE and CN.

Non-Access Stratum: The Non-Access Stratum (NAS) forms the highest stratum for control plane between the UE and the 6G CN.

We assume that the confusions on "the highest stratum" came from the understanding that some other types of messages being transported by NAS is also part of control plane and also part of NAS.

Consider LPP which can be transported over NAS in 5G but also can be transported over user plane. So it is not true that LPP is part of the control plane or NAS. LPP is something that can be transported whatever transport mechanisms being supported between UE and CN. Having said that, we see no issue on “the highest stratum”. So our proposal is to keep the definition as-is.

Proposal 2: Keep the NAS definition as-is.

**** First Change ****

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Access Stratum: The Non-Access Stratum (NAS) forms the highest stratum of the control plane between the UE and the 6G CN.

Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT1.1 bullet 1a, WT3, WT4, WT5, WT6.

NOTE 4: This WT1.1 bullet 2a covers services that are only controlled by MNO. MNO shall have full controllability over operator services being covered by the WT1.1 bullet 2a.

**** End of Changes ****